The reference file defines this topic simply as `03_Hierarchy → Machine → cell → line → shop → plant` within 02_INDUSTRY3_0 → 01_Fundamentals. Pasted markdown(1)

# 03_Hierarchy: Machine → Cell → Line → Shop → Plant

The purpose of this topic is to understand how a manufacturing system is organized from the smallest operational unit to the complete manufacturing facility.

The hierarchy is:

Machine → Cell → Line → Shop → Plant

Think of it as increasing scope of operation, coordination, material flow, production responsibility, and management visibility.

It is not simply a hierarchy of physical size. A larger physical object is not automatically a higher hierarchy level. The important question is:

What operational scope does this level control or coordinate?

## 1. Machine

A machine is the basic individual production or processing unit.

It performs a defined manufacturing operation on a workpiece, component, material, or product.

Examples:

CNC machining center

CNC lathe

Press

Welding machine

Assembly machine

Robot

Conveyor machine

Testing machine

Heat-treatment equipment

Packaging machine

A machine normally contains several subsystems:

Machine

→ mechanical system

→ electrical system

→ sensors

→ actuators

→ controller

→ HMI/operator interface

→ safety system

→ communication/networking

→ process tooling

### Example

Consider a CNC machining center.

It may contain:

CNC controller

Servo drives

Servo motors

Spindle drive

Spindle motor

Tool changer

Coolant system

Hydraulic/pneumatic system

Limit/proximity sensors

Safety door

PLC logic

HMI/CNC display

All of these together form one production machine.

### Automation perspective

At the machine level, automation is primarily concerned with:

sensing

decision-making

sequencing

motion

actuation

interlocking

safety

operator interaction

machine diagnostics

A simplified flow is:

Sensor → Controller → Logic → Actuator → Physical operation

For example:

Part present sensor

→ PLC detects part

→ PLC checks permissives

→ clamp command

→ clamp cylinder operates

→ clamp confirmation

→ machining cycle starts

That entire sequence belongs primarily to the machine level.

# 2. Cell

A cell is a group of machines/equipment arranged to perform a related manufacturing operation or a sequence of operations.

So:

Machine = individual production unit

Cell = coordinated group of production units

A cell may contain:

multiple machines

robots

conveyors

fixtures

inspection equipment

loading/unloading systems

material handling equipment

safety systems

cell controller/network

### Example

Imagine an automated machining cell:

Part loading

↓

Robot

↓

CNC Machine 1

↓

Robot

↓

CNC Machine 2

↓

Inspection station

↓

Unloading

Here, CNC 1, CNC 2, robot, inspection station and material handling are not independent from the production point of view.

They operate together as a manufacturing cell.

### Why a cell exists

The purpose is generally to coordinate several machines/equipment around a related manufacturing process.

Instead of thinking:

"Machine 1 is doing something."

you start thinking:

"This group of machines is producing a particular operation or product flow."

# 3. Machine vs Cell

This distinction is extremely important.

| Machine | Cell |
| --- | --- |
| Individual production unit | Group of coordinated equipment |
| Performs a specific operation | Coordinates multiple operations/equipment |
| Local machine control | Cell-level coordination |
| PLC/CNC/HMI may control it | Cell controller/network may coordinate machines |
| One machine failure affects that machine | One cell problem may affect several machines |

Example:

Machine:

CNC-01

Cell:

CNC-01 + CNC-02 + Robot-01 + Inspection-01 + conveyors + fixtures

The robot may load CNC-01 and CNC-02, while the inspection station verifies the completed component.

That entire coordinated area is the cell.

# 4. Line

A production line consists of multiple machines and/or cells arranged in a defined production sequence to manufacture or process a product.

The key idea is:

Material/product flows through a sequence of operations.

For example:

Raw component

↓

Operation 1

↓

Operation 2

↓

Operation 3

↓

Inspection

↓

Operation 4

↓

Finished component

Each operation may itself be a machine or an entire cell.

Therefore:

Line = coordinated production flow consisting of multiple operations/cells/machines.

### Example

An automotive component production line could contain:

Cell 1

→ machining

Cell 2

→ drilling

Cell 3

→ washing

Cell 4

→ inspection

Cell 5

→ assembly

Together they form a production line.

# 5. Cell vs Line

The distinction is mainly about scope and production flow.

### Cell

Focus:

"How do these machines work together?"

### Line

Focus:

"How does the product move through these operations?"

Example:

Cell

Robot + CNC1 + CNC2 + inspection

↓

Line

Cell A → Cell B → Cell C → Cell D

The cell is a coordinated local manufacturing group.

The line represents the broader sequence of production operations.

# 6. Shop

A shop is a larger manufacturing area containing multiple production lines, cells, machines, support systems, and associated activities for a particular manufacturing function or product family.

This is where the scope becomes significantly larger.

A shop might be organized around activities such as:

machining

body manufacturing

paint

assembly

forging

stamping

engine manufacturing

transmission manufacturing

welding

For example:

Machining Shop

may contain:

Line 1

Line 2

Line 3

Line 4

inspection areas

material handling

tool management

maintenance facilities

utilities

production support

So:

Machine

↓

Cell

↓

Line

↓

Shop

The shop is therefore a larger production organization containing multiple production systems.

# 7. Plant

A plant is the complete manufacturing facility.

It may contain multiple shops and all the infrastructure required to operate the manufacturing facility.

A simplified structure could be:

Plant

├── Machining Shop

├── Assembly Shop

├── Paint Shop

├── Welding Shop

├── Quality

├── Maintenance

├── Utilities

├── Stores

├── Material Handling

└── Other support functions

Therefore:

Plant = overall manufacturing facility

It represents the highest level in this particular hierarchy.

# 8. Complete Hierarchy

The entire concept can now be visualized as:

PLANT

│

├── SHOP

│   │

│   ├── LINE

│   │   │

│   │   ├── CELL

│   │   │   │

│   │   │   ├── MACHINE

│   │   │   ├── MACHINE

│   │   │   └── MACHINE

│   │   │

│   │   ├── CELL

│   │   └── CELL

│   │

│   ├── LINE

│   └── LINE

│

├── SHOP

│   ├── LINE

│   └── LINE

│

└── SHOP

├── LINE

└── LINE

The important relationship is:

Many machines → form a cell

Many cells/machines → form a line

Many lines/cells → form a shop

Many shops → form a plant

This is the conceptual structure, not a rigid rule that every factory must physically implement exactly this way.

# 9. A Manufacturing Example

Consider a hypothetical automotive component plant.

### Machine level

One CNC machine:

CNC-101

Its job:

Machine one specific feature on a component.

### Cell level

A machining cell:

Robot → CNC-101 → CNC-102 → Inspection

The robot transfers components between machines.

Now several machines cooperate.

### Line level

A complete machining line:

Raw component

↓

Machining Cell 1

↓

Machining Cell 2

↓

Washing

↓

Inspection

↓

Final machining

The product progresses through multiple manufacturing stages.

### Shop level

The machining shop contains:

Machining Line 1

Machining Line 2

Machining Line 3

Machining Line 4

Tool management

Inspection

Material handling

Maintenance

Now the scope is the complete machining production area.

### Plant level

The complete plant may contain:

Plant

│

├── Press Shop

├── Welding Shop

├── Paint Shop

├── Machining Shop

├── Assembly Shop

├── Quality

├── Maintenance

├── Utilities

└── Logistics

Now we are looking at the complete manufacturing facility.

# 10. What Happens When You Move Up the Hierarchy?

As we move upward:

Machine → Cell → Line → Shop → Plant

the scope increases.

| Level | Primary focus |
| --- | --- |
| Machine | Individual operation |
| Cell | Coordinated equipment |
| Line | Product/process flow |
| Shop | Production area |
| Plant | Complete manufacturing facility |

Another way to remember it:

Machine = Do

Cell = Coordinate

Line = Flow

Shop = Organize production area

Plant = Integrate the factory

These are conceptual memory aids, not formal definitions.

# 11. Control and Coordination

The hierarchy also matters from an automation perspective.

At the machine level, you may have:

sensors

actuators

PLC

CNC

VFD

servo

HMI

safety controller

At the cell level, you may additionally have:

robot controller

machine-to-machine communication

cell PLC

conveyors

inspection systems

handshake signals

At the line level:

line coordination

production sequencing

line status

material flow

production monitoring

line-level data collection

At the shop level:

multiple production lines

production monitoring

maintenance coordination

quality coordination

material coordination

At the plant level:

multiple shops

plant-wide production

quality

maintenance

logistics

utilities

management systems

enterprise integration

This is where the hierarchy begins connecting naturally with the industrial automation architecture you already studied.

# 12. Important: Hierarchy Is NOT the Same as Automation Architecture

This distinction should be kept very clear.

You previously studied:

Industrial Automation Architecture

Now you are studying:

Machine → Cell → Line → Shop → Plant

These are related, but they answer different questions.

### Hierarchy asks:

Where does the manufacturing activity belong?

Machine → Cell → Line → Shop → Plant

### Automation architecture asks:

How are sensing, control, supervision, communication and higher-level systems organized?

For example:

Field → Control → Supervisory

These two concepts can exist simultaneously.

A machine can have:

field devices

PLC/control system

HMI/supervisory interface

while that machine itself belongs to:

Cell → Line → Shop → Plant

Do not merge these two hierarchies into one giant conceptual soup. Humanity already has enough badly labelled architecture diagrams.

# 13. Physical Hierarchy vs Control Hierarchy

A useful mental model is:

### Manufacturing hierarchy

Machine

↓

Cell

↓

Line

↓

Shop

↓

Plant

This describes manufacturing organization/scope.

### Automation/control hierarchy

Field

↓

Control

↓

Supervisory

↓

Higher-level systems

This describes automation/control organization.

They overlap, but they are not identical.

For example:

A machine can contain field devices and a PLC.

A cell can coordinate several PLC-controlled machines.

A line can coordinate several cells.

A plant can collect information from many lines and shops.

# 14. Why This Hierarchy Matters to Maintenance

For an automation/maintenance engineer, the hierarchy is extremely useful for troubleshooting.

Suppose production has stopped.

The first question should not always be:

"Which PLC has the fault?"

Instead determine the affected scope.

### Machine problem

One machine stopped.

Example:

CNC-03 fault

Other machines continue operating.

Likely scope:

Machine

### Cell problem

Several machines/equipment cannot operate because the cell coordination is unavailable.

Example:

Robot communication failure prevents loading/unloading of multiple CNCs.

Likely scope:

Cell

### Line problem

Several cells are affected because the line-level flow or coordination has stopped.

Example:

A major conveyor or line-control issue stops material flow between multiple cells.

Likely scope:

Line

### Shop problem

Multiple production lines are affected.

Possible causes:

shop-level power problem

utility failure

network infrastructure failure

material issue

common automation infrastructure

safety-related shutdown

Likely scope:

Shop

### Plant problem

Multiple shops or the entire facility are affected.

Possible causes include:

plant-wide electrical failure

major utility failure

network infrastructure failure

plant-wide emergency shutdown

major common infrastructure failure

Likely scope:

Plant

This gives maintenance personnel a very useful principle:

Identify the smallest level that explains the observed failure, then move upward only when evidence requires it.

# 15. Fault Propagation

A lower-level failure can propagate upward.

Example:

Machine failure

↓

Cell cannot complete operation

↓

Line production stops

↓

Shop production affected

↓

Plant production impact

But the reverse is also possible:

Plant utility/network failure

↓

Shop affected

↓

Multiple lines affected

↓

Multiple cells affected

↓

Machines stop

Therefore, seeing a machine stopped does not automatically mean the machine itself is faulty.

This is a very important maintenance concept.

A machine may be healthy but stopped because:

upstream → cell → line → shop → plant

something outside the machine is preventing operation.

# 16. Production Flow Perspective

Another useful way to visualize the hierarchy is through material flow.

PLANT

│

├── SHOP

│   │

│   ├── LINE

│   │   │

│   │   ├── CELL

│   │   │   ├── MACHINE

│   │   │   └── MACHINE

│   │   │

│   │   └── CELL

│   │

│   └── LINE

│

└── SHOP

Material generally moves through progressively larger organizational structures:

Machine operation

→ contributes to

Cell operation

→ contributes to

Line production

→ contributes to

Shop production

→ contributes to

Plant production

# 17. Data Perspective

The same hierarchy is useful when collecting production data.

For example:

### Machine data

Machine ID

Cycle status

Fault status

Running status

Part count

Cycle time

Alarm

Temperature

Motor current

↓

### Cell data

Cell status

Machine availability

Robot status

Cell production count

Cell faults

↓

### Line data

Line status

Production count

Line downtime

Line performance

Quality information

↓

### Shop data

Shop production

Multiple line status

Shop downtime

Shop performance

↓

### Plant data

Plant production

Shop performance

Plant downtime

Quality

Overall production information

This becomes particularly important later when you study SCADA, IIoT, dashboards, SQL databases, data collection and Industry 4.0.

Your current PLC → Node-RED → SQL → Grafana work is essentially dealing with this same concept from the data side.

# 18. A Very Important Nuance

The hierarchy is not universally rigid.

A real factory may use different organizational terminology.

For example:

Plant

└── Area

└── Line

└── Cell

└── Machine

Another factory might use:

Plant

└── Shop

└── Area

└── Line

└── Station

Another may call a group of machines an island, work center, production unit, or something else.

Therefore, the exact names can vary.

The fundamental idea remains:

Group smaller production units into progressively larger operational scopes.

For your study reference, however, the terminology to retain is specifically:

Machine → Cell → Line → Shop → Plant. Pasted markdown(1)

# 19. Machine, Cell, Line, Shop, Plant: One-Line Definitions

These are the definitions worth memorizing.

Machine

An individual production or processing unit performing a defined operation.

Cell

A coordinated group of machines/equipment performing related manufacturing operations.

Line

A sequence of machines/cells through which material or product progresses through multiple manufacturing operations.

Shop

A larger manufacturing area containing multiple production lines/cells and associated production/support activities.

Plant

The complete manufacturing facility containing multiple shops and the infrastructure required for production.

# 20. The Mental Picture to Keep

If you remember only one diagram from this topic, remember this:

PLANT

│

┌─────────────┼─────────────┐

│             │             │

SHOP          SHOP          SHOP

│

┌─────┼─────┐

│     │     │

LINE  LINE   LINE

│

┌──┼──┐

│  │  │

CELL CELL CELL

│

┌──┼──┐

│  │  │

M1 M2 M3

And remember the direction:

Machine → Cell → Line → Shop → Plant

means:

individual operation → coordinated equipment → production flow → production area → complete factory

# 21. Interview-Level Questions

### Q1. What is a manufacturing hierarchy?

A manufacturing hierarchy organizes production from individual machines to larger coordinated production units, typically:

Machine → Cell → Line → Shop → Plant.

### Q2. What is the difference between a machine and a cell?

A machine is an individual production unit. A cell is a coordinated group of machines/equipment performing related operations.

### Q3. What is the difference between a cell and a line?

A cell focuses on coordinated local equipment, while a line represents a broader sequence of production operations and product flow.

### Q4. What is a shop?

A shop is a larger manufacturing area containing multiple production lines/cells and associated production activities.

### Q5. What is a plant?

A plant is the complete manufacturing facility containing multiple shops and supporting infrastructure.

### Q6. Does every factory strictly follow Machine → Cell → Line → Shop → Plant?

No. Terminology and physical organization vary between factories. The hierarchy is a conceptual way to represent increasing manufacturing scope.

### Q7. Is machine → cell → line → shop → plant the same as field → control → supervisory?

No.

Machine → Cell → Line → Shop → Plant represents manufacturing/operational hierarchy.

Field → Control → Supervisory represents automation/control architecture.

# 22. Final Consolidated Concept

The complete concept can be reduced to:

MACHINE

Individual production unit

↓

CELL

Coordinated group of machines/equipment

↓

LINE

Sequence of manufacturing operations

↓

SHOP

Group of production lines/areas

↓

PLANT

Complete manufacturing facility

The critical idea is scope.

A machine answers:

What operation is being performed?

A cell answers:

How are several machines/equipment coordinated?

A line answers:

How does the product move through multiple operations?

A shop answers:

How is a broader production area organized?

A plant answers:

How does the complete manufacturing facility operate?

For your 01_Fundamentals sequence, this topic should establish the physical/operational scope of manufacturing before moving into the next concepts such as Discrete Automation, Process Automation, Batch Automation, Fixed/Programmed/Flexible Automation, and the Field/Control/Supervisory levels listed later in your reference. Pasted markdown(1)

Core line to retain in your notes:

Machine → Cell → Line → Shop → Plant = increasing scope of manufacturing activity, coordination, production flow, and operational responsibility.