# 04_Discrete: Discrete Automation

Your study reference places `04_Discrete → Discrete automation` immediately after the Machine → Cell → Line → Shop → Plant hierarchy, and before Process Automation, Batch Automation, and Fixed/Programmed/Flexible Automation. Pasted markdown(1)

The reference itself gives only the topic title, so the detailed definitions below are the foundational technical explanation of that topic. The later topics in your index are kept separate rather than being mixed into this one.

# 1. What is Discrete Automation?

Discrete automation is automation used to manufacture, assemble, move, inspect, or test individual products, parts, or units through distinct operations.

The word discrete is the key.

A discrete manufacturing process deals with things that can generally be treated as individual identifiable units.

Examples:

automobile

engine

gearbox

shaft

bracket

bearing

electronic PCB

bottle

packaged product

machined component

The product exists as an individual unit.

For example:

Part 1

Part 2

Part 3

Part 4

Part 5

Each part can be counted, tracked, inspected, rejected, transferred, assembled, or reworked.

That is fundamentally different from a process where material continuously flows through the plant, such as:

liquid

gas

steam

oil

chemical solution

# 2. The Core Idea

The simplest mental model is:

Discrete automation controls a sequence of operations performed on individual products or components.

For example:

Part arrives

↓

Part detected

↓

Part clamped

↓

Machining

↓

Part unclamped

↓

Part transferred

↓

Inspection

↓

PASS / FAIL

Each stage is a distinct event or operation.

The automation system determines:

when an operation starts

whether it is allowed to start

what sequence must be followed

whether the previous operation is complete

whether the part is present

whether the machine is ready

whether a fault exists

when the next operation can begin

This is why sequence logic is so important in discrete automation.

# 3. What Makes a Process "Discrete"?

There are several characteristics.

### 3.1 Individual products

The product can be identified as an individual unit.

Example:

Part A

Part B

Part C

rather than an uninterrupted quantity of material.

### 3.2 Distinct operations

The manufacturing process consists of identifiable steps.

For example:

Load

↓

Clamp

↓

Drill

↓

Unclamp

↓

Unload

Each operation has a beginning and an end.

### 3.3 Sequential behavior

Operations often need to happen in a particular order.

For example:

Clamp

↓

Clamp confirmation

↓

Spindle ON

↓

Spindle at speed

↓

Machining

↓

Spindle OFF

↓

Unclamp

The machine should not simply execute everything simultaneously because apparently even machines appreciate being told what happens first.

### 3.4 Events and states

Discrete automation commonly deals with conditions such as:

READY

RUN

STOP

FAULT

PART PRESENT

CLAMPED

CYCLE COMPLETE

PASS

FAIL

These are distinct states or events.

# 4. Typical Discrete Automation Example

Consider an automated drilling machine.

The sequence might be:

START

↓

Check safety

↓

Check machine ready

↓

Detect part

↓

Clamp part

↓

Confirm clamp

↓

Start spindle

↓

Confirm spindle ready

↓

Feed drill

↓

Reach drilling depth

↓

Retract drill

↓

Stop spindle

↓

Unclamp

↓

Part removed

↓

Cycle complete

This is a classic discrete automation problem.

The controller must continuously determine:

"What state is the machine currently in, and what condition allows it to move to the next state?"

# 5. Main Components of Discrete Automation

A typical discrete automation system contains:

DISCRETE AUTOMATION

│

┌──────────────┼──────────────┐

↓              ↓              ↓

INPUT          CONTROL        OUTPUT

│              │              │

Sensors           PLC          Actuators

Switches          CNC          Motors

Encoders          Robot        Valves

Proximity         Controller   Cylinders

Vision             │            Drives

│

Logic

│

Sequence

Let's separate these properly.

# 6. Inputs

Inputs provide information about the physical process.

Examples:

### Sensors

proximity sensor

photoelectric sensor

inductive sensor

capacitive sensor

pressure switch

temperature sensor

encoder

limit switch

vision sensor

### Digital inputs

Typical examples:

Part_Present

Clamp_Extended

Clamp_Retracted

Door_Closed

Motor_Running

Cylinder_Extended

Cycle_Start

Emergency_Stop_OK

The signal generally represents a discrete condition such as:

ON/OFF

or:

TRUE/FALSE

# 7. Outputs

Outputs allow the automation system to control physical equipment.

Examples:

Motor ON

Valve ON

Cylinder extend

Cylinder retract

Conveyor run

Clamp close

Robot start

Indicator lamp ON

Buzzer ON

The controller determines when these outputs should operate.

# 8. Controller

In Industry 3.0 discrete automation, the PLC is one of the most important controllers.

Examples from your broader study area include:

Allen-Bradley PLCs

Siemens PLCs

Mitsubishi PLCs

other industrial controllers

The PLC receives inputs, executes logic, and controls outputs.

Simplified:

Physical process

↓

Sensors

↓

PLC

↓

Logic

↓

Outputs

↓

Actuators / Machines

↓

Physical process

This forms the basic control loop.

# 9. PLC Logic in Discrete Automation

The PLC may perform:

sequence control

interlocking

permissive checking

timing

counting

position control

motor control

valve control

fault detection

machine state management

communication

safety-related coordination

For example:

IF

Part_Present = TRUE

AND

Door_Closed = TRUE

AND

Clamp_Retracted = TRUE

AND

Machine_Ready = TRUE

THEN

Clamp_Extend = TRUE

Then:

IF Clamp_Extended = TRUE

THEN

Start_Machining = TRUE

That is the essence of discrete sequence control.

# 10. Discrete Automation Is Heavily Sequence-Oriented

This is one of the most important concepts to understand.

A discrete machine frequently behaves like:

STATE 0

IDLE

↓

STATE 1

WAIT FOR PART

↓

STATE 2

CLAMP

↓

STATE 3

PROCESS

↓

STATE 4

UNCLAMP

↓

STATE 5

UNLOAD

↓

STATE 6

CYCLE COMPLETE

↓

STATE 0

This is effectively a state-based process.

The machine changes state when certain conditions are satisfied.

# 11. Example: Automated Assembly

Suppose a machine assembles a bearing into a housing.

The sequence could be:

1. Wait for housing

2. Detect housing

3. Clamp housing

4. Verify clamp

5. Detect bearing

6. Pick bearing

7. Move bearing to housing

8. Position bearing

9. Press bearing

10. Verify press position/force

11. Release component

12. Inspect assembly

13. Mark PASS/FAIL

14. Transfer assembly

15. Cycle complete

Every step is discrete.

The controller has to coordinate:

sensors

pneumatic cylinders

servo axes

robot

gripper

press

inspection system

conveyor

This is exactly where discrete automation becomes powerful.

# 12. Discrete Automation and Robotics

Robotics is commonly associated with discrete manufacturing.

A robot may:

pick a component

place a component

weld

screw

dispense

load a CNC

unload a CNC

perform inspection

palletize

transfer material

Example:

Part detected

↓

Robot receives permission

↓

Robot picks part

↓

Robot moves

↓

Robot places part

↓

Robot confirms complete

↓

PLC continues sequence

Notice the important point:

The robot and PLC may need a handshake.

For example:

PLC → Robot:

START PERMISSION

Robot → PLC:

ROBOT READY

PLC → Robot:

PART AVAILABLE

Robot → PLC:

PICK COMPLETE

PLC → Robot:

PLACE PERMISSION

Robot → PLC:

PLACE COMPLETE

Your reference later has a separate topic for Handshake, so that concept should be studied fully there rather than consuming this topic. Pasted markdown(1)

Here, just understand that discrete automation frequently depends on coordination between equipment.

# 13. Discrete Automation and CNC

CNC systems are another major example.

A CNC machine may perform:

Load part

↓

Clamp

↓

Tool selection

↓

Spindle start

↓

Tool movement

↓

Machining

↓

Tool change

↓

Machining

↓

Spindle stop

↓

Unload

The CNC controller handles the machining motion and process.

A PLC or machine-control system may coordinate:

door

chuck

hydraulic system

coolant

tool changer

loading/unloading

machine sequence

external equipment

Thus, CNC-based manufacturing is strongly associated with discrete automation.

# 14. Discrete Automation and Material Handling

Material handling is another major area.

Examples:

conveyors

elevators

transfer units

pallet systems

AGVs

robots

lifters

indexing mechanisms

Consider a conveyor system:

Sensor 1 detects part

↓

Conveyor starts

↓

Part reaches Sensor 2

↓

Conveyor stops

↓

Stopper extends

↓

Processing begins

Again, individual objects are being detected and moved.

That is discrete behavior.

# 15. Discrete Signals

Discrete automation commonly uses digital signals.

A digital signal generally has two logical states:

0 = OFF

1 = ON

or:

FALSE

TRUE

Examples:

Part present = 1

Door closed = 1

Motor running = 1

Clamp extended = 1

Fault = 0/1

This is one reason the term discrete is associated with digital control.

But an important distinction:

Discrete automation does not mean that every signal in the system must be digital.

A discrete machine can also use analog signals.

For example:

pressure

temperature

position

force

current

speed

So:

Discrete manufacturing ≠ only digital signals.

# 16. Discrete Automation Can Use Analog Data

Suppose a press machine performs an assembly operation.

The machine may use:

Part detected → digital

Clamp confirmed → digital

Press start → digital

Press force → analog

Press position → analog

Temperature → analog

The manufacturing operation is still discrete automation because the product is processed as an individual unit through distinct operations.

This distinction is important.

# 17. Discrete vs Continuous

A simple comparison:

| Discrete Automation | Continuous Process Automation |
| --- | --- |
| Individual products/units | Continuous material flow |
| Distinct operations | Continuous process |
| Strong sequence orientation | Strong process-variable orientation |
| Parts can be counted individually | Material often measured by flow/volume |
| Machines/robots/conveyors common | Pumps/valves/process equipment common |
| PLC/CNC/robot controllers common | PLC/DCS/process controllers common |
| Automotive manufacturing | Oil refining |
| Assembly | Chemical processing |
| Machining | Continuous fluid processing |

Do not confuse continuous with simply "the machine runs continuously."

A CNC can run for hours continuously while still being a discrete manufacturing process because it processes individual parts.

# 18. Example: Automotive Manufacturing

Automotive manufacturing is heavily associated with discrete automation.

Consider a body shop.

A simplified sequence:

Panel arrives

↓

Robot picks panel

↓

Positioning fixture

↓

Clamp

↓

Welding

↓

Inspection

↓

Transfer

↓

Next operation

The vehicle body is assembled from individual components.

Robots, welding guns, fixtures, sensors, PLCs, safety systems, conveyors and inspection equipment all operate together.

This is a classic discrete automation environment.

# 19. Example: Assembly Line

Consider an assembly line:

Station 1

Part loading

↓

Station 2

Component assembly

↓

Station 3

Fastening

↓

Station 4

Inspection

↓

Station 5

Testing

↓

Station 6

Unloading

Each station performs a defined operation.

The line controller coordinates:

station readiness

product movement

machine states

safety conditions

part presence

production sequence

fault conditions

This is discrete automation at the line level.

# 20. Machine-Level Discrete Automation

At one machine, you might have:

Inputs

↓

PLC

↓

Sequence logic

↓

Outputs

↓

Actuators

↓

Machine

Example:

Part sensor

↓

PLC

↓

Clamp command

↓

Solenoid valve

↓

Cylinder

↓

Clamp

↓

Clamp sensor

↓

PLC

The PLC closes the information loop.

# 21. Important Control Concepts

Discrete automation commonly depends on concepts that appear later in your Fundamentals list:

Interlock

Permissive

Sequence

Handshake

Recipe

Alarm

Fault

Trip

Diagnostic

Commissioning

FAT

SAT

Your reference explicitly lists these as separate topics later in the Fundamentals section. Pasted markdown(1)

So for this topic, understand their role broadly:

### Interlock

Prevents an operation from occurring under an unsafe or invalid condition.

Example:

Door open

→ Machine cannot start

### Permissive

A condition that must be satisfied before an operation is allowed.

Example:

Machine Ready

AND

Part Present

AND

Safety OK

→ Start permitted

### Sequence

Defines the order of operations.

Clamp

→ Process

→ Unclamp

→ Transfer

These are fundamental building blocks of discrete automation.

# 22. Discrete Automation Architecture

A typical system may look like:

HMI

│

↓

Sensors → PLC ← Communication → Robot

│       │

│       ├────────→ VFD

│       ├────────→ Servo

│       ├────────→ Solenoid valves

│       └────────→ Contactor

│

└←──────── Actuators / Machine

A more complete manufacturing system:

HMI

│

↓

PLC / Controller

/       │        \

/        │         \

Sensors      Drives     Robot

│           │          │

↓           ↓          ↓

Machine     Motors     Gripper

│

↓

Product/Part

The exact architecture varies, but the basic principle remains:

Sense → Decide → Actuate → Verify → Continue sequence

# 23. The Fundamental Discrete Automation Cycle

A very useful mental model:

1. SENSE

↓

2. DECIDE

↓

3. ACT

↓

4. VERIFY

↓

5. MOVE TO NEXT STATE

Example:

SENSE

Part present

↓

DECIDE

All permissives OK

↓

ACT

Clamp

↓

VERIFY

Clamp sensor ON

↓

NEXT STATE

Start machining

Then the process repeats.

This is the foundation of many PLC-controlled machines.

# 24. What Does "Automation" Actually Mean Here?

It is not simply:

"PLC controls machine."

Automation means that the system performs decisions and actions with reduced direct human intervention.

For example, instead of an operator manually doing:

Detect part

↓

Clamp

↓

Start machine

↓

Wait

↓

Stop machine

↓

Unclamp

↓

Remove part

the automated system performs these operations automatically.

The operator may instead:

load material

monitor the process

respond to faults

perform setup

change tooling

approve production

perform maintenance

# 25. Manual vs Semi-Automatic vs Automatic

These concepts help understand the degree of automation.

### Manual

Operator performs most operations.

Operator

↓

Machine

### Semi-automatic

Operator initiates or participates in some steps.

Operator

↓

Automatic sequence

↓

Operator intervention

↓

Automatic sequence

### Automatic

The system performs the defined sequence automatically.

Part detected

↓

Automatic sequence

↓

Automatic processing

↓

Automatic inspection

↓

Automatic transfer

The exact boundaries depend on the machine.

# 26. Discrete Automation and Quality

Discrete automation is particularly useful when each unit needs to be:

inspected

measured

identified

tracked

accepted/rejected

serialized

tested

Example:

Part

↓

Machining

↓

Measurement

↓

PASS?

├── YES → Production

└── NO  → Reject

A vision system could inspect:

presence

orientation

dimensions

surface condition

assembly correctness

markings

The automation system can then make a discrete decision:

PASS

or

FAIL

# 27. Discrete Automation and Traceability

Individual products make traceability especially useful.

Example:

Part ID: 10542

Operation 1 → PASS

Operation 2 → PASS

Operation 3 → PASS

Inspection → PASS

Final test → PASS

The system can associate process information with the individual part.

This becomes increasingly important when you later study Industry 4.0, databases, IIoT and production dashboards.

# 28. Discrete Automation and Maintenance

For maintenance, a discrete machine can be viewed as:

INPUT

↓

CONTROL

↓

SEQUENCE

↓

ACTUATION

↓

PROCESS

↓

FEEDBACK

A fault can occur anywhere.

Example:

### Input problem

Part sensor fails.

Result:

PLC never detects part

→ sequence does not start

### Control problem

PLC logic/configuration problem.

Result:

Correct input

→ incorrect decision

→ wrong output

### Output problem

PLC commands valve ON but output hardware fails.

Result:

PLC output = ON

but actuator = OFF

### Actuator problem

Solenoid operates but cylinder cannot move.

### Feedback problem

Cylinder physically reaches position, but sensor fails to report it.

Result:

Machine waits indefinitely

This is why troubleshooting discrete automation requires understanding the entire chain, not merely looking at the PLC program.

# 29. A Practical Troubleshooting Chain

When a discrete machine does not perform an operation:

1. Is the required condition actually present?

↓

2. Is the input reaching the controller?

↓

3. Is the logic allowing the operation?

↓

4. Is the output being generated?

↓

5. Is the actuator responding?

↓

6. Is the physical mechanism moving?

↓

7. Is feedback confirming the movement?

↓

8. Is the sequence advancing?

This is extremely useful in real maintenance work.

# 30. Discrete Automation and PLC Programming

Common PLC programming tasks include:

digital I/O

timers

counters

sequence logic

state machines

interlocks

permissives

alarms

fault handling

motor control

valve control

communication

analog processing

position coordination

Typical programming languages may include:

Ladder Diagram

Function Block Diagram

Structured Text

Sequential Function Chart

The exact language depends on the PLC/platform.

# 31. Discrete Automation Does Not Mean "Simple"

This is an important misconception.

A discrete system can become extremely complex.

For example, an automotive production line can contain:

Hundreds of sensors

Hundreds of actuators

Multiple PLCs

Multiple robots

CNC machines

Vision systems

Safety controllers

Servo systems

Industrial networks

HMIs

SCADA

Databases

Traceability

MES integration

The manufacturing process is still discrete.

So:

Discrete ≠ simple

and:

Continuous ≠ necessarily more advanced

They describe the nature of the manufacturing process, not its technological sophistication.

# 32. Discrete Automation vs Discrete Manufacturing

These two terms should also be separated.

### Discrete manufacturing

Describes the type of manufacturing process.

Example:

Automobile manufacturing

Machine component manufacturing

Electronic assembly

### Discrete automation

Describes the automation used to control and execute that manufacturing process.

Example:

PLC

Robot

Servo

CNC

Sensors

Vision

Conveyors

Control logic

So:

Discrete manufacturing = what kind of production

Discrete automation = how that production is automated

# 33. Where Discrete Automation Fits in Your Study

Your Fundamentals sequence is deliberately structured like this:

01 Evolution

↓

02 Architecture

↓

03 Hierarchy

↓

04 Discrete

↓

05 Process

↓

06 Batch

↓

07 Fixed / Programmed / Flexible

The progression makes sense:

Evolution

→ How automation developed

Architecture

→ How an automation system is structured

Hierarchy

→ At what manufacturing scope the system operates

Discrete

→ Automation of individual-unit manufacturing

Process

→ Automation of continuous process behavior

Batch

→ Automation of batch-based production

Fixed / Programmed / Flexible

→ Different approaches to automation flexibility

So don't drag Process Automation into this topic too deeply yet. Its dedicated slot is immediately next in your study structure. Pasted markdown(1)

# 34. The Most Important Difference: Discrete vs Process

Keep this comparison ready:

DISCRETE

Part

↓

Operation

↓

Part

↓

Operation

↓

Finished product

versus:

PROCESS

Raw material

↓

Continuous process

↓

Controlled variables

↓

Finished material

Discrete automation often asks:

What should happen next?

Process automation often asks:

What should the process variable be maintained at?

Examples of discrete concerns:

Is the part present?

Is the clamp closed?

Is the door closed?

Is the robot ready?

Has the operation completed?

Should the conveyor start?

Examples of process concerns:

Is temperature at the setpoint?

Is pressure maintained?

Is flow correct?

Is level controlled?

Is composition within limits?

There can absolutely be overlap, but this distinction is an excellent foundation.

# 35. The One Mental Model to Remember

For your career-level understanding, remember:

DISCRETE AUTOMATION

Individual Product / Part

│

↓

Detect / Identify

│

↓

Sequence

│

↓

Interlocks

│

↓

Permissives

│

↓

Actuate

│

↓

Process

│

↓

Verify

│

↓

PASS / FAIL

│

↓

Next Operation

And underneath it:

Sensors

↓

PLC / Controller

↓

Logic / Sequence

↓

Outputs

↓

Actuators

↓

Machine

↓

Product

↓

Feedback

└────────────→ PLC

That loop is the heart of discrete automation.

# 36. Interview-Level Questions

Q1. What is discrete automation?

Discrete automation is the automation of manufacturing processes involving individual products or units and distinct, sequenced operations.

Q2. Give examples of discrete automation.

Automated assembly, CNC machining, robotic welding, packaging, material handling, automated inspection and testing.

Q3. Why is PLC widely used in discrete automation?

Because discrete machines require sequencing, logical decisions, interlocks, timing, counting, I/O control and coordination of multiple devices.

Q4. Is discrete automation limited to digital signals?

No. Discrete automation can use both digital and analog signals. The term refers primarily to the nature of the manufacturing process, not exclusively to signal type.

Q5. Is CNC discrete automation?

Yes. CNC machining processes individual workpieces through defined machining operations.

Q6. Is robotics part of discrete automation?

Yes, robots are commonly used in discrete manufacturing for handling, assembly, welding, inspection and other operations.

Q7. What is the basic control principle?

Sense → Decide → Act → Verify → Continue sequence.

Q8. What is the difference between discrete manufacturing and continuous manufacturing?

Discrete manufacturing processes individual identifiable units through distinct operations, while continuous manufacturing processes material continuously through a process.

Q9. Can a discrete automation system contain analog signals?

Yes. For example, a discrete assembly machine may use analog force, pressure, temperature or position measurements.

Q10. Does discrete mean simple?

No. Discrete automation systems can contain highly complex PLC, robot, CNC, servo, vision, safety and networking systems.

# 37. Final Consolidated Definition

Discrete automation is the use of automatic control systems to manufacture, assemble, handle, inspect, test, or process individual products or units through defined and usually sequential operations.

The core characteristics are:

Individual units

→ distinct operations

→ sequence

→ sensing

→ decision/logic

→ actuation

→ feedback/verification

→ next operation

For your Industry 3.0 foundation, the most important mental chain is:

Part → Sensor → PLC/Controller → Sequence Logic → Actuator → Operation → Feedback → Next State

And the distinction to retain before moving to the next topic is:

Discrete automation deals primarily with individual units and distinct operations; process automation deals primarily with continuously varying process conditions.

Your reference places `05_Process → Process automation` directly after this topic, so that is the next logical comparison point in the study sequence. Pasted markdown(1)