# 06. Batch Automation

Your study index places 06_Batch → Batch automation after Discrete Automation and Process Automation. The index itself provides only the topic name, so the detailed explanation below is foundational industrial-automation knowledge rather than additional material explicitly contained in the index.

## 1. What is Batch Automation?

Batch automation is the automatic production of a finite quantity of material or product according to a defined recipe, procedure, sequence, and set of process conditions.

The important word is finite.

A batch has:

Start → Processing → Completion

For example:

1,000 kg of Paint-A is produced according to Recipe-PA-01.

The system starts the batch, verifies the required conditions, adds the required materials, controls temperature/flow/pressure/agitation, completes the required sequence, records the results, and finally declares the batch complete.

So:

Batch Automation = Recipe + Sequence + Process Control + Material/Equipment Management + Traceability

# 2. Why Batch Automation is Different

There are three important automation categories you are studying:

| Type | Basic idea | Typical example |
| --- | --- | --- |
| Discrete Automation | Individual items/events | Assembly line |
| Process Automation | Continuous physical process | Continuous water flow |
| Batch Automation | Finite quantity made according to recipe | 5,000 L chemical batch |

The distinction is not always about the equipment.

The same tank could participate in process automation or batch automation depending on how it is operated.

For example:

### Continuous process

Raw material continuously enters:

Feed → Tank/Process → Product

There may be no meaningful "Batch #125" boundary.

### Batch process

A defined quantity is produced:

Batch #125 → Charge → Mix → Heat → Hold → Cool → Discharge

Then the equipment becomes available for another batch.

# 3. The Core Concept

The simplest mental model is:

RECIPE

↓

BATCH ID

↓

MATERIAL / EQUIPMENT CHECK

↓

CHARGE / LOAD

↓

EXECUTE SEQUENCE

↓

CONTROL PROCESS VARIABLES

↓

MONITOR

↓

HOLD / INTERVENTION IF REQUIRED

↓

COMPLETE

↓

DISCHARGE / TRANSFER

↓

RECORD BATCH DATA

↓

CLEAN / CHANGEOVER

↓

NEXT BATCH

This is the fundamental structure you should remember.

# 4. What is a Batch?

A batch is a distinct quantity of product produced or processed under defined conditions.

Example:

Batch ID      : PAINT-2026-00125

Product       : Red Industrial Paint

Quantity      : 2,000 kg

Recipe        : RED_PAINT_R03

Equipment     : MIX-TANK-05

Start Time    : 08:30

End Time      : 11:45

Operator      : Operator-07

Status        : COMPLETE

The Batch ID is extremely important because it provides traceability.

If a quality problem appears later, the manufacturer should be able to determine:

which materials were used

which equipment was used

which recipe/version was used

who operated it

when it was produced

what process values occurred

whether alarms occurred

whether the batch was held

whether parameters were changed

what the final quality result was

# 5. Recipe

The recipe is one of the most important concepts in batch automation.

A recipe defines how a particular product should be produced.

For example:

### Chocolate batch recipe

Product       : Chocolate-A

Batch Size    : 500 kg

Ingredient A  : 250 kg

Ingredient B  : 150 kg

Ingredient C  : 100 kg

Mix Speed     : 80 RPM

Heating SP    : 65 °C

Hold Time     : 20 min

Cooling SP    : 30 °C

The automation system uses this information to execute the batch.

A recipe can contain:

material quantities

process setpoints

temperatures

pressures

flow rates

mixing speeds

timing

sequence

limits

equipment requirements

operating parameters

product-specific instructions

# 6. Recipe vs Batch

These are not the same thing.

### Recipe

Defines what should be done.

### Batch

Represents one actual execution of that recipe.

Example:

Recipe:

PAINT_RED_R03

Batch:

PAINT_RED_2026_001

Batch:

PAINT_RED_2026_002

Batch:

PAINT_RED_2026_003

One recipe can therefore produce many batches.

Think:

Recipe = instruction/template

Batch = actual production instance

This distinction becomes very important in MES, DCS and batch-management systems.

# 7. Batch Sequence

A batch is normally executed through a sequence of operations.

Example:

START

↓

Check equipment

↓

Check raw materials

↓

Charge Material A

↓

Charge Material B

↓

Start Agitator

↓

Heat to 70°C

↓

Hold 30 min

↓

Add Material C

↓

Mix 15 min

↓

Cool to 35°C

↓

Quality Check

↓

Discharge

↓

COMPLETE

This is where discrete automation and process automation meet.

# 8. Batch Automation Combines Discrete + Process Automation

This is one of the most important concepts.

A batch system can contain both.

For example:

### Discrete control

Open Valve V101

↓

Wait for open feedback

↓

Start Pump P101

↓

Wait for running feedback

↓

Close Valve V102

These are discrete events.

But simultaneously:

### Process control

Temperature SP = 70°C

Pressure SP = 4 bar

Flow SP = 100 L/min

The controller continuously regulates these variables.

Therefore:

Batch automation is often a combination of sequence control and process control.

# 9. Example: Automated Mixing Tank

Consider an industrial mixing tank.

Equipment:

Tank

Agitator

Heating system

Cooling system

Temperature transmitter

Level transmitter

Flowmeter

Control valves

Pump

PLC/DCS

HMI

Batch sequence:

1. Verify tank empty

2. Verify valves in correct state

3. Add Material A

4. Verify required quantity

5. Add Material B

6. Start agitator

7. Heat to 80°C

8. Maintain 80°C for 30 min

9. Add Material C

10. Mix for 15 min

11. Cool to 40°C

12. Transfer product

13. Record batch completion

14. Clean tank

Notice what is happening.

The sequence determines what happens next.

The process controller determines how the physical variables are controlled.

# 10. Batch Phases

A large batch process is usually broken into smaller executable sections called phases.

For example:

Batch

│

├── Charge Raw Material

├── Heating

├── Mixing

├── Reaction

├── Cooling

├── Quality Check

└── Discharge

Each phase may have:

starting conditions

commands

setpoints

feedback

completion conditions

timeout

alarm conditions

abort conditions

Example:

### Heating phase

START HEATING

↓

Check Level OK

↓

Open Heating Valve

↓

Control Temperature

↓

Temperature reaches 80°C

↓

Hold 30 minutes

↓

PHASE COMPLETE

# 11. Equipment Conditions

Before executing a batch, the automation system may verify that equipment is ready.

For example:

Tank available = TRUE

Agitator healthy = TRUE

Temperature sensor healthy = TRUE

Level sensor healthy = TRUE

Required valves available = TRUE

No active critical alarm

Emergency stop healthy

Only then:

BATCH START PERMITTED

This connects directly with concepts you will study later such as:

permissive

interlock

alarm

fault

trip

Do not mix those concepts together yet. They deserve their own topics later in your index.

# 12. Material Verification

Batch manufacturing frequently requires material traceability.

Suppose the recipe requires:

Material A = 500 kg

Material B = 100 kg

Material C = 50 kg

The system may verify:

Material identity

Quantity

Lot number

Expiry

Availability

Correct storage/location

This prevents an operator from accidentally charging the wrong material.

# 13. Weighing and Dosing

Many batch processes require accurate dosing.

Typical equipment:

Load cells

Weighing systems

Flowmeters

Dosing pumps

Screw feeders

Gravimetric feeders

Batch scales

Example:

Required quantity = 500 kg

Start dosing

↓

Load cell measurement

↓

PV = 450 kg

↓

Continue

↓

PV = 495 kg

↓

Slow dosing

↓

PV = 500 kg

↓

Stop dosing

This is a combination of:

measurement + sequence + control + recipe parameter

# 14. Batch States

A batch system needs to know its current state.

Typical states include:

IDLE

SCHEDULED

READY

RUNNING

HELD

ABORTED

COMPLETED

For example:

IDLE

↓

READY

↓

RUNNING

↓

HELD

↓

RUNNING

↓

COMPLETED

A batch may be placed on HOLD because:

operator intervention is required

raw material is unavailable

process value is outside an allowed condition

quality inspection is required

equipment issue occurred

downstream equipment is unavailable

The exact state model depends on the implementation.

# 15. Hold vs Abort

These are conceptually different.

### Hold

The batch temporarily stops progressing.

Example:

Temperature not reaching required value

↓

HOLD

↓

Maintenance investigates heater

↓

Problem corrected

↓

RESUME

### Abort

The batch is terminated rather than normally completed.

Example:

Major equipment failure

↓

ABORT

↓

Batch terminated

The material may then require:

disposal

reprocessing

investigation

deviation handling

# 16. Batch Traceability

One of the biggest reasons for automation is traceability.

A system may record:

Batch ID

Recipe ID

Recipe version

Material lot numbers

Equipment ID

Operator ID

Start time

End time

Process values

Setpoints

Valve states

Motor states

Alarms

Holds

Operator actions

Manual interventions

Quality results

This creates the batch history.

# 17. Batch Record

The final batch record can answer:

What happened during this batch?

For example:

Batch: CHEM-2026-00421

Recipe: CHEM-R12

Equipment: Reactor-03

Start: 09:10

End: 14:35

Material A: LOT-A127

Material B: LOT-B431

Maximum temperature: 82.4°C

Required temperature: 80°C

Mixing time: 45 min

Alarm:

09:58 - Cooling valve response slow

Hold:

10:02 - 10:17

Final status:

COMPLETED

This becomes extremely important in industries where quality and regulatory traceability matter.

# 18. Batch Automation Architecture

A simplified architecture is:

MES / Production System

│

↓

Batch / Recipe Management

│

↓

HMI / SCADA

│

↓

PLC / DCS Controller

│

┌──────────────┴──────────────┐

↓                             ↓

Sequence Control             PID Control

│                             │

↓                             ↓

Valves / Motors              Temperature

Pumps / Devices               Pressure

Flow / Level

│                             │

└──────────────┬──────────────┘

↓

PROCESS

In more advanced plants, historians, MES, ERP and analytics systems are integrated above this architecture.

# 19. PLC in Batch Automation

A PLC can execute:

sequence logic

equipment control

valve control

motor control

interlocks

permissives

phase execution

analog control

PID

equipment status

communication

For example:

IF Phase = HEATING

AND Tank_Level_OK

AND Heater_Available

THEN

Enable_Heating

Then:

IF Temperature >= 80°C

THEN

Heating_Phase_Complete

This is basic batch sequence logic.

# 20. DCS in Batch Automation

DCS is common in process-oriented industries and can provide:

continuous control

PID loops

sequence control

operator interfaces

alarms

historical data

batch management capabilities

But remember:

Batch automation does not mean "DCS automation."

A batch system can be implemented using:

PLC

DCS

industrial PC

dedicated batch-control software

combinations of these

The architecture depends on the application.

# 21. Batch Automation and MES

A useful hierarchy is:

ERP

│

↓

MES

│

↓

Batch Management

│

↓

PLC / DCS

│

↓

Field Devices

For example:

### ERP

"Produce 10,000 kg of Product A."

### MES

Creates production order and batch requirements.

### Batch system

Selects recipe and executes the batch.

### PLC/DCS

Controls actual equipment.

### Field devices

Perform the physical work.

# 22. Batch Automation and SCADA

SCADA/HMI provides visibility and operator interaction.

The operator may see:

BATCH: 2026-00125

Phase:

HEATING

Temperature:

72.4°C

Setpoint:

80°C

Agitator:

RUNNING

Heating Valve:

65%

Level:

72%

Batch Status:

RUNNING

The operator can also receive:

alarms

prompts

phase status

equipment status

batch status

operator instructions

# 23. Campaign Production

A campaign means producing multiple batches of the same or related product before changing the setup.

Example:

Campaign: RED PAINT

Batch 001

Batch 002

Batch 003

Batch 004

Batch 005

↓

Changeover

↓

BLUE PAINT

Why?

Because changing products may require:

cleaning

flushing

tool/setup change

recipe change

material change

quality verification

Reducing unnecessary changeovers improves production efficiency.

# 24. Changeover

After a batch finishes, the equipment may need to be prepared for another product.

Typical sequence:

Batch Complete

↓

Drain

↓

Clean

↓

Flush

↓

Inspect

↓

Configure

↓

Load New Recipe

↓

Verify

↓

Next Batch

For some industries, cleaning itself is an automated sequence.

This is particularly important when contamination between products is unacceptable.

# 25. Batch Automation Example: Pharmaceutical

Suppose a liquid pharmaceutical product requires:

Water       = 500 L

Ingredient A = 20 kg

Ingredient B = 5 kg

Mixing      = 30 min

Temperature = 40°C

Automation:

Verify equipment

↓

Verify materials

↓

Charge water

↓

Verify 500 L

↓

Start agitator

↓

Add Ingredient A

↓

Mix

↓

Add Ingredient B

↓

Control temperature at 40°C

↓

Mix for 30 min

↓

Quality check

↓

Transfer

↓

Batch record

That is batch automation.

# 26. Batch Automation Example: Heat Treatment

A heat-treatment machine is another excellent example because it demonstrates all three automation concepts.

Suppose a component must undergo:

Load component

↓

Close door

↓

Heat to 900°C

↓

Hold 30 min

↓

Cool to 500°C

↓

Quench

↓

Unload

### Discrete automation

door open/close

component presence

valve ON/OFF

pump ON/OFF

sequence

### Process automation

temperature control

pressure control

flow control

### Batch automation

one defined treatment cycle

defined recipe

defined start/end

batch/cycle record

traceability

This is why these three categories should not be treated as completely separate worlds.

# 27. Batch vs Discrete vs Process

| Feature | Discrete | Process | Batch |
| --- | --- | --- | --- |
| Production form | Individual units | Continuous material | Finite quantity |
| Main concern | Sequence | Regulation | Recipe + sequence + process |
| Start/end | Unit/cycle | Often continuous | Clearly defined batch |
| Typical control | Logic | PID/continuous | Logic + PID |
| Recipe | Sometimes | Sometimes | Very important |
| Batch ID | Usually less central | Usually not central | Very important |
| Traceability | Unit-based | Process/time-based | Batch-based |
| Example | Car assembly | Water treatment | Chemical batch |
| Typical data | State/event | Trends | Recipe + process + history |

# 28. Most Important Difference

A useful way to remember all three:

### Discrete Automation

Which event happens next?

Clamp → Drill → Retract → Unclamp

### Process Automation

What should the physical variable be?

Temperature = 80°C

Pressure = 5 bar

Flow = 100 L/min

### Batch Automation

How do I produce this defined quantity according to this recipe?

Recipe

↓

Charge

↓

Mix

↓

Heat

↓

Hold

↓

Cool

↓

Discharge

# 29. Maintenance Perspective

For your maintenance work, batch automation is particularly important because a batch failure can originate from either sequence logic or physical process control.

Suppose:

Batch stops during heating.

Do not immediately blame the PLC, because humans have historically enjoyed replacing expensive electronics when a temperature transmitter was simply lying.

Trace the chain:

Batch Phase

↓

Heating command?

↓

Controller output?

↓

Heating valve/heater command?

↓

Final element actually operating?

↓

Temperature changing?

↓

Temperature sensor correct?

↓

Transmitter signal correct?

↓

PLC/DCS PV correct?

↓

Recipe setpoint correct?

↓

Sequence completion condition satisfied?

Possible faults:

Sensor fault

Transmitter fault

Analog input fault

Scaling error

PLC logic problem

Recipe parameter error

Valve fault

Heater fault

VFD fault

Pump fault

Communication failure

Permissive not satisfied

Interlock active

Sequence timeout

# 30. Example Maintenance Fault

Suppose the recipe says:

Heat to 80°C

Actual temperature:

80°C

But HMI shows:

68°C

The batch does not proceed because:

PV < 80°C

The process itself may be perfectly healthy.

Troubleshooting:

Actual temperature

↓

Sensor

↓

Transmitter

↓

4–20 mA signal

↓

Analog input

↓

PLC/DCS scaling

↓

Program tag

↓

HMI tag

This is exactly where your electronic/PLC maintenance background becomes useful.

# 31. Batch Data in Industry 4.0

Batch automation produces valuable historical data.

For example:

Batch ID

Timestamp

Temperature

Pressure

Flow

Level

Valve position

Motor speed

Recipe

Alarm

Phase

Operator action

Quality result

That data can move into:

PLC / DCS

↓

SCADA

↓

Historian / SQL

↓

Dashboard

↓

Analytics

↓

AI / ML

This connects directly with the SCADA → SQL → Grafana architecture you have been working on.

For batch production, instead of merely storing:

Temperature = 72.4

you can store:

Batch_ID

Phase

Tag_Name

Value

Timestamp

Recipe_ID

Equipment_ID

That gives the data context.

# 32. Why Batch Context Matters

Consider two identical temperature values:

Temperature = 75°C

Without context, the value tells you very little.

But:

Batch ID     = B20261006-015

Phase        = HEATING

Recipe       = PRODUCT_A_R03

Equipment    = TANK_05

SP            = 80°C

PV            = 75°C

Timestamp     = 10:32:14

Now the data is meaningful.

This is one of the major transitions from basic automation toward Industry 4.0.

# 33. Important Batch Terms

You should become comfortable with these terms:

Batch

A finite quantity processed under defined conditions.

Recipe

Instructions and parameters required to produce a product.

Batch ID

Unique identifier for a production batch.

Phase

A defined executable portion of a batch procedure.

Operation

A logical group of activities within the production procedure.

Equipment

Physical production equipment used by the batch.

Batch record

Historical record of what occurred during the batch.

Traceability

Ability to determine what materials, equipment, parameters and events were associated with a batch.

Hold

Temporary suspension of batch progression.

Abort

Termination of the batch.

Campaign

Production of multiple related batches before changeover.

Changeover

Preparation of equipment from one product/process to another.

Recipe version

Specific revision of the recipe used for a batch.

# 34. ISA-88 / S88 Connection

As you progress into advanced industrial automation, you will encounter ISA-88, commonly called S88, which provides a structured approach to batch control.

A simplified conceptual hierarchy is:

PROCESS CELL

↓

UNIT

↓

EQUIPMENT MODULE

↓

CONTROL MODULE

And on the procedural side:

PROCEDURE

↓

UNIT PROCEDURE

↓

OPERATION

↓

PHASE

You do not need to memorize the complete S88 standard at this stage.

The important idea is:

Separate the production procedure from the physical equipment so that recipes and automation can be structured and reused.

This becomes increasingly important in large batch systems.

# 35. Reusable Equipment

Suppose the same mixing tank can produce:

Product A

Product B

Product C

Instead of creating completely different physical control systems, the equipment can provide reusable capabilities:

Start Agitator

Stop Agitator

Heat

Cool

Add Material

Transfer

Clean

Different recipes can then use those capabilities differently.

This is one of the reasons structured batch automation becomes powerful at scale.

# 36. Common Batch Automation Applications

Typical industries include:

Pharmaceutical manufacturing

Chemical manufacturing

Specialty chemicals

Paints and coatings

Food and beverage

Cosmetics

Plastics and polymers

Biotechnology

Water treatment

Cement/material processing

Heat treatment

Mixing/blending operations

The exact automation architecture varies substantially by industry.

# 37. Interview-Level Questions

### Q1. What is batch automation?

Answer:

Batch automation automatically produces a finite quantity of product according to a defined recipe, sequence, process parameters and equipment conditions, while maintaining batch status and traceability.

### Q2. What is the difference between recipe and batch?

Answer:

A recipe defines how a product should be produced. A batch is one actual production execution of that recipe.

### Q3. Can batch automation use PLCs?

Yes. PLCs are widely used for sequence control, equipment control, analog control, interlocks, permissives and phase execution.

### Q4. Can batch automation use DCS?

Yes. DCS platforms are commonly used in process-oriented batch applications and can provide continuous control, sequence control, operator interfaces, alarms, history and batch functionality.

### Q5. Does batch automation mean only sequence control?

No.

Batch automation commonly combines:

Sequence control + continuous/process control + recipe management + batch tracking.

### Q6. What is a batch record?

A historical record containing information about a particular batch, such as recipe, materials, equipment, process values, events, alarms, operator actions and completion information.

### Q7. Why is Batch ID important?

It provides unique identification and traceability for a specific production execution.

### Q8. What happens when a batch is put on hold?

The batch temporarily stops progressing until the required condition or intervention is resolved and the batch is allowed to resume.

### Q9. What is campaign production?

Producing multiple batches of the same or related product before performing a major changeover.

# 38. Final Mental Model

Keep this in your head:

RECIPE

↓

BATCH ID

↓

MATERIAL + EQUIPMENT CHECK

↓

CHARGE

↓

EXECUTE PHASES

↓

┌───────────┴───────────┐

↓                       ↓

SEQUENCE CONTROL       PROCESS CONTROL

↓                       ↓

Valves/Motors/etc.       T/P/F/L/etc.

└───────────┬───────────┘

↓

MONITOR / RECORD

↓

HOLD / RESUME IF REQUIRED

↓

COMPLETE

↓

DISCHARGE / TRANSFER

↓

BATCH RECORD

↓

CLEAN / CHANGEOVER

↓

NEXT BATCH

The three foundational concepts can therefore be reduced to:

Discrete = individual events and sequences

Process = continuous physical-variable control

Batch = finite production according to a recipe, combining sequence and process control

And the most important batch-specific concept is:

Recipe describes the intended production method. Batch records one actual execution of that method.

The next topic in your index is 07_Fixed_Programmed_Flexible → Fixed / Programmed / Flexible Automation, which moves from what type of process is being automated toward how adaptable the automation system is.