# 02_PLAN_WISE_2026\02_INDUSTRY3_0\04_Installation\04_Robot_Install\05_Tool_Install - Tool installation

## 1. Definition and purpose

Robot tool installation is the process of selecting, mechanically mounting, electrically and pneumatically connecting, configuring, and validating the end-effector attached to an industrial robot.

The tool is the device that interacts with the workpiece or performs the manufacturing operation. It is usually mounted at the robot wrist or flange.

Examples include:

Grippers for picking and placing components.

Welding guns and torches.

Vacuum cups and vacuum grippers.

Dispensing guns for adhesive or sealant.

Drilling, deburring, grinding, and polishing tools.

Nut runners and fastening tools.

Inspection cameras and measurement sensors.

Assembly tools and specialised process heads.

Tool installation is not complete merely because the tool is bolted to the robot. The installation must also satisfy mechanical, electrical, pneumatic, process, safety, and programming requirements.

The main objectives are:

Securely mount the tool to the robot flange.

Maintain the required tool position and orientation.

Keep the complete tool assembly within the robot's payload and inertia limits.

Connect power, signals, air, vacuum, cooling, and process services correctly.

Configure the tool centre point (TCP).

Define tool mass, centre of gravity, and inertia as required.

Verify gripping or process functionality.

Prevent collisions, dropped parts, cable damage, and unsafe motion.

Document the installation and configuration for future maintenance.

## 2. Robot tool system architecture

Robot Controller

|

+--> Motion Control and Tool Configuration

|

Robot Manipulator

|

Robot Wrist / Mounting Flange

|

Tool Adapter / Mounting Plate

|

End-Effector

|

+--> Gripper / Welding Gun / Process Tool

|

+--> Sensors and Tool I/O

|

+--> Pneumatic / Vacuum / Cooling Services

|

+--> Workpiece or Manufacturing Process

The actual arrangement depends on the robot model and application. Some tools connect directly to the robot flange, while others use adapters, quick changers, dress packs, or integrated service interfaces.

## 3. Tool installation terminology

| Term | Meaning | Why it matters |
| --- | --- | --- |
| End-effector | Device attached to the robot to perform the task | Defines the robot's working function |
| Robot flange | Mechanical interface at the robot wrist | Determines mounting geometry and compatibility |
| Tool adapter | Intermediate plate or interface between robot and tool | Provides correct mounting or orientation |
| Tool Centre Point (TCP) | Defined reference point used to describe the tool's position and orientation | Essential for accurate Cartesian motion |
| Centre of gravity (CoG) | Point representing the mass distribution of the tool assembly | Affects robot loading and motion |
| Payload | Mass carried by the robot under the manufacturer's specified definition | Must remain within the robot's approved limits |
| Moment of inertia | Measure of resistance to angular acceleration about an axis | Affects dynamic loading and motion performance |
| Tool frame | Coordinate frame describing the tool relative to the robot flange | Used in programming and motion control |
| Tool changer | Mechanism that connects and releases interchangeable tools | Requires secure locking and correct utility connections |
| Dress pack | Cables, hoses, and supports routed to the tool | Must accommodate the complete movement envelope |

## 4. Types of robot tools

### 4.1 Mechanical grippers

Mechanical grippers hold a workpiece using fingers, jaws, or other mechanical contact surfaces.

Common types include:

Parallel grippers.

Angular grippers.

Three-finger grippers.

Custom fixture-style grippers.

Servo-electric grippers.

Pneumatic grippers.

Selection depends on part geometry, mass, surface condition, gripping force, cycle time, repeatability, and the risk of damaging the component.

### 4.2 Vacuum grippers

Vacuum tools use a pressure difference to hold a component against a suction surface.

Common components include:

Vacuum cups.

Vacuum ejectors or pumps.

Vacuum switches or pressure sensors.

Filters.

Valves.

Vacuum reservoirs where required.

Release or blow-off circuits.

Vacuum tooling must be evaluated for surface porosity, leakage, acceleration, cup placement, and the consequences of vacuum loss.

### 4.3 Process tools

Process tools perform manufacturing operations rather than simply gripping parts.

Examples include:

Spot-welding guns.

Arc-welding torches.

Adhesive and sealant dispensers.

Screwdriving systems.

Drilling or cutting heads.

Grinding and polishing spindles.

Inspection and vision systems.

These tools may require additional cooling, process gas, welding power, extraction, force control, or specialised safety measures.

## 5. Pre-installation engineering checks

Before mounting the tool, verify the complete tool assembly and its compatibility with the robot.

Check:

Robot model and wrist-flange specification.

Tool model and mounting drawing.

Adapter dimensions and orientation.

Tool mass and centre of gravity.

Inertia requirements.

Required robot payload configuration.

Fastener type, size, and tightening torque.

Tool clearance and collision envelope.

Required cables, hoses, and connectors.

Pneumatic pressure and flow requirements.

Electrical voltage and current ratings.

Process-specific cooling or supply requirements.

Sensor locations and tool I/O.

Required guarding and risk controls.

Access for tool maintenance and replacement.

The payload assessment must consider the complete assembly, not just the bare gripper or process head.

A complete assembly may include the tool, adapter, sensors, brackets, valves, connectors, cables, hoses, and any other equipment carried by the robot.

## 6. Mechanical mounting at the robot flange

The robot flange provides the mechanical interface for mounting the end-effector. The flange geometry, locating features, and fastener arrangement are model-specific.

### 6.1 Mounting requirements

Confirm the correct flange pattern.

Verify the tool and adapter mounting dimensions.

Inspect mating surfaces for contamination or damage.

Check locating pins or centring features where specified.

Use the approved fasteners and washers.

Follow the specified tightening sequence and torque.

Prevent unintended rotation or movement of the tool.

Ensure the tool does not obstruct robot joints or covers.

Verify the orientation against the design drawing.

Confirm that the tool's utility connections are accessible.

Do not use longer bolts simply because they are available. Incorrect bolt length can cause interference, inadequate engagement, or damage to internal components.

### 6.2 Installation procedure

1.	Place the robot in the approved installation state.

2.	Isolate hazardous energy as required by the task and site procedure.

3.	Support the tool with suitable handling equipment.

4.	Verify the flange and tool mounting surfaces.

5.	Align the adapter and locating features.

6.	Install the specified fasteners.

7.	Tighten using the approved torque and sequence.

8.	Check the mechanical security of the assembly.

9.	Verify tool orientation and surrounding clearances.

10.	Record the installation details.

Heavy tools should not be manually supported in a way that creates a risk of crushing, dropping, or uncontrolled robot movement.

## 7. Fasteners and mechanical integrity

Tool fasteners must withstand the static and dynamic loads produced during robot movement and process operation.

Important considerations include:

Fastener grade and material.

Thread engagement.

Tightening torque.

Friction and surface condition.

Vibration and repeated acceleration.

Tool mass and offset.

Process reaction forces.

Maintenance and inspection requirements.

An incorrectly tightened fastener may loosen under repeated motion. Excessive tightening can damage threads or fasteners and can distort the mounting interface.

Where required, use the specified locking method, such as an approved locking fastener, locking compound, or mechanical locking feature. Do not substitute an improvised method for the manufacturer's design.

## 8. Payload, centre of gravity, and inertia

Robot loading is a critical part of tool installation.

### 8.1 Payload

The robot manufacturer's payload definition and limits must be checked for the particular model. Payload capability may depend on wrist loading, speed, acceleration, mounting orientation, and other restrictions.

A tool may be below the robot's nominal payload mass limit yet still violate an allowable centre-of-gravity or inertia limit.

### 8.2 Centre of gravity

The centre of gravity describes where the assembly's mass is concentrated.

A heavy tool mounted far from the wrist creates a greater moment than the same mass mounted close to the wrist.

For a simplified static load, the moment magnitude can be expressed as:

M=F×d

Where:

M is the moment.

F is the force.

d is the perpendicular distance from the reference axis to the force line of action.

This simplified relationship helps explain why tool offset matters. Actual robot loading calculations must follow the manufacturer's model and coordinate conventions.

### 8.3 Tool inertia

Moment of inertia affects the torque required to accelerate or decelerate a tool about an axis.

In general, a tool with mass distributed farther from the rotation axis has greater rotational inertia than a tool of the same mass concentrated close to the axis.

Incorrect inertia data may contribute to:

Reduced motion performance.

Excessive vibration.

Servo or collision-monitoring faults.

Increased mechanical loading.

Poor path accuracy.

### 8.4 Payload configuration

After installing the tool:

1.	Determine the complete tool assembly's mass and centre of gravity.

2.	Obtain inertia data where required.

3.	Enter the approved payload parameters into the controller.

4.	Confirm the active tool and payload configuration.

5.	Verify any required load-identification or calibration procedure.

6.	Check for alarms or performance restrictions.

7.	Test under the approved commissioning conditions.

Never guess payload values to clear a controller alarm.

## 9. Tool Centre Point (TCP)

The Tool Centre Point (TCP) is a defined reference point and orientation associated with the end-effector. It allows the controller to describe tool motion in Cartesian coordinates.

For a welding torch, the TCP may be defined at the specified torch reference point. For a gripper, it may be located at a point representing the intended gripping or handling reference. The exact definition depends on the application.

### 9.1 Why TCP matters

An incorrect TCP can cause:

Positioning errors.

Incorrect approach and retreat paths.

Unexpected tool orientation during motion.

Poor welding or dispensing quality.

Misalignment with fixtures.

Collision risk.

Inconsistent taught positions after tool replacement.

The physical tool can be mounted securely while the programmed TCP remains wrong. Mechanical correctness and coordinate correctness are separate requirements.

### 9.2 TCP definition

The tool frame is generally defined relative to the robot flange using translation and orientation.

Conceptually:

Robot Flange Frame

|

| Defined translation and rotation

|

Tool Frame

|

+--> TCP Reference Point

The translation describes the TCP location relative to the flange. The orientation describes how the tool frame is rotated relative to the flange frame.

### 9.3 TCP calibration methods

Depending on the robot system, methods may include:

Manual measurement using approved references.

Multi-point calibration.

Tool-tip calibration using a fixed reference point.

Calibration fixtures.

External metrology.

Manufacturer-specific automatic calibration tools.

A multi-orientation method can estimate the tool reference point by observing how the robot flange moves while the physical TCP is held at a fixed reference. The exact number of poses and procedure depend on the controller.

### 9.4 TCP verification

Confirm the correct tool frame is selected.

Verify the calibration procedure and input data.

Check that the defined TCP matches the installed tool.

Test movement in the expected coordinate system.

Check orientation and approach direction.

Validate representative process positions.

Record the TCP values and calibration date.

Recalibrate or verify the TCP after tool replacement, mechanical modification, collision, or any event that may change the tool geometry.

## 10. Tool wiring and electrical connections

Powered or instrumented tools may need electrical connections for actuation, sensing, feedback, or communication.

Examples include:

Gripper motor power.

Solenoid valve outputs.

Open and closed position sensors.

Part-present sensors.

Force or torque sensors.

Tool identification circuits.

Vision cameras.

Process equipment signals.

Industrial communication interfaces.

### 10.1 Electrical checks

Verify supply voltage and polarity where applicable.

Confirm output current and inrush requirements.

Check whether outputs are relay, transistor, or other types.

Verify sensor compatibility and signal type.

Confirm connector pin assignments.

Check common reference and grounding requirements.

Maintain required shielding and EMC arrangements.

Protect cables from repeated movement.

Verify that the tool interface is rated for the connected load.

Do not connect a tool load directly to a robot output without verifying its current rating and the required protection. A separate relay, valve driver, or power supply may be necessary.

### 10.2 Tool I/O example

| Signal | Direction relative to controller | Example function |
| --- | --- | --- |
| Gripper Open Command | Output | Commands gripper opening |
| Gripper Close Command | Output | Commands gripper closing |
| Gripper Open Feedback | Input | Confirms open position |
| Gripper Closed Feedback | Input | Confirms closed position |
| Part Present | Input | Indicates component detection |
| Tool Fault | Input | Reports tool-system fault |
| Tool Ready | Input | Indicates tool availability |

The direction labels must be interpreted relative to the controller or I/O module used in the actual design.

## 11. Pneumatic and vacuum connections

Many grippers, tool changers, and vacuum systems require compressed air or vacuum services.

### 11.1 Pneumatic tool connections

Check:

Required pressure and flow.

Hose diameter and pressure rating.

Valve type and actuation method.

Fittings and sealing method.

Leakage.

Hose bend radius.

Hose movement during robot operation.

Exhaust and noise requirements.

Isolation and stored-pressure hazards.

The supply must remain within the tool manufacturer's specified operating range. Excessive pressure can damage components, while insufficient pressure can reduce gripping force or prevent reliable actuation.

### 11.2 Vacuum tool connections

Check:

Vacuum generator or pump capacity.

Vacuum cup type and condition.

Seal quality and workpiece surface.

Filter condition.

Vacuum sensor threshold.

Hose size and leakage.

Response time.

Blow-off or release function.

Behaviour during supply failure.

A vacuum switch indicates a pressure condition, not necessarily successful workpiece retention. The design should account for the consequences of a dropped part.

### 11.3 Gripping-force principle

For an idealised vacuum cup, the theoretical holding force is related to pressure difference and effective area:

F=ΔP×A

Where:

F is the theoretical force.

ΔP is the pressure difference across the cup.

A is the effective sealed area.

Actual holding capacity is affected by leakage, surface texture, cup deformation, acceleration, orientation, and safety factors. Theoretical force alone is not enough to approve a lifting application.

## 12. Tool changer installation

A robot may use a quick tool changer to exchange end-effectors automatically or manually.

A typical tool changer can include:

Robot-side master plate.

Tool-side adapter plate.

Mechanical locking mechanism.

Position or lock-status sensors.

Pneumatic or electrical actuation.

Electrical contacts.

Air, vacuum, or process-media passages.

### 12.1 Installation and verification

Verify compatibility of the robot-side and tool-side units.

Confirm the payload and moment ratings.

Install mounting hardware according to specification.

Verify locking and unlocking actuation.

Confirm the lock-status feedback.

Check utility connection alignment.

Verify the correct tool identification.

Test the release and connection sequence.

Confirm the tool is mechanically retained before movement.

The controller should not assume that a tool is securely attached merely because a command to lock it was issued. Where the design provides locking confirmation, the system should use the required feedback and fault handling.

### 12.2 Tool-change sequence

Robot Approaches Tool Station

|

v

Correct Position and Conditions Verified

|

v

Tool Coupling / Uncoupling Command

|

v

Lock or Unlock Status Checked

|

v

Tool Identity and Utility Connections Checked

|

v

Approved Tool Configuration Activated

|

v

Tool-Change Result Confirmed

The sequence and required interlocks depend on the tool changer and robot system. The example is conceptual, not a universal control program.

## 13. Tool sensors and feedback

Tool sensors help the controller or PLC determine whether the tool and process are in the expected state.

Common examples include:

Gripper open/closed sensors.

Part-present sensors.

Vacuum pressure switches.

Tool changer locked sensors.

Tool identification sensors.

Force/torque sensors.

Temperature sensors.

Process-ready feedback.

Tool fault outputs.

### Diagnostic example: gripper closes but no part is detected

1.	Verify the gripper-close command.

2.	Check whether the gripper physically reaches the expected position.

3.	Monitor the part-present input.

4.	Check the sensor's position and sensing range.

5.	Verify wiring and I/O mapping.

6.	Inspect the component's orientation and gripping surface.

7.	Check whether the tool is configured for the correct part.

8.	Correct the cause and repeat the approved test.

The controller should not infer successful gripping from the close command alone. Command status and physical feedback are different pieces of information.

## 14. Tool installation and safety

Tool installation can introduce new hazards even when the robot itself has not changed.

Examples include:

Sharp or hot tooling.

Heavy tools that can fall.

Pneumatic or hydraulic stored energy.

Welding current and hot workpieces.

Rotating spindles.

Pinch points in grippers.

Dropped loads.

Unexpected tool actuation.

Collision due to increased tool dimensions.

Cable and hose entanglement.

### Safety requirements

Use the approved hazardous-energy isolation procedure.

Support heavy tools with suitable lifting equipment.

Isolate pneumatic, hydraulic, electrical, and other stored energy as required.

Verify the tool is secured before releasing handling equipment.

Confirm the tool's complete collision envelope.

Check safeguarding and risk-assessment requirements.

Verify that safety-related functions remain effective after installation.

Validate tool-changing sequences where applicable.

Confirm the tool and payload configuration before motion.

Do not place hands inside a gripper or hazardous tool mechanism while energy can be applied.

A robot tool may be capable of injuring someone independently of robot movement. Tool hazards must be assessed as part of the complete application.

## 15. Commissioning after tool installation

Tool commissioning confirms that the mechanical assembly, configuration, utilities, signals, and process function operate as intended.

### Commissioning sequence

1.	Verify the tool's mechanical mounting.

2.	Confirm fasteners and specified tightening requirements.

3.	Verify electrical, pneumatic, vacuum, and process connections.

4.	Confirm payload, centre of gravity, and inertia configuration.

5.	Verify the TCP and active tool frame.

6.	Check tool I/O mapping and signal polarity.

7.	Confirm sensor and actuator feedback.

8.	Check clearances and the collision envelope.

9.	Verify safety measures and required interlocks.

10.	Test tool actuation under the approved safe conditions.

11.	Verify the tool through representative robot movements.

12.	Test process functionality using the approved procedure.

13.	Record the final configuration and test results.

The exact order may vary with the tool type. Some tests must be completed before energisation or before robot motion is permitted.

## 16. Common tool installation faults

| Fault | Symptom | Investigation |
| --- | --- | --- |
| Loose mounting fasteners | Tool movement, vibration, or poor repeatability | Inspect mounting and prescribed torque |
| Incorrect adapter | Misalignment or incompatible mounting | Verify flange pattern and adapter drawing |
| Excessive payload | Motion restrictions or controller alarms | Verify complete assembly mass and limits |
| Incorrect centre of gravity | Poor motion performance or excessive loading | Check load data and mounting offset |
| Incorrect TCP | Tool-position error | Verify tool geometry and calibration |
| Incorrect tool frame | Unexpected Cartesian movement | Check active tool and coordinate selection |
| Missing I/O mapping | Tool does not respond to commands | Verify configuration and wiring |
| Incorrect sensor polarity | Feedback state is inverted | Check sensor type, wiring, and input logic |
| Insufficient air pressure | Weak or unreliable gripping | Measure supply pressure and check flow |
| Vacuum leakage | Dropped parts or failed grip detection | Inspect cups, hoses, filters, and fittings |
| Damaged dress pack | Intermittent signals or leakage | Inspect routing, flex points, and strain relief |
| Tool changer not locked | Tool not retained or change fault | Check locking mechanism and confirmation signals |
| Incorrect utility connection | Tool malfunction or leakage | Verify port assignment and connection diagram |
| Collision after tool change | Unexpected contact during motion | Recheck TCP, payload, tool geometry, and path |

## 17. Troubleshooting examples

### Example A: Robot position is correct, but the tool misses the component

Possible causes include an incorrect TCP, incorrect tool orientation, wrong work-object frame, loose mounting, or changed tool geometry.

Investigation:

1.	Inspect the mechanical mounting.

2.	Verify the active tool and work-object frames.

3.	Compare the stored TCP with the actual tool geometry.

4.	Check whether the tool or adapter was recently changed.

5.	Verify representative taught positions.

6.	Recalibrate where required.

7.	Test using the approved commissioning procedure.

### Example B: Gripper output is ON, but the gripper does not close

Possible causes include missing air supply, a failed valve, a wiring fault, a mechanical jam, or an incorrect output assignment.

Investigation:

1.	Confirm the output command and mapped channel.

2.	Verify the correct supply is available.

3.	Check the valve and its electrical or pneumatic actuation.

4.	Inspect the gripper for obstruction or damage.

5.	Check the feedback signals.

6.	Correct the root cause and retest.

### Example C: The robot generates a collision or load alarm after tool replacement

Possible causes include incorrect payload data, changed centre of gravity, altered tool inertia, an incorrect tool configuration, or actual interference.

Investigation:

1.	Stop and assess the condition according to the approved procedure.

2.	Verify the installed tool and adapter.

3.	Compare the new tool's mass and geometry with the previous configuration.

4.	Confirm payload and inertia parameters.

5.	Verify the TCP and collision envelope.

6.	Check mechanical clearance.

7.	Complete the required configuration and safety checks before further motion.

Do not simply disable collision detection to make a new tool operate.

## 18. Tool maintenance and inspection

Tool maintenance depends on the process and the manufacturer's recommendations.

Common inspection items include:

Mounting fasteners and locating features.

Tool adapter condition.

Gripper fingers, pads, and contact surfaces.

Vacuum cups, seals, and filters.

Pneumatic valves and fittings.

Electrical connectors and cables.

Sensor mounting and alignment.

Tool changer locking components.

Cooling or process connections.

Dress-pack wear and clearance.

Tool mass or geometry changes after repair.

After replacing fingers, modifying brackets, adding sensors, or changing tool geometry, reassess the TCP and payload configuration. A seemingly small modification can change the tool frame, centre of gravity, and collision envelope.

## 19. Documentation and handover

Maintain the following records:

Tool identification and part number.

Robot and flange interface details.

Mounting drawing and fastener specifications.

Tool mass, centre of gravity, and inertia data.

TCP and tool-frame configuration.

Payload configuration and revision.

Electrical and pneumatic connection diagrams.

I/O list and signal mapping.

Tool changer configuration where applicable.

Calibration and commissioning results.

Safety validation records where required.

Preventive maintenance requirements.

Tool replacement and repair history.

Approved program and configuration backup references.

Documentation should identify the exact installed configuration rather than merely naming the tool family.

## 20. Practical tool installation checklist

### Mechanical

Correct tool and adapter confirmed.

Flange and mounting interfaces inspected.

Approved fasteners installed and tightened.

Tool orientation verified.

Tool assembly securely supported.

Collision envelope checked.

### Configuration

Tool mass and load data verified.

Centre of gravity and inertia addressed as required.

TCP calibrated or verified.

Correct tool and work-object frames selected.

Payload and tool configuration saved.

### Utilities and signals

Electrical supply and connectors verified.

Tool I/O mapping checked.

Sensors and feedback tested.

Pneumatic or vacuum supply verified.

Process connections checked.

Dress-pack routing inspected.

### Safety and release

Stored-energy hazards controlled.

Tool hazards assessed.

Required safety functions and interlocks verified.

Tool actuation tested under approved conditions.

Representative motion and process tests completed.

Documentation and backups updated.

Required release approval obtained.

## 21. Interview questions and answers

1\. What is an end-effector?

An end-effector is the device attached to a robot that interacts with a workpiece or performs a manufacturing process, such as a gripper, welding gun, or dispensing tool.

2\. What is the TCP?

The Tool Centre Point is the defined reference point and orientation used to describe the tool's position and motion.

3\. Why must payload be configured after tool installation?

The controller needs suitable load data to operate within the robot's permitted loading and motion limits. Incorrect data can degrade performance or create excessive mechanical loading.

4\. Why is the centre of gravity important?

It describes the distribution of mass. A greater offset from the wrist can increase the moment applied to the robot even if the tool mass remains unchanged.

5\. Can a tool remain within the payload mass limit and still be incorrectly loaded?

Yes. Centre-of-gravity, inertia, wrist-moment, speed, and acceleration limits may still be exceeded.

6\. What is the difference between tool mounting and TCP calibration?

Tool mounting establishes the physical mechanical connection. TCP calibration establishes the tool's geometric reference in the robot coordinate system.

7\. Why does a gripper need feedback sensors?

Feedback helps confirm the actual tool state, such as open, closed, or part detected, instead of assuming the action succeeded because a command was issued.

8\. What should be checked after changing gripper fingers?

Inspect the mechanical mounting and assess whether the change affects TCP, tool geometry, payload, centre of gravity, inertia, or gripping performance.

9\. Why is a tool changer's lock confirmation important?

It provides feedback about the coupling state where the design supports it. The system must follow the approved locking and verification sequence before permitting the next operation.

10\. What must be checked before releasing a newly installed tool for production?

Mechanical integrity, payload configuration, TCP, utility connections, I/O and sensor feedback, collision clearance, required safety functions, and successful commissioning tests.

## 22. Core engineering principles

Verify the mechanical flange and tool compatibility before mounting.

Account for the complete tool assembly, including adapters and carried accessories.

Configure payload, centre of gravity, and inertia according to the manufacturer's requirements.

Calibrate and verify the TCP after relevant installation or geometry changes.

Test actual tool feedback rather than relying only on command signals.

Verify electrical, pneumatic, vacuum, and process connections.

Inspect dress-pack movement and the full collision envelope.

Validate tool changer locking and identification where applicable.

Preserve safeguarding and safety-related functions.

Record the final tool configuration and commissioning results.

Core principle: A robot tool is correctly installed only when it is mechanically secure, properly configured, functionally verified, and safe throughout the intended operating envelope.

## NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\04_Installation\04_Robot_Install\06_External_Axes - External axes