# 02_PLAN_WISE_2026\02_INDUSTRY3_0\04_Installation\05_CNC_Install\04_Motors - Motors

## 1. Purpose and Scope of CNC Machine Motors

Electric motors convert electrical energy into mechanical motion. In CNC machines, different motors perform spindle rotation, feed-axis movement, tool changing, hydraulic and coolant pumping, lubrication, and other auxiliary functions.

Motor selection and installation affect machining accuracy, cutting performance, reliability, thermal stability, and machine safety.

The main motor categories found in CNC installations include:

Servo motors: Provide precise position, velocity, and torque control for feed axes.

Spindle motors: Provide the rotational speed and torque required for cutting.

Induction motors: Commonly drive pumps, fans, conveyors, and other auxiliary equipment.

Brake motors: Incorporate a holding brake, often used on vertical axes or auxiliary mechanisms.

Linear motors: Generate linear motion directly without a rotary-to-linear transmission.

Stepper motors: Used in selected positioning systems, although modern industrial CNC feed axes commonly use servo systems.

This topic focuses on motor selection, installation, electrical connections, mechanical integration, commissioning, maintenance, and troubleshooting.

## 2. Motor Applications in a CNC Machine

| Application | Typical motor type | Main requirement |
| --- | --- | --- |
| X, Y, and Z feed axes | AC servo motor | Accurate positioning and dynamic response |
| Rotary table axes | Servo motor | Controlled angular positioning |
| Main spindle | AC spindle motor or suitable induction motor | Cutting speed and torque |
| Tool magazine | Servo, induction, or geared motor | Controlled indexing |
| Coolant pump | Induction motor | Continuous or intermittent fluid delivery |
| Hydraulic power unit | Induction motor | Hydraulic pressure and flow |
| Lubrication pump | Induction or specialized motor | Reliable lubrication delivery |
| Chip conveyor | Geared induction motor | Low-speed torque |
| Automatic door | Servo or geared motor | Controlled opening and closing |
| Linear axis | Linear servo motor | Direct linear force and positioning |

The actual motor type depends on the machine design. A function does not uniquely determine a motor type.

## 3. Basic Motor Energy Conversion

An electric motor produces torque through electromagnetic interaction between its stator and rotor.

ELECTRICAL SUPPLY

↓

MOTOR DRIVE OR STARTER

↓

STATOR MAGNETIC FIELD

↓

ELECTROMAGNETIC INTERACTION

↓

ROTOR TORQUE

↓

MECHANICAL ROTATION

↓

COUPLING / GEARBOX / BALL SCREW

↓

MACHINE MOVEMENT

In a servo-controlled system, feedback and control electronics regulate the motor response. In a conventional induction-motor application, a contactor and overload relay may be used for basic starting and protection, or a VFD may be used for variable-speed control.

## 4. Main Motor Components

### 4.1 Stator

The stator is the stationary portion of the motor. It contains the magnetic circuit and, in most AC motors, the electrical windings that produce the rotating magnetic field.

Common faults include:

Winding insulation deterioration.

Phase-to-phase faults.

Phase-to-earth faults.

Overheating.

Contamination or moisture ingress.

### 4.2 Rotor

The rotor is the rotating part of the motor.

Depending on the motor type, it may contain a squirrel-cage structure, permanent magnets, or another rotor arrangement.

Potential problems include:

Mechanical imbalance.

Bearing damage.

Rotor damage.

Magnet damage in permanent-magnet motors.

Excessive temperature.

Mechanical interference.

### 4.3 Bearings

Bearings support the rotor and maintain the required mechanical alignment.

Bearing condition affects vibration, noise, temperature, efficiency, and service life.

Common causes of bearing failure include:

Incorrect lubrication.

Contamination.

Misalignment.

Excessive radial or axial loading.

Incorrect installation.

Electrical bearing currents in susceptible systems.

### 4.4 Shaft and Coupling

The motor shaft transmits torque to the connected mechanism.

The coupling connects the motor shaft to a load such as a ball screw, gearbox, or spindle transmission.

Misalignment can cause vibration, bearing damage, coupling wear, and positioning problems.

### 4.5 Cooling System

Motor cooling may use:

Natural convection.

Shaft-mounted fans.

Separate forced-air fans.

Liquid cooling.

Cooling arrangements integrated into the spindle assembly.

The cooling method must match the motor design and operating duty.

## 5. Major Motor Types

### 5.1 Three-Phase Induction Motor

An induction motor produces torque through electromagnetic induction in its rotor.

Common applications include coolant pumps, hydraulic pumps, fans, and conveyors.

Advantages include rugged construction, relatively simple maintenance, and widespread availability.

Limitations include speed variation with load, the need for a suitable control system for precise variable-speed operation, and lower suitability than a properly configured servo system for high-precision positioning.

### 5.2 Permanent-Magnet AC Servo Motor

A permanent-magnet servo motor uses permanent magnets in its rotor and is typically controlled by a compatible servo drive.

Applications include CNC feed axes, rotary tables, and precision positioning mechanisms.

Important characteristics include:

High torque density.

Rapid dynamic response.

Precise feedback-based control.

High acceleration capability.

Compatibility requirements for the motor, drive, and feedback system.

### 5.3 AC Spindle Motor

A spindle motor is designed for the operating range and duty required by the machine-tool spindle.

Important considerations include:

Rated and maximum speed.

Continuous and peak torque.

Constant-power operating range.

Cooling.

Encoder compatibility where required.

Bearing and spindle thermal behavior.

Spindle-motor selection cannot be based on rated power alone. Available torque at the required cutting speed is equally important.

### 5.4 Linear Motor

A linear motor generates linear force directly instead of rotating a shaft.

SERVO DRIVE

↓

LINEAR MOTOR FORCE

↓

DIRECT LINEAR AXIS MOVEMENT

↑

LINEAR POSITION FEEDBACK

Advantages can include high acceleration, reduced transmission backlash, and high dynamic performance.

Challenges include heat management, magnetic attraction forces in some designs, installation accuracy, contamination control, and the need for compatible feedback and control.

### 5.5 Stepper Motor

A stepper motor moves in discrete angular increments in response to electrical commands.

It is common in selected low-cost positioning applications and some auxiliary mechanisms.

A stepper system may operate open-loop, although closed-loop stepper systems also exist. An open-loop stepper can lose steps when the applied load exceeds its available torque.

Stepper motors are not automatically interchangeable with servo motors in a CNC axis because their torque-speed characteristics, feedback, dynamic performance, and control interfaces differ.

### 5.6 Geared Motor

A geared motor combines a motor with a gearbox to reduce speed and increase output torque.

It is useful for tool magazines, conveyors, doors, and other auxiliary mechanisms.

The gearbox introduces additional considerations, including backlash, lubrication, efficiency, mounting alignment, and mechanical wear.

## 6. Motor Nameplate and Ratings

The motor nameplate identifies its rated operating characteristics. Always use the actual nameplate and manufacturer documentation when selecting protection, cables, or drive parameters.

| Nameplate parameter | Meaning | Practical use |
| --- | --- | --- |
| Rated power | Rated mechanical output or specified motor rating | Application sizing |
| Rated voltage | Specified operating voltage | Supply and drive compatibility |
| Rated current | Current at specified rated conditions | Protection and cable selection |
| Rated frequency | Rated electrical frequency for applicable AC motors | Supply and operating-range checks |
| Rated speed | Specified rotational speed | Speed and transmission calculations |
| Power factor | Ratio of real to apparent electrical power for AC applications | Electrical-system calculations |
| Efficiency | Ratio of mechanical output to electrical input | Energy and thermal assessment |
| Duty rating | Permitted operating duty | Thermal suitability |
| Insulation class | Thermal classification of the insulation system | Temperature assessment |
| IP rating | Degree of protection against ingress | Environmental suitability |
| Frame or mounting designation | Mechanical mounting and dimensional information | Installation compatibility |
| Serial or model number | Motor identification | Spare-part selection and traceability |

For servo motors, additional information may include motor constant, torque ratings, peak current, feedback type, brake details, and drive compatibility.

## 7. Motor Speed, Torque, and Power

For a rotating motor, mechanical power is related to torque and angular speed.

P=Tω

For rotational speed in RPM:

P=2πnT60

When power is expressed in kilowatts:

PkW=nT9550

Therefore:

T=9550PkWn

Where:

P = mechanical output power.

T = torque in newton-metres.

n = rotational speed in RPM.

Example: a motor producing 7.5 kW at 1,450 RPM has an approximate rated torque of:

T=9550×7.51450≈49.4 N·m

This calculation uses mechanical output power. Actual available torque depends on the motor and drive operating limits.

### 7.1 Starting Torque

Starting torque is the torque available when the motor begins rotating.

A motor driving a high-inertia or heavily loaded mechanism may require more starting torque than a lightly loaded application.

### 7.2 Continuous and Peak Torque

Continuous torque is the torque that the motor can deliver within its specified thermal limits under the stated operating conditions.

Peak torque is available only for a limited period, subject to motor and drive restrictions.

Repeated peak-torque operation can cause overheating if the application exceeds the motor's thermal capability.

### 7.3 Inertia Matching

Servo systems must accelerate the combined motor and load inertia.

The reflected load inertia depends on the transmission arrangement and ratio. A large mismatch between motor inertia and load inertia can make tuning more difficult and reduce dynamic performance.

The permitted inertia ratio and tuning method are manufacturer-specific.

## 8. Motor Selection for CNC Applications

Motor selection should consider the actual mechanical load, required movement, and operating environment.

### 8.1 Feed-Axis Motor Selection

Evaluate:

Required travel and maximum speed.

Acceleration and deceleration.

Continuous and peak torque.

Load inertia.

Ball-screw lead or transmission ratio.

Positioning accuracy and repeatability.

Feedback-device compatibility.

Holding-brake requirements.

Cooling and environmental conditions.

### 8.2 Spindle-Motor Selection

Evaluate:

Required cutting speed.

Torque at low and high speeds.

Continuous and peak power.

Cutting duty cycle.

Spindle and tool inertia.

Transmission ratio.

Cooling requirements.

Feedback and orientation requirements.

Regenerative-energy handling.

### 8.3 Auxiliary Motor Selection

For pumps, fans, and conveyors, consider:

Supply voltage and frequency.

Starting method.

Starting and running current.

Load torque characteristics.

Duty cycle.

Environmental conditions.

IP rating.

Required speed control.

Mechanical mounting and transmission.

### 8.4 Motor Compatibility

A replacement motor must be checked for more than power and frame size.

Verify:

Voltage and current ratings.

Rated and maximum speed.

Torque characteristics.

Shaft dimensions and mounting.

Feedback device.

Brake voltage and control requirements.

Cooling arrangement.

Drive compatibility.

Mechanical load and inertia.

Connector pinout and cable compatibility.

An apparently identical motor may have different feedback, winding, brake, or thermal-sensor arrangements.

## 9. Motor Electrical Connections

### 9.1 Three-Phase Induction Motor

A conventional three-phase motor typically has three phase connections, often identified as U, V, and W or equivalent terminal markings. Motors with multiple winding connections may have six or more terminals.

Star and delta arrangements depend on the motor nameplate, winding configuration, and supply voltage.

Never select a star or delta connection from habit. Verify the nameplate and approved wiring diagram.

### 9.2 Servo Motor

A servo motor commonly has separate connections for:

Motor power.

Encoder or feedback.

Brake, where fitted.

Temperature sensor, where fitted.

These connections must match the drive and motor documentation.

### 9.3 Cable Selection

Motor power cables must be selected according to:

Rated current.

Cable length.

Installation method.

Ambient temperature.

Voltage rating.

Short-circuit protection.

Drive output characteristics.

EMC requirements.

Manufacturer restrictions.

Feedback cables require suitable electrical characteristics, connector compatibility, and shielding according to the drive manufacturer's instructions.

### 9.4 Protective Earth and Shielding

Protective earth provides a fault-current path and helps ensure electrical safety.

Cable shields support electromagnetic compatibility when installed according to the system design.

A cable shield is not a substitute for protective earth. Shield termination and bonding arrangements must follow the manufacturer's installation requirements.

## 10. Mechanical Motor Installation

### 10.1 Alignment

The motor and driven equipment must be aligned within the permitted tolerances.

Misalignment can cause:

Bearing wear.

Coupling failure.

Vibration.

Shaft loading.

Excessive temperature.

Mechanical noise.

Use the approved alignment method and measuring equipment.

### 10.2 Coupling Installation

Check:

Correct coupling type.

Shaft diameter and key arrangement where applicable.

Coupling alignment.

Permitted axial and radial misalignment.

Fastener specifications.

Required balancing.

Manufacturer-specified tightening procedures.

Do not use the coupling to force misaligned shafts into position.

### 10.3 Ball-Screw Connection

For CNC feed axes, check:

Motor-to-ball-screw coupling.

Coupling condition.

Ball-screw alignment.

Bearing support.

Mechanical binding.

Backlash and lost motion.

Lubrication.

Axis travel limits.

Motor rotation alone does not prove that the machine axis moves accurately.

### 10.4 Spindle Motor Installation

Check:

Spindle alignment and mounting.

Belt or coupling condition.

Gearbox condition where fitted.

Spindle-bearing requirements.

Cooling connections.

Lubrication.

Encoder or orientation-sensor installation.

Vibration and thermal behavior.

High-speed spindle systems may have strict balancing, mounting, and warm-up requirements.

## 11. Motor Cooling and Thermal Protection

### 11.1 Sources of Motor Heating

Motor temperature can increase because of:

Excessive current.

Sustained overload.

Poor cooling.

High ambient temperature.

Incorrect drive configuration.

Mechanical friction.

Excessive cycling.

Inadequate ventilation.

Winding or bearing faults.

### 11.2 Thermal Sensors

Depending on the motor, thermal protection may use:

PTC thermistors.

RTD sensors.

Thermostatic switches.

Drive-estimated thermal models.

Manufacturer-specific monitoring arrangements.

The sensor type and interface must be compatible with the connected drive or protection device.

### 11.3 Duty Cycle

Duty cycle describes how the motor operates over time, including loading and rest periods.

A motor that can handle a brief peak load may overheat if that load is repeated too frequently.

Motor sizing should reflect actual machine operation rather than only the maximum instantaneous load.

## 12. Motor Brakes and Vertical Axes

A brake motor or brake-equipped servo motor may use an electromagnetic holding brake.

When the brake is applied, it holds the shaft within its rated capability. When released, the motor can rotate.

A typical control sequence may be:

DRIVE AND CONTROL SYSTEM READY

↓

REQUIRED SAFETY CONDITIONS SATISFIED

↓

MOTOR TORQUE ESTABLISHED AS REQUIRED

↓

BRAKE RELEASED

↓

CONTROLLED MOVEMENT PERMITTED

STOP / HOLD REQUEST

↓

AXIS BROUGHT TO THE REQUIRED STATE

↓

BRAKE APPLIED ACCORDING TO THE APPROVED SEQUENCE

The actual sequence is motor-, drive-, and machine-specific.

Important considerations:

Brake voltage and current.

Brake release and engagement timing.

Permitted holding torque.

Vertical-axis load.

Brake wear and condition.

Feedback and drive status.

Required mechanical support during maintenance.

A motor brake is not automatically a safety-rated holding device. Follow the machine's risk assessment and approved safety design.

## 13. Motor Commissioning

### 13.1 Pre-Commissioning Checks

Confirm motor identity and nameplate ratings.

Verify motor-drive compatibility.

Inspect mounting and mechanical alignment.

Verify power and feedback wiring.

Confirm protective earth.

Check cable and connector identification.

Verify cooling and lubrication.

Confirm that the load can move safely.

Review the approved parameter backup.

Check that required guards and safety systems are functional.

### 13.2 Electrical Checks

Perform electrical tests using the approved procedure and appropriate instruments.

Checks may include:

Supply-voltage verification.

Protective-earth continuity.

Winding resistance where applicable.

Insulation condition using an approved test method.

Cable integrity.

Brake circuit verification.

Thermal-sensor continuity.

Feedback-device diagnostics.

Insulation testing requires particular care with servo motors. Disconnect or isolate the drive and sensitive electronic circuits as specified by the manufacturer. Never apply a test voltage through a connected drive or encoder.

### 13.3 Initial Operation

1.	Confirm all mechanical and electrical installation checks.

2.	Energize the system according to the approved sequence.

3.	Verify the motor and drive status.

4.	Confirm the expected direction at a safe, approved test condition.

5.	Check for abnormal vibration, noise, and temperature.

6.	Verify the required speed or positioning response.

7.	Check brake operation where fitted.

8.	Confirm overload and thermal protection.

9.	Record results and save the approved configuration.

### 13.4 Acceptance Criteria

Acceptance should establish that:

The motor matches the approved specification.

The motor operates without unexplained alarms.

Direction and speed are correct.

Positioning and feedback are valid where applicable.

Cooling and thermal monitoring function correctly.

Mechanical alignment is acceptable.

No abnormal vibration or noise is present.

Protective devices and safety functions are validated.

Required documentation and backups are complete.

## 14. Common Motor Faults and Troubleshooting

| Symptom | Possible causes | Diagnostic approach |
| --- | --- | --- |
| Motor does not start | Supply fault, drive disabled, interlock, wiring issue | Check power, control conditions, and drive status |
| Motor overheats | Overload, cooling failure, high ambient temperature, mechanical friction | Check load, cooling, current, and mechanical condition |
| Excessive vibration | Misalignment, imbalance, bearing damage, loose mounting | Inspect mounting, coupling, bearings, and rotating components |
| Abnormal noise | Bearing wear, mechanical contact, gearbox or coupling fault | Localize the source and inspect under approved conditions |
| Motor trips on overload | Excessive load, incorrect protection settings, frequent starts | Review load current, duty cycle, and protection coordination |
| Servo following error | Feedback issue, excessive load, binding, incorrect tuning | Check feedback, mechanics, and drive diagnostics |
| Motor rotates in the wrong direction | Incorrect configuration or wiring | Stop and verify the approved wiring and direction configuration |
| Brake fails to release | Missing brake supply, wiring fault, brake damage | Check the brake circuit and manufacturer's diagnostics |
| Motor runs but machine axis does not move correctly | Coupling failure, transmission issue, scaling error | Inspect the mechanical connection and feedback scaling |
| Bearing temperature rises | Lubrication issue, misalignment, excess load | Inspect lubrication, alignment, and bearing condition |
| Insulation fault | Moisture, contamination, insulation deterioration, cable damage | Use an approved isolation and testing procedure |
| Repeated thermal alarms | Sustained overload, blocked cooling, wrong motor configuration | Review thermal trends, cooling, and application sizing |

### 14.1 Motor Overheating Investigation

1.	Record the motor temperature, operating time, and load conditions.

2.	Check the motor current and drive alarm history.

3.	Inspect cooling airflow or liquid-cooling conditions.

4.	Check whether the mechanical load moves freely.

5.	Verify motor and drive configuration.

6.	Review duty cycle and repeated peak-load operation.

7.	Inspect for bearing, winding, or cable faults using approved procedures.

8.	Correct the cause and verify temperature behavior during a controlled operating test.

Do not treat a thermal trip as merely a nuisance to be reset. It may be the only indication of an underlying electrical or mechanical fault.

### 14.2 Vibration Investigation

Investigate the following categories separately:

Mechanical: Misalignment, imbalance, loose mounting, damaged coupling, bearing wear.

Electrical: Phase imbalance, winding problems, supply issues.

Control-related: Incorrect tuning, feedback noise, resonance.

Application-related: Excessive load, unsuitable speed, abnormal cutting forces.

Vibration frequency and operating conditions can help distinguish these causes. Where necessary, use a suitable vibration-analysis method rather than relying only on sound.

## 15. Motor Maintenance

### Routine inspection

Check operating temperature.

Listen for abnormal noise.

Observe vibration trends.

Inspect cooling fans and filters.

Check for oil, coolant, or moisture contamination.

Inspect accessible cables and connectors.

Review motor and drive alarms.

Verify brake-related alarms where fitted.

### Periodic maintenance

Inspect coupling and alignment.

Check bearings according to the maintenance plan.

Inspect motor mounting and fasteners during scheduled isolation.

Verify cooling-system condition.

Check electrical connections under approved isolation procedures.

Review thermal and current trends.

Check spare-motor compatibility.

Update repair and replacement records.

Lubrication intervals, bearing replacement methods, and permissible test procedures must follow the motor manufacturer's requirements.

## 16. Motor Replacement and Spare Management

Before replacing a motor, record:

Manufacturer and exact model.

Serial number and rated data.

Frame and mounting dimensions.

Shaft dimensions and coupling type.

Motor-drive compatibility.

Feedback type and connector.

Brake details.

Thermal sensor type.

Cooling requirements.

Cable and connector details.

Mechanical load and transmission ratio.

### Replacement procedure

1.	Identify the failure and confirm the replacement requirement.

2.	Verify the replacement motor's mechanical and electrical compatibility.

3.	Save the current drive configuration.

4.	Isolate energy and secure the mechanism.

5.	Label and disconnect the wiring.

6.	Remove the motor using the approved mechanical procedure.

7.	Inspect the mounting surface, coupling, and connected mechanism.

8.	Install and align the replacement motor.

9.	Connect power, feedback, brake, and sensor wiring correctly.

10.	Verify motor identification and configuration.

11.	Commission movement under controlled conditions.

12.	Validate the relevant safety functions and machine performance.

13.	Update the asset register and spare records.

A spare-motor register should identify the exact machine or axis where the motor is installed, the compatible drive, the feedback arrangement, and any configuration requirements.

## 17. Motor Safety and Isolation

Motors and their connected mechanisms can contain electrical, mechanical, thermal, and gravitational hazards.

Follow the approved lockout/tagout procedure.

Verify electrical isolation before disconnecting motor wiring.

Allow stored electrical energy to discharge according to the drive manufacturer's instructions.

Secure vertical axes and suspended loads against movement.

Prevent unexpected motor starting.

Allow hot motors and braking components to cool before handling.

Follow approved lifting procedures for heavy motors.

Do not rotate or test a motor in a way that could back-feed a connected drive unless the manufacturer explicitly permits the procedure.

Use approved methods for insulation testing and feedback-device testing.

Restore guards and verify safety functions before returning the machine to service.

## 18. Motor Manufacturers and Product Families

| Manufacturer | Common product families | Typical application awareness |
| --- | --- | --- |
| Siemens | SIMOTICS | Industrial, servo, and spindle motor applications |
| FANUC | Servo and spindle motor families | CNC feed axes and spindle systems |
| Mitsubishi Electric | Servo and spindle motor families | CNC and industrial motion systems |
| Yaskawa | SIGMA servo motor families | Industrial positioning and motion control |
| Bosch Rexroth | IndraDyn motor families | Industrial and machine-tool motion |
| ABB | Industrial motor families | Pumps, fans, and general industrial applications |
| WEG | Industrial motor families | General-purpose industrial drives |
| SEW-EURODRIVE | Geared motor families | Conveyors and auxiliary machine mechanisms |
| Beckhoff | AM motor families and linear motor solutions | Servo and direct-drive motion applications |

Exact compatibility depends on the installed model, revision, drive platform, feedback interface, and application. Manufacturer documentation must be used for installation and replacement decisions.

## 19. Practical Diagnostic Example

Scenario: A CNC machine's Z-axis servo motor becomes unusually hot, and the drive occasionally reports an overload.

Initial observations:

The motor runs normally at low load.

Temperature increases during repeated vertical movement.

The machine has a brake-equipped motor.

The overload appears after several production cycles.

A structured investigation should check:

1.	Whether the vertical-axis load is within the motor and drive ratings.

2.	Whether the brake releases completely during movement.

3.	Whether the brake engages correctly when the axis is stopped.

4.	Whether the mechanical transmission has abnormal friction.

5.	Whether motor current rises beyond expected levels.

6.	Whether cooling and thermal monitoring operate correctly.

7.	Whether motor and drive parameters match the installed hardware.

8.	Whether the actual duty cycle exceeds the system's thermal capability.

A dragging brake, mechanical binding, or excessive load can all produce overheating. Replacing the motor without checking these conditions risks damaging the replacement as well.

## 20. Interview Questions and Answers

### Q1. What is the difference between a servo motor and an induction motor?

A servo motor is used in a feedback-based motion-control system for precise positioning, velocity, or torque regulation. An induction motor is widely used for general industrial rotation and can also be speed-controlled with a suitable VFD.

### Q2. Why is motor nameplate information important?

It establishes the motor's rated electrical and mechanical characteristics and supports correct drive configuration, protection, cable selection, and replacement.

### Q3. What causes motor overheating?

Common causes include overload, poor cooling, excessive duty cycle, incorrect drive parameters, mechanical friction, bearing faults, and winding problems.

### Q4. Why is alignment important?

Misalignment increases mechanical loading and can cause vibration, bearing damage, coupling wear, and reduced machine accuracy.

### Q5. What is the difference between continuous and peak torque?

Continuous torque is available within the specified thermal limits. Peak torque is available for a limited duration and is subject to the motor and drive ratings.

### Q6. Why is feedback compatibility important for servo motors?

The drive must correctly interpret the motor's feedback signals to regulate position and speed. An incompatible encoder or incorrect configuration can prevent operation or cause motion faults.

### Q7. What is the purpose of a motor brake?

A motor brake can hold a shaft when the motor is not producing the required holding torque. Its suitability for safety-related holding depends on the specific design and application.

### Q8. What causes motor vibration?

Potential causes include mechanical misalignment, imbalance, bearing damage, loose mounting, electrical faults, control-loop tuning, and resonance.

### Q9. Can a motor be replaced with another motor having the same power rating?

Not without checking voltage, current, speed, torque, mounting, shaft dimensions, feedback, brake, cooling, and drive compatibility.

### Q10. Why should insulation testing be performed carefully on servo motors?

Connected drives and feedback electronics can be damaged by inappropriate test voltages. The manufacturer-approved isolation and testing procedure must be followed.

### Q11. What is the difference between a rotary motor and a linear motor?

A rotary motor produces rotational movement. A linear motor generates linear force directly and can eliminate the need for a ball screw or similar transmission.

### Q12. What should be done before returning a replaced motor to production?

Verify electrical and mechanical installation, feedback and drive configuration, direction, cooling, brake operation where fitted, controlled motion, relevant safety functions, and documentation.

## 21. Practical Knowledge Checklist

Identify the motor types used in CNC machines.

Read and interpret motor nameplates.

Calculate motor torque from rated power and speed.

Understand continuous and peak torque.

Verify motor-drive and feedback compatibility.

Understand motor power, feedback, brake, and sensor connections.

Check mechanical alignment, couplings, and mounting.

Understand cooling and thermal protection.

Explain motor brake operation on vertical axes.

Troubleshoot overheating, vibration, overload, and feedback faults.

Follow approved isolation and insulation-testing procedures.

Maintain motor configuration, replacement, and spare records.

## NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\04_Installation\05_CNC_Install\05_Encoders - Encoders