# 02_PLAN_WISE_2026\02_INDUSTRY3_0\04_Installation\05_CNC_Install\06_IO - I/O

## 1. Purpose and Scope of CNC I/O

I/O stands for Input/Output. It is the interface through which a CNC machine controller, machine PLC, drives, sensors, actuators, and auxiliary equipment exchange electrical signals.

Inputs communicate the state of the machine to the control system. Outputs allow the control system to command devices such as solenoid valves, contactors, indicator lamps, relays, and other actuators.

Correct I/O installation is essential for machine sequencing, interlocking, fault detection, process reliability, and safe operation.

This topic covers digital and analog I/O, CNC interface signals, electrical wiring, I/O modules, signal assignment, commissioning, troubleshooting, and documentation.

## 2. Role of I/O in a CNC Machine

Typical CNC machine inputs include:

Limit switches and home-reference sensors.

Door interlock feedback.

Hydraulic and pneumatic pressure switches.

Coolant level and flow switches.

Tool-clamp and tool-unclamp confirmation.

Spindle status and drive-ready signals.

Overload and fault contacts.

Push buttons and selector switches.

Typical outputs include:

Solenoid valves.

Coolant pumps and lubrication commands.

Hydraulic unit commands.

Tool-changer commands.

Clamping and unclamping commands.

Stack lights and audible alarms.

Contactor or relay control signals.

Drive enable or reset signals, where supported.

Some machine functions use direct communication over a fieldbus rather than individual hardwired I/O. Safety-related functions may use dedicated safety I/O or a safety network.

## 3. CNC I/O System Architecture

OPERATOR DEVICES

Push Buttons / Selectors / Indicators

↓

SENSORS AND ACTUATORS

Limit Switches / Valves / Relays

↓

ELECTRICAL I/O MODULES

Digital Inputs / Digital Outputs

Analog Inputs / Analog Outputs

↓

MACHINE PLC OR CNC I/O INTERFACE

↓

CNC CONTROL AND MACHINE SEQUENCE

↓

DRIVES / SPINDLE / TOOL CHANGER / AUXILIARIES

The actual architecture varies by machine builder. The CNC may contain integrated I/O, use remote I/O modules, communicate with a separate machine PLC, or combine these arrangements.

## 4. Digital Inputs

A digital input represents a discrete state, usually interpreted as ON/OFF or TRUE/FALSE.

Common industrial digital input voltage levels include 24 V DC. Other voltages may be used in specific systems, so the module specification must always be checked.

### 4.1 Common Digital Input Devices

| Device | Typical purpose | Example input state |
| --- | --- | --- |
| Proximity sensor | Detect a target | Target detected |
| Limit switch | Detect a mechanical position | Switch actuated |
| Pressure switch | Confirm pressure threshold | Pressure acceptable |
| Float switch | Detect liquid level | Level reached |
| Push button | Operator command | Button pressed |
| Relay auxiliary contact | Confirm a contactor state | Contactor feedback |
| Thermal switch | Detect excessive temperature | Thermal condition normal |
| Door switch | Detect door position | Door closed indication |

The electrical state of an input is not always identical to the physical state of the device. Normally open or normally closed contacts, PNP/NPN sensor types, wiring faults, and program logic all affect the interpretation.

### 4.2 Normally Open and Normally Closed Contacts

Normally open (NO): The contact is open in its defined normal, unactuated state and closes when actuated.

Normally closed (NC): The contact is closed in its defined normal, unactuated state and opens when actuated.

The term normal refers to the device's specified unactuated condition, not necessarily the machine's normal operating condition.

NC wiring can help detect some broken-wire faults when the control logic is designed appropriately. However, a conventional NC contact is not automatically a safety-rated arrangement.

## 5. Digital Outputs

A digital output commands a discrete state, such as switching a valve coil or activating an indicator.

Common output types include transistor outputs and relay outputs.

### 5.1 Transistor Outputs

Transistor outputs are electronic switching outputs, commonly used with DC loads.

Advantages include fast switching and suitability for repeated switching.

Important limitations include:

Maximum output current.

Permitted load type.

Short-circuit protection.

Leakage current.

Voltage drop.

Inductive-load suppression requirements.

Thermal and channel-group restrictions.

### 5.2 Relay Outputs

Relay outputs use mechanical contacts to switch a circuit.

Advantages include the ability to switch certain AC or DC loads within the relay's specified ratings and electrical isolation between the control circuit and switched circuit.

Limitations include contact wear, switching-speed limitations, arcing, and restrictions on inductive loads.

### 5.3 Typical Output Loads

| Output load | Typical control purpose | Key consideration |
| --- | --- | --- |
| Solenoid valve | Pneumatic or hydraulic actuation | Coil current and surge suppression |
| Interposing relay | Interface to another circuit | Coil voltage and contact rating |
| Contactor coil | Switch a motor or power circuit | Coil suppression and interface rating |
| Indicator lamp | Machine status indication | Voltage and output current |
| Buzzer | Audible warning | Load type and current |
| Drive command input | Enable or request a function | Correct control interface |
| Lubrication pump command | Request lubrication | Feedback and interlock logic |

A PLC output must not directly switch a load unless the output type and ratings explicitly permit it. Interposing relays or suitable interface modules may be required.

## 6. Sourcing and Sinking I/O

Sourcing and sinking describe the direction of conventional DC current flow relative to the switching device or input circuit.

### 6.1 PNP Sensor

A PNP sensor typically sources positive voltage to its output when active.

+24 V DC

↓

PNP SENSOR OUTPUT

↓

PLC DIGITAL INPUT

↓

0 V DC RETURN

### 6.2 NPN Sensor

An NPN sensor typically sinks current toward 0 V when active.

+24 V DC

↓

PLC DIGITAL INPUT CIRCUIT

↓

NPN SENSOR OUTPUT

↓

0 V DC

These diagrams are conceptual. The actual circuit depends on the input module and sensor design.

### 6.3 Compatibility

| Sensor type | Typical active output behavior | Required check |
| --- | --- | --- |
| PNP | Sources positive voltage | Input must accept the corresponding sourcing signal |
| NPN | Sinks current toward 0 V | Input must accept the corresponding sinking signal |
| Two-wire sensor | Changes current through a series circuit | Input compatibility and off-state leakage |
| Push-pull sensor | Can source or sink according to its design | Electrical specifications and wiring |

Do not assume every PLC input accepts both PNP and NPN sensors. Verify the module's input circuit, common-terminal arrangement, and voltage specifications.

## 7. Analog Inputs and Outputs

Analog I/O communicates a value over a range rather than representing only two states.

Common industrial signals include:

4–20 mA.

0–20 mA.

0–10 V.

Other voltage or current ranges supported by the module.

Applications include pressure, temperature, flow, level, force, and speed references.

### 7.1 Analog Inputs

An analog input receives a signal from a transmitter or another analog source.

Example:

PRESSURE TRANSMITTER

↓

4–20 mA SIGNAL

↓

ANALOG INPUT MODULE

↓

PLC SCALING

↓

ENGINEERING VALUE

↓

ALARM / DISPLAY / CONTROL

### 7.2 Analog Outputs

An analog output generates a variable signal for a compatible device, such as a drive speed reference or control valve.

The output range and receiving-device configuration must match.

### 7.3 Why 4–20 mA Is Widely Used

A 4–20 mA signal uses 4 mA as the nominal lower measurement value and 20 mA as the nominal upper value.

The live zero allows some open-circuit or under-range faults to be distinguished from a valid zero process value, provided the transmitter and receiving system support the required diagnostics.

A reading near 0 mA can indicate an open loop, loss of supply, or another fault. It should not automatically be interpreted as a genuine zero measurement.

### 7.4 Analog Scaling

For a linear signal, the engineering value can be calculated as:

PV=LRV+I−IminImax−Imin(URV−LRV)

Where:

PV is the calculated process value.

I is the measured current.

Imin and Imax are the signal-range limits.

LRV is the lower range value.

URV is the upper range value.

For a 4–20 mA transmitter configured for 0–100 bar, a measured current of 12 mA corresponds to:

PV=0+12−420−4(100−0)=50 bar

This result assumes correct transmitter configuration, linear scaling, and no relevant measurement error.

## 8. I/O Module Types

| Module type | Function | Important parameters |
| --- | --- | --- |
| Digital input module | Reads discrete states | Voltage, input current, filter time, channel count |
| Digital output module | Switches discrete loads | Output type, current rating, protection |
| Analog input module | Measures variable signals | Signal range, resolution, accuracy, isolation |
| Analog output module | Generates variable signals | Output range, resolution, load capability |
| High-speed input module | Processes fast pulses or events | Frequency, timing, supported signal type |
| Temperature input module | Measures supported temperature sensors | Sensor type, range, wiring arrangement |
| Remote I/O adapter | Connects remote modules to a controller network | Protocol, addressing, update time |
| Safety I/O module | Processes supported safety signals | Safety certification, architecture, diagnostics |

High-speed signals may require dedicated modules rather than standard digital inputs. Safety-related signals require appropriate safety-rated equipment and validated design.

## 9. CNC Machine I/O Categories

### 9.1 Operator I/O

Examples include cycle start, feed hold, reset, mode selectors, handwheel controls, and status indicators.

The control logic must distinguish between commands, maintained states, and momentary actions.

### 9.2 Machine Sequence I/O

These signals coordinate machine operations such as:

Door opening and closing.

Tool change.

Pallet change.

Workpiece clamping.

Coolant operation.

Lubrication.

Hydraulic and pneumatic functions.

Each sequence should include suitable permissives, timeouts, completion feedback, and fault handling.

### 9.3 CNC-to-PLC Interface Signals

A CNC and machine PLC may exchange commands and status information.

Examples include:

Cycle start request.

Machine-ready status.

Feed-hold status.

Spindle start request.

Tool-change request.

Axis or spindle fault status.

Program-running status.

Machine alarm status.

Signal names, addresses, polarity, and timing depend on the CNC platform and machine-builder implementation.

### 9.4 Safety-Related Signals

Examples include emergency-stop devices, guard interlocks, safety-rated stop requests, and safety-system status feedback.

These signals must be implemented using the required safety architecture. A normal PLC input and ordinary ladder logic do not become safety-rated merely because they monitor an emergency-stop contact.

## 10. I/O Addressing and Signal Mapping

Each I/O point needs a unique and traceable identity.

Depending on the platform, the point may be identified by a rack and slot address, a channel number, a symbolic tag, a fieldbus node, or a CNC-specific signal identifier.

Example signal register:

| Tag name | Device | I/O type | Address | Normal condition |
| --- | --- | --- | --- | --- |
| X_AXIS_HOME | Home sensor | Digital input | Platform-specific | Home reference detected |
| DOOR_CLOSED_FB | Door switch | Digital input | Platform-specific | Door confirmed closed |
| HYD_PRESS_OK | Pressure switch | Digital input | Platform-specific | Pressure acceptable |
| COOLANT_CMD | Coolant contactor interface | Digital output | Platform-specific | Command depends on sequence |
| TOOL_CLAMP_FB | Clamp confirmation sensor | Digital input | Platform-specific | Tool clamped |
| SPINDLE_SPEED_REF | Spindle drive interface | Analog output or network data | Platform-specific | Commanded speed value |

The example addresses are intentionally symbolic. Actual addresses must come from the machine's electrical drawings, PLC configuration, and CNC documentation.

### 10.1 Signal Naming

A useful naming convention should communicate:

Machine or subsystem.

Device or function.

Signal direction or role.

Physical state or command.

Examples include DOOR_CLOSED_FB, COOLANT_CMD, and HYD_PRESS_OK.

Use the plant or machine-builder naming convention consistently rather than creating a separate naming system without coordination.

## 11. I/O Installation and Wiring

### 11.1 Pre-Installation Checks

Verify module type and catalog number.

Confirm slot, rack, node, and channel assignment.

Check voltage and current requirements.

Confirm sensor and actuator compatibility.

Review the latest electrical drawings.

Identify the correct terminal numbers.

Confirm common-terminal and grounding arrangements.

Verify fieldbus configuration for remote I/O.

Check the required environmental rating.

### 11.2 Field Wiring

During installation:

1.	Isolate the relevant energy sources according to the approved procedure.

2.	Identify each conductor and terminal from the wiring documentation.

3.	Verify wire size and terminal suitability.

4.	Separate signal wiring from power wiring as required.

5.	Maintain shielding and grounding arrangements for analog and communication circuits.

6.	Provide suitable strain relief and mechanical protection.

7.	Verify polarity and terminal numbering.

8.	Inspect for loose strands, damaged insulation, and incorrect ferrules.

9.	Update wiring and terminal records when approved changes are made.

### 11.3 Inductive Loads

Solenoid coils, relay coils, and contactor coils can generate voltage transients when switched.

Suppression may be required using an approved diode, TVS device, RC network, or another suitable solution.

The suppression method must match the coil voltage, AC/DC type, switching device, and required release time. A suppression component that slows valve or contactor release excessively can create functional problems.

## 12. I/O Commissioning

### 12.1 Digital Input Testing

For each digital input:

Verify the field device's physical state.

Check its supply and wiring.

Observe the input LED, if provided.

Monitor the corresponding PLC or CNC diagnostic bit.

Confirm that the displayed state matches the expected electrical condition.

Test both active and inactive states where safe and applicable.

Confirm that the logic interprets the signal correctly.

An input LED indicates an electrical input state, not necessarily correct program logic or machine behavior.

### 12.2 Digital Output Testing

Before activating an output, verify that the connected mechanism can operate safely.

Then:

1.	Confirm the correct output address.

2.	Verify the connected load and supply.

3.	Check interlocks and permissives.

4.	Command the output using the approved commissioning method.

5.	Observe the output indicator and electrical response.

6.	Confirm the actuator operates.

7.	Verify the corresponding feedback signal.

8.	Confirm the output turns off as expected.

Never force an output if doing so could move an axis, release a clamp, open a valve, or create another hazard.

### 12.3 Analog I/O Testing

For analog input channels:

Confirm signal type and range.

Verify transmitter supply and loop wiring.

Compare the measured signal with the displayed value.

Confirm scaling and engineering units.

Check under-range and over-range behavior.

Validate alarms and fault handling.

For analog outputs:

Confirm output range and load compatibility.

Check the output value using a suitable measuring instrument.

Verify the receiving device interprets the signal correctly.

Validate scaling, direction, and limits.

### 12.4 Point-to-Point Verification

A point-to-point test confirms that each physical device is connected to the intended control-system channel and mapped to the correct logical signal.

It should verify the field device, terminal, module channel, controller tag, program use, and expected machine response.

## 13. CNC Machine Sequence and Interlocking

I/O signals often determine whether a machine sequence can proceed.

Example: a tool-clamping sequence may require a clamp command followed by confirmation that the tool is correctly clamped.

TOOL CLAMP REQUEST

↓

CHECK REQUIRED PERMISSIVES

↓

ACTIVATE CLAMP OUTPUT

↓

WAIT FOR CLAMP CONFIRMATION

↓

CONFIRMATION RECEIVED?

↙           ↘

YES            NO

↓              ↓

CONTINUE       TIMEOUT / FAULT

SEQUENCE       HANDLING

This is a conceptual sequence. The actual implementation must account for the machine design, actuator behavior, sensor arrangement, and safety requirements.

### 13.1 Permissives

A permissive is a condition that must be satisfied before an action is allowed.

Examples include:

Hydraulic pressure acceptable.

Door confirmed closed.

Required axis in position.

Spindle stopped.

Tool magazine in a valid state.

No relevant machine fault present.

### 13.2 Feedback Confirmation

A command does not prove that an actuator completed its action.

For example, a tool-clamp output may be active while the clamp mechanism remains stuck. A suitable feedback sensor and timeout can help detect the failure.

### 13.3 Timeouts

A timeout identifies that a required response did not occur within the permitted period.

Timeout values must be based on normal equipment response, operating conditions, and machine-builder requirements. Excessively short values cause nuisance faults; excessively long values delay fault detection.

## 14. I/O Troubleshooting

| Symptom | Possible causes | Diagnostic approach |
| --- | --- | --- |
| Input LED remains OFF | Missing supply, open wire, sensor fault, incorrect polarity | Check supply, field wiring, and sensor output |
| Input LED is ON but PLC bit is OFF | Wrong channel, configuration mismatch, module fault, mapping issue | Check hardware configuration and diagnostics |
| Input remains ON continuously | Short circuit, incorrect sensor adjustment, wrong logic polarity | Verify electrical state and sensor operation |
| Output command is ON but actuator does not operate | Missing load supply, open coil, blown fuse, failed output | Check load circuit and module diagnostics |
| Output operates unexpectedly | Logic error, forced output, wrong mapping, wiring error | Review program state, forces, and drawings |
| Analog value is zero | Open loop, missing supply, wrong range, wiring fault | Measure loop and verify configuration |
| Analog value is unstable | Electrical noise, poor shield termination, loose connection, unstable process | Check wiring, grounding, signal quality, and process |
| Analog value is incorrect but stable | Scaling error, wrong range, incorrect transmitter configuration | Verify raw value, range, and engineering scaling |
| Remote I/O unavailable | Network fault, wrong address, power loss, configuration mismatch | Check node status, power, cabling, and network diagnostics |
| Intermittent I/O faults | Loose terminals, damaged cables, EMI, unstable supply | Inspect wiring and review event history |
| Tool-change sequence times out | Missing confirmation, mechanical jam, failed actuator, wrong sensor position | Check sequence state, output, mechanism, and feedback |
| Input flickers during switching | Noise, inadequate filtering, poor contact, unstable sensor | Check signal quality and configured input filter |

## 15. Diagnostic Method for a Failed Digital Input

Scenario: A proximity sensor detects a machine component, but the CNC diagnostic screen does not show the expected input state.

Investigate systematically:

1.	Confirm that the correct sensor and target are being observed.

2.	Verify the sensor's supply voltage.

3.	Check the sensor's output indicator, if available.

4.	Verify whether the sensor output is PNP, NPN, two-wire, or another type.

5.	Check compatibility with the PLC input circuit.

6.	Verify terminal numbers, common connections, and polarity.

7.	Check the relevant module channel indicator.

8.	Monitor the controller's raw input bit.

9.	Confirm the correct address and tag mapping.

10.	Verify the machine logic uses the intended input.

11.	Record the fault and corrective action.

Do not replace the sensor until the supply, wiring, input compatibility, and channel mapping have been checked.

## 16. Analog Signal Fault Diagnosis

Scenario: A hydraulic pressure transmitter is configured for 4–20 mA and a 0–100 bar range, but the HMI shows an implausible pressure value.

Possible causes include:

Incorrect loop wiring.

Loss of transmitter supply.

Open or shorted circuit.

Wrong input channel configuration.

Incorrect raw-value scaling.

Incorrect engineering range.

Wrong transmitter configuration.

Electrical interference.

Faulty transmitter or analog input channel.

A useful diagnostic order is:

1.	Confirm the transmitter's model and configured range.

2.	Check the channel configuration.

3.	Verify loop supply and wiring.

4.	Measure the current using an approved method.

5.	Compare the measured current with the controller's raw input value.

6.	Check the scaling calculation and engineering units.

7.	Verify the HMI tag references the correct PLC variable.

8.	Check fault thresholds and under-range handling.

Avoid opening a live current loop without understanding the effect on the process and the measurement circuit.

## 17. I/O Safety Considerations

Follow approved lockout/tagout and electrical isolation procedures.

Treat unexpected actuator movement as a credible hazard during commissioning.

Verify the function and electrical ratings of protective devices.

Do not bypass guards, interlocks, or safety devices to make a sequence run.

Do not assume standard PLC I/O provides a safety-rated function.

Use appropriate safety PLCs, safety relays, and safety I/O where required by the machine risk assessment.

Validate safety functions using the approved test procedure.

Use controlled methods when testing outputs connected to valves, clamps, drives, and motors.

Account for stored pneumatic, hydraulic, gravitational, and electrical energy.

Restore guards, wiring covers, and interlocks before returning the machine to production.

## 18. I/O Documentation and Maintenance

Accurate I/O documentation makes breakdown diagnosis and future modifications faster.

Maintain the following records:

I/O list and signal descriptions.

Electrical schematics.

Terminal and cable schedules.

Module catalog numbers and channel assignments.

PLC or CNC tag mappings.

Sensor and actuator model numbers.

Signal voltage, current, and range.

Network addresses for remote I/O.

Normal and fault states.

Interlock and sequence descriptions.

Commissioning and point-to-point test results.

Approved program and configuration backups.

Modification history and corrective actions.

For each I/O point, document not only its address but also its function, physical device, terminal, expected state, and relevant fault behavior.

## 19. I/O Spare-Part and Obsolescence Management

When maintaining CNC machines, a spare I/O module must match the installed control platform and application.

Check:

Exact catalog number and hardware revision.

Digital or analog function.

Voltage and current ratings.

Channel count and channel grouping.

Input or output circuit type.

Isolation and protection features.

Terminal base and connector compatibility.

Firmware and controller compatibility.

Remote I/O adapter compatibility.

Configuration and replacement requirements.

A module that fits physically may still be electrically or functionally incompatible.

For discontinued modules, document the approved successor, required hardware changes, wiring changes, configuration changes, and validation requirements before replacement.

## 20. Interview Questions and Answers

### Q1. What is I/O in industrial automation?

I/O is the interface through which a controller receives signals from field devices and sends commands to actuators or other equipment.

### Q2. What is the difference between digital and analog I/O?

Digital I/O represents discrete states, commonly ON and OFF. Analog I/O represents a variable quantity using a signal range such as 4–20 mA or 0–10 V.

### Q3. What is the difference between PNP and NPN sensors?

A PNP sensor typically sources positive voltage to its output when active. An NPN sensor typically sinks current toward 0 V when active.

### Q4. Why is 4–20 mA commonly used?

It provides a standardized current range, supports transmission over suitable industrial wiring, and allows some loop faults to be distinguished from valid low process values.

### Q5. What is the difference between a relay output and a transistor output?

A relay output switches mechanical contacts and can support specified AC or DC loads. A transistor output switches electronically and is commonly used for DC loads and faster switching.

### Q6. Why is feedback required after an output command?

The output confirms only that a command was issued. Feedback confirms whether the intended machine action or physical state was achieved.

### Q7. What is a permissive?

A permissive is a required condition that must be satisfied before a machine action or sequence can proceed.

### Q8. Why is a timeout used in machine sequencing?

A timeout detects when a required response fails to occur within the permitted period, allowing fault handling instead of indefinite waiting.

### Q9. What should be checked if an input LED is ON but the PLC bit is OFF?

Check module configuration, channel mapping, controller diagnostics, tag addressing, and any relevant hardware or communication faults.

### Q10. Why can an analog value be wrong even when the current signal is correct?

The input range, raw-value conversion, engineering scaling, HMI tag mapping, or engineering units may be incorrect.

### Q11. Can an ordinary PLC input be used as an emergency-stop safety input?

Not as a safety function unless the complete design uses appropriate safety-rated hardware, architecture, and validation. A conventional input alone does not provide the required safety integrity.

### Q12. What is point-to-point I/O testing?

It verifies that each field device is connected to the correct terminal and I/O channel, mapped to the correct controller signal, and interpreted correctly by the machine logic.

### Q13. What causes intermittent input signals?

Potential causes include loose terminals, damaged cables, unstable sensor supply, electrical interference, poor connections, and unsuitable input filtering.

### Q14. Why should I/O documentation include normal and fault states?

It helps maintenance personnel determine whether a signal is behaving correctly and understand how the machine should respond when a device or circuit fails.

## 21. Practical Knowledge Checklist

Distinguish digital and analog I/O.

Identify common CNC machine input and output devices.

Understand NO and NC contacts.

Explain PNP, NPN, sourcing, and sinking.

Distinguish transistor and relay outputs.

Understand 4–20 mA and 0–10 V signals.

Calculate analog engineering values from signal ranges.

Read I/O lists, electrical drawings, and terminal schedules.

Perform approved point-to-point I/O verification.

Understand CNC-to-machine-PLC interface signals.

Diagnose input, output, analog, and remote-I/O faults.

Understand permissives, feedback confirmation, and timeouts.

Distinguish standard I/O from safety-rated I/O.

Maintain I/O mapping, configuration, and spare-part records.

## NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\04_Installation\05_CNC_Install\07_Machine_PLC - Machine PLC