# 02_PLAN_WISE_2026\02_INDUSTRY3_0\10_Detail_Eng\12_Control_Philosophy - Control philosophy

## 1. Definition and purpose

A Control Philosophy is an engineering document that defines the overall principles, rules, operating modes, control strategies, equipment behavior, operator responsibilities, and abnormal-condition responses governing an automated system.

It explains how the control system is intended to behave across the complete machine or process, rather than documenting only individual PLC instructions or sequence steps.

For example, in an automotive manufacturing line, the control philosophy establishes how machines operate in Auto and Manual modes, how the line responds when a downstream station becomes unavailable, how local and remote control authority is managed, how faults affect production, and how the system recovers after a power interruption.

It provides a common reference for electrical engineers, automation engineers, PLC programmers, HMI developers, machine builders, commissioning teams, maintenance personnel, and operations.

The control philosophy should be agreed upon before detailed software development. Otherwise, different engineers may independently implement reasonable-looking solutions that behave inconsistently when integrated. Humans have managed to make even a simple Start button a multidisciplinary negotiation.

## 2. Objectives

Establish consistent control principles across machines and subsystems.

Define operating modes, control authority, and operator interactions.

Specify automatic, manual, setup, maintenance, and recovery behavior.

Define equipment control strategies, including interlocks and permissives.

Establish fault handling, stopping behavior, reset rules, and restart prevention.

Define how interconnected machines coordinate through handshakes and production-line signals.

Establish alarm, diagnostic, and HMI display principles.

Define startup, shutdown, power-loss, and controller-restart behavior.

Establish conventions for PLC software, tags, reusable logic, and diagnostics.

Support integration, FAT, SAT, commissioning, and maintenance.

Provide a consistent basis for software specifications and design reviews.

## 3. Scope of a control philosophy

The scope depends on the project. A small standalone machine may need a short document, while an integrated production line may need a comprehensive philosophy covering multiple PLCs, robots, CNCs, safety controllers, utilities, and supervisory systems.

Typical subjects include:

Control-system architecture.

Operating modes and command authority.

Equipment control methods.

Sequence execution principles.

Start permissives and interlocks.

Alarm and fault management.

Stop, abort, reset, and recovery behavior.

Machine-to-machine communication.

HMI operation and diagnostics.

Safety-system interfaces.

Startup and shutdown.

Data handling and production reporting.

Software standards and naming conventions.

Testing, commissioning, and change management.

A control philosophy defines the governing principles. Detailed specifications and engineering documents provide the implementation-level information.

## 4. Control philosophy versus related documents

| Document | Main question answered | Level of detail |
| --- | --- | --- |
| Control Philosophy | What overall rules govern system behavior? | System-wide principles |
| Functional Design Specification | What functions must the system provide? | Functional requirements |
| Sequence Description | How does an operation progress step by step? | Sequence behavior |
| Interlock List | Which conditions inhibit or terminate operations? | Individual interlocks |
| Cause-and-Effect Matrix | What actions follow specified causes? | Cause-to-effect relationships |
| Alarm List | Which abnormal conditions must be reported? | Alarm behavior and configuration |
| Software Specification | How should the software be structured and implemented? | Software requirements and design |
| I/O List | Which signals are required? | Signal-level definition |
| Electrical Schematics | How are electrical circuits connected? | Circuit-level design |
| HMI Specification | What screens, controls, and indications are required? | Operator interface |
| Safety Requirements Specification | What safety functions and performance are required? | Safety-related requirements |

The documents should be mutually consistent. For example, if the control philosophy states that a fault reset must never initiate a production cycle, the sequence description, software specification, PLC implementation, and HMI behavior must follow that rule.

## 5. Control-system architecture

A typical industrial control architecture may be organized as follows.

Enterprise and Production Systems

↓

MES / Production Tracking / Reporting

↓

SCADA / Supervisory Monitoring

↓

Line PLC / Cell PLC / Machine PLC

↓

Remote I/O / Drives / Robots / CNCs

↓

Sensors / Actuators / Motors / Valves

Safety Sensors

↓

Safety Relay or Safety Controller

↓

Safety Outputs and Safety-Related Devices

This is a conceptual arrangement, not a mandatory architecture. Actual systems may combine, separate, or add layers according to project requirements.

The control philosophy should identify:

Which controller owns each machine or subsystem.

Which functions are performed locally and centrally.

Which functions are supervisory rather than time-critical.

How PLCs communicate.

How control authority is transferred.

How safety-related functions are separated or integrated.

What happens when a controller, network, or supervisory system becomes unavailable.

## 6. Control-system operating principles

### 6.1 Local and remote control

The philosophy must define where an operator or control system may issue commands.

Typical arrangements include:

Local control through a machine-mounted station.

Operator control through an HMI.

Line-level control through a supervisory PLC.

Supervisory monitoring through SCADA.

Remote engineering access under an approved authorization process.

A control-authority hierarchy should be explicit. Two control sources must not independently command the same actuator without a defined arbitration mechanism.

For example, if a motor is placed in Local mode, the line PLC may be prevented from issuing normal remote start commands. However, the required protective functions must remain effective.

### 6.2 Command ownership

Every controlled device should have a defined owner for its normal operating commands.

Examples:

The machine PLC controls the machine's normal sequence.

The robot controller controls robot motion.

The CNC controls machining motion.

The line PLC coordinates production requests and station readiness.

The safety controller executes designated safety functions.

The line PLC may request a robot operation without directly controlling every robot motion. This separation improves modularity and clarifies troubleshooting responsibility.

### 6.3 Authority and arbitration

Where several command sources exist, define:

Priority of command sources.

Conditions for transferring authority.

Required acknowledgment of the transfer.

Behavior when authority becomes invalid.

Restrictions during active movement.

Operator indications of the active authority.

Recovery behavior after a communication failure.

A software selector alone does not guarantee safe transfer of control. The design must consider the physical process state and any safety-related requirements.

## 7. Operating modes

The control philosophy must define each operating mode and the behavior permitted within it.

| Mode | Purpose | Typical control behavior |
| --- | --- | --- |
| Auto | Normal automatic production | PLC executes the approved sequence |
| Manual | Individual equipment operation | Authorized operator commands selected functions |
| Setup | Preparation and adjustment | Restricted setup operations under specified safeguards |
| Maintenance | Service and diagnosis | Authorized maintenance functions and diagnostic access |
| Recovery | Return from abnormal or interrupted operation | Controlled restoration of a valid equipment state |
| Local | Local command authority | Device or machine station controls permitted functions |
| Remote | External command authority | Designated PLC or supervisory source controls permitted functions |

These categories are not necessarily mutually exclusive. For example, Auto/Manual may describe a machine operating mode, while Local/Remote describes command authority.

### 7.1 Auto mode

Typical principles:

Start is accepted only when required conditions are satisfied.

Sequence execution follows the approved sequence description.

Equipment actions occur in the specified order.

Interlocks and required safety functions remain effective.

Abnormal conditions produce the specified hold, stop, abort, or trip behavior.

A new cycle cannot begin while the machine is in an incompatible state.

### 7.2 Manual mode

Manual mode usually permits authorized individual movements or equipment operations.

The philosophy should define:

Which functions are available.

Which commands require continuous operator action.

Which feedback and interlocks remain mandatory.

Whether simultaneous manual commands are permitted.

How command release affects the equipment.

Which operations require a dedicated setup procedure.

Manual mode must not be used as a general-purpose bypass for automatic-cycle restrictions or safety requirements.

### 7.3 Setup and maintenance modes

Setup and maintenance activities may require different access levels and operating permissions.

The design should define the authorized functions, applicable safeguards, speed restrictions where required, enabling devices where specified, and restrictions on automatic operation.

Maintenance mode does not replace approved energy isolation or lockout/tagout procedures.

### 7.4 Recovery mode

Recovery behavior should be defined for conditions such as:

PLC restart during an active cycle.

Power failure.

Robot or CNC fault.

Part stuck between stations.

Incomplete clamping or unclamping.

Loss of communication.

Aborted sequence.

Uncertain physical equipment state.

Recovery should verify the actual machine condition before allowing continuation. Resuming from a stored step number without checking the physical state can create unexpected movement or damage.

## 8. Start, stop, reset, and restart philosophy

These functions must have distinct definitions.

### 8.1 Start

A start request asks the system to initiate an operation. The system should accept it only when the defined prerequisites are met.

Typical prerequisites include:

Correct mode and command authority.

No blocking fault.

Required equipment available.

Required initial positions confirmed.

Necessary utilities available.

Relevant interlocks satisfied.

Upstream and downstream conditions valid.

### 8.2 Stop

A normal stop should follow the defined stopping strategy. Depending on the equipment, the response may involve controlled deceleration, completing a limited operation, holding a sequence, or terminating movement.

The philosophy should specify what happens to active sequences, actuators, workpieces, and downstream requests.

### 8.3 Reset

Reset clears eligible latched faults when the required conditions are satisfied.

A reset should not automatically initiate production or hazardous movement. Fault acknowledgment, fault reset, and cycle start should be separate functions where required by the design.

### 8.4 Restart

Restart behavior must address:

Fault clearance.

Restoration of utilities.

Power restoration.

PLC or network recovery.

Verification of physical equipment states.

Revalidation of start permissives.

Operator authorization to resume production.

Automatic resumption after power restoration should never be assumed. It must be explicitly justified and permitted by the approved design and applicable safety requirements.

## 9. Equipment control strategies

A control philosophy defines how different classes of equipment are commanded, monitored, and diagnosed.

### 9.1 Motors and contactors

For a conventional motor starter, define:

Start and stop command behavior.

Run feedback.

Overload and protection handling.

Permissives for starting.

Fault latching and reset.

Behavior following loss of feedback.

Local/remote authority.

A motor run command does not prove that the motor is running. The design may require auxiliary contact feedback, speed feedback, or another suitable indication.

### 9.2 Variable-frequency drives

For VFD-controlled motors, define:

Run and stop command ownership.

Speed reference source.

Permitted reference range.

Acceleration and deceleration strategy.

Drive-ready and running feedback.

Fault reporting and reset authority.

Behavior during communication loss.

Local drive control restrictions.

Restart behavior after fault clearance.

The drive's configured response to communication loss must agree with the overall machine control philosophy.

### 9.3 Servo systems

Servo-controlled axes may require:

Enable and disable behavior.

Homing and reference establishment.

Position and velocity commands.

In-position confirmation.

Following-error handling.

Motion limits and interlocks.

Controlled stopping behavior.

Recovery after power loss or drive fault.

The philosophy should distinguish an axis being enabled from an axis being homed, ready, or in position.

### 9.4 Pneumatic and hydraulic actuators

For cylinders, clamps, and hydraulic equipment, define:

Extend and retract commands.

Required position confirmation.

Pressure and utility permissives.

Timeout handling.

Behavior during loss of pressure or electrical power.

Restart and recovery conditions.

Interaction with workpiece handling.

The actual physical response depends on valve configuration, stored energy, actuator design, and mechanical loading. The control philosophy must not assume that de-energizing an output always moves an actuator to a safe position.

### 9.5 Robots and CNC machines

Define the command and handshake boundaries between the machine PLC and the equipment controller.

Typical exchanged states include:

Ready.

Busy.

Faulted.

Cycle start request.

Cycle complete.

Access or transfer request.

Acknowledgment.

Part-present or part-transfer confirmation.

The design must define timeouts, signal-clearing order, communication-loss behavior, and recovery after either controller restarts.

## 10. Interlocks and permissives

The control philosophy establishes the overall rules for interlocks and permissives. Individual conditions belong in the detailed interlock list and related engineering documents.

### 10.1 Permissive philosophy

A permissive is a condition that must be satisfied before an operation may start.

Example:

Conveyor_Start_Permitted =

Remote_Control_Active

AND Drive_Ready

AND Required_Guards_Healthy

AND Downstream_Available

AND NOT Conveyor_Fault

The actual logic must include the approved project requirements. Safety-related conditions must be implemented in the required safety architecture.

### 10.2 Interlock philosophy

An interlock prevents or terminates an operation when a specified condition is not satisfied.

Examples include:

Preventing a clamp operation when the required workpiece condition is not confirmed.

Preventing CNC cycle start when the robot has not cleared the defined transfer area.

Preventing conveyor transfer when the downstream station cannot accept the part.

Inhibiting an operation when a required process utility is unavailable.

The philosophy should define which interlocks apply in each mode, how they are displayed, and how they interact with faults and recovery.

### 10.3 Bypass and override

Bypass and override functions require strict governance.

The philosophy should specify:

Which conditions, if any, can be bypassed.

Who is authorized to approve and apply the bypass.

How the bypass is indicated.

Whether a time limit is required.

How the action is recorded.

What operating restrictions apply.

How restoration is verified.

Required safety functions must not be bypassed through ordinary HMI or PLC logic as a convenience feature. Any permitted safety-related override must follow the approved safety design, risk assessment, and authorization process.

## 11. Fault and alarm philosophy

The control philosophy should establish how abnormal conditions affect equipment and how operators are informed.

### 11.1 Fault categories

Typical categories include:

Process faults.

Equipment faults.

Sequence timeouts.

Sensor and instrumentation faults.

Drive and servo faults.

Robot and CNC faults.

Communication faults.

Safety-system diagnostics.

Utility failures.

Data or traceability faults.

### 11.2 Fault response

For each class of fault, establish whether the expected response is to:

Display a warning.

Inhibit a new cycle.

Hold sequence progression.

Stop an affected operation.

Abort the current sequence.

Trip specified equipment.

Coordinate a line stop.

Enter a recovery state.

The exact response must be documented in the appropriate interlock list, cause-and-effect matrix, alarm list, and sequence description.

### 11.3 Alarm principles

A useful alarm should identify the condition and support action.

Prefer:

Hydraulic pressure low: clamp cycle inhibited

Robot handshake timeout: transfer sequence held

CNC not ready: cycle start blocked

Avoid generic messages such as PLC error when a more precise condition is available.

Alarm acknowledgment should not automatically clear a latched interlock or restart the machine.

## 12. Production-line coordination

For integrated manufacturing lines, the control philosophy defines how stations coordinate production.

Typical rules include:

Upstream equipment must not transfer a part unless the downstream station can accept it.

A station must not report cycle complete before the required completion conditions are met.

A robot must not enter a machine's transfer area without the required access permission.

Production requests and acknowledgments must follow the approved handshake.

Faulted or unavailable stations must communicate their status to dependent equipment.

Workpiece identity and process results must remain associated with the correct part.

Recovery must account for parts that are between stations or have uncertain status.

A line-level control philosophy should also define whether a fault stops only the affected station, a group of stations, or the complete line.

The answer depends on process dependencies, safety, quality, equipment design, and production requirements. Stopping everything for every minor fault is wasteful; continuing through a fault that invalidates part location or process quality is worse.

## 13. HMI and operator interaction philosophy

The HMI should support operation, fault diagnosis, and recovery without obscuring the underlying machine state.

Define:

Screen hierarchy and navigation.

Overview and detailed equipment screens.

Operating-mode indication.

Active control authority.

Start, stop, reset, and permitted manual controls.

Current sequence state and step.

Interlock and permissive diagnostics.

Alarm display and acknowledgment.

Drive, robot, and CNC status.

Recovery instructions.

User roles and access permissions.

Audit records for significant changes.

The operator should be able to determine why a cycle cannot start, what condition is preventing progress, and what action is authorized.

For example, the HMI should ideally identify Downstream station unavailable instead of merely showing Cycle blocked.

HMI controls must not provide an unrestricted route around required interlocks or safety functions.

## 14. PLC software design principles

The control philosophy should define general rules for the PLC software architecture.

### 14.1 Program organization

A maintainable program may separate:

Input acquisition and signal validation.

Equipment control functions.

Sequence controllers.

Interlock evaluation.

Fault and alarm handling.

HMI interface logic.

Communication and handshake management.

Production data and traceability.

Diagnostics and maintenance functions.

This is a logical separation. The physical program organization depends on the PLC platform and project standards.

### 14.2 Naming conventions

Use consistent, meaningful tag names.

Examples:

CV101_RunCmd

CV101_RunFb

CV101_Fault

HYD101_PressurePV

ROB01_Ready

CNC01_CycleComplete

SEQ101_CurrentStep

The project should define naming conventions, data types, units, state meanings, and whether each signal represents a command, feedback, request, acknowledgment, or diagnostic.

### 14.3 Reusable control blocks

Reusable equipment blocks can standardize common functions such as:

Motor control.

VFD control.

Valve control.

Cylinder control.

Analog signal processing.

Alarm generation.

Communication health monitoring.

Reusable blocks should have clearly defined interfaces and documented behavior. A common block must not hide machine-specific requirements or imply that different equipment has identical failure behavior.

### 14.4 Deterministic behavior

For a given valid input state and controller state, the logic should produce predictable behavior.

The design should address:

Initialization.

Scan-cycle effects.

Timer behavior.

Edge detection.

Data validity.

Simultaneous events.

Command priority.

Fault latching.

State retention.

Controller restart.

Communication delays.

Safety-related timing and behavior must meet their separate approved requirements.

## 15. Communication philosophy

The control philosophy should define the behavior of the system when communication is delayed, lost, or invalid.

Potential communication links include:

PLC-to-PLC communication.

PLC-to-robot communication.

PLC-to-CNC communication.

PLC-to-VFD and servo communication.

Remote I/O networks.

SCADA communication.

MES and production-tracking interfaces.

For each critical link, define:

Communication purpose and owner.

Required data and update behavior.

Heartbeat or connection monitoring where appropriate.

Timeout requirements.

Response to lost communication.

Handling of invalid or stale data.

Reconnection behavior.

State reconciliation after reconnection.

Restart authorization.

A stale value must not be treated as proof that the remote device remains healthy. Communication recovery must not automatically resume an operation unless the approved control requirements explicitly permit it.

## 16. Startup and shutdown philosophy

### 16.1 Startup

The startup procedure should define:

Required power and utility conditions.

Controller initialization.

Input and communication validation.

Drive and equipment readiness checks.

Physical-state verification.

Fault-state handling.

Safety-system readiness.

Operator authorization.

Conditions required before production can start.

### 16.2 Normal shutdown

Define how production stops under normal circumstances.

For example, the system may finish the current part, stop accepting new parts, complete a controlled transfer, and then bring the line to its defined idle state.

The exact behavior depends on the production process.

### 16.3 Abnormal shutdown

Abnormal shutdown may result from a fault, power loss, communication failure, or protective action.

The philosophy should define which equipment stops, what states are retained, how work-in-progress is handled, and how the system returns to service.

### 16.4 Power restoration

Power restoration must not be interpreted as permission to resume production automatically.

The system may need to verify physical positions, re-establish drive readiness, check robot and CNC states, confirm workpiece location, and require an operator-authorized restart.

## 17. Data and production tracking

Where the control system interfaces with SCADA, MES, databases, or production dashboards, define the data responsibilities.

Typical information includes:

Machine status.

Cycle start and completion timestamps.

Production count.

Good and rejected quantities.

Fault and downtime reasons.

Part identity.

Test results.

Equipment availability.

Recipe or program identifiers.

Operator and batch information where required.

The philosophy should define which system is the authoritative source for each data item.

For example, the PLC may be authoritative for the current machine state, while MES may be authoritative for the production order. A dashboard should not silently become the source of truth for control-critical decisions simply because it displays convenient data.

Production reporting and historian communication should generally be separated from time-critical machine control so that a reporting-system outage does not cause unintended machine behavior unless the approved requirements explicitly specify otherwise.

## 18. Safety-system philosophy

The control philosophy must identify the boundary between normal automation and safety-related control.

It should establish:

Which safety functions exist.

Which equipment or hazards they protect.

Which safety controller or relay implements them.

How safety status is communicated to the standard PLC.

How safety-related stops affect sequence progression.

How reset and restart are controlled.

How safety diagnostics are displayed.

How validation and change control are managed.

Applicable standards depend on the project and jurisdiction. Common references include ISO 12100, ISO 13849-1, IEC 62061, and IEC 60204-1.

A standard PLC can display safety status and coordinate normal sequence recovery, but it must not be assumed to provide the required safety function merely because its software reacts to a safety input.

The safety architecture, required performance, device selection, and validation must follow the approved risk assessment and safety requirements.

## 19. Example control philosophy for a CNC loading cell

Consider a cell containing a line PLC, robot, CNC machine, hydraulic clamping system, and part inspection equipment.

### 19.1 Operating rules

Automatic production is permitted only when the required equipment and safety conditions are healthy.

The line PLC coordinates production requests and station readiness.

The robot controller owns robot motion.

The CNC controller owns machining motion.

The hydraulic system confirms the required clamping state.

The sequence starts only when the initial physical conditions are valid.

A robot-to-CNC transfer uses a defined request-and-acknowledgment protocol.

Machining starts only when the workpiece and machine conditions are confirmed.

A relevant fault holds or aborts the sequence according to the approved fault-response design.

Fault reset does not initiate a new cycle.

Recovery verifies the physical state and part location before allowing further automatic operation.

Required safety functions remain active in all operating modes in accordance with their design.

### 19.2 Responsibility allocation

| Function | Responsible system | Key responsibility |
| --- | --- | --- |
| Line coordination | Line PLC | Production sequence and station handshakes |
| Robot motion | Robot controller | Robot motion and robot-specific diagnostics |
| Machining | CNC controller | Machining execution and CNC status |
| Clamping | Machine PLC or designated controller | Clamp command and state verification |
| Safety functions | Designated safety system | Required safety-related responses |
| Operator interaction | HMI | Commands, status, alarms, and recovery guidance |
| Production tracking | MES or designated data system | Order, part, and production records |

The exact ownership must be established during design. A shared interface should never leave a critical command or state without a clearly responsible controller.

## 20. Review and approval process

A control philosophy should be reviewed before detailed programming and updated when major design decisions change.

Typical reviewers include:

Process or manufacturing engineering.

Electrical design.

Automation and PLC engineering.

Mechanical engineering.

Safety engineering.

Production and operations.

Maintenance.

IT/OT networking and data engineering, where applicable.

The machine builder or system integrator.

Review should confirm that the document is consistent with process requirements, equipment capabilities, the risk assessment, the control architecture, and the project's acceptance criteria.

Changes should be controlled through revision history, approval records, and links to affected engineering documents.

## 21. Common engineering mistakes

Treating the control philosophy as a generic description copied between projects.

Defining operating modes without specifying permitted behavior.

Failing to assign control ownership.

Allowing multiple controllers to command the same equipment without arbitration.

Confusing a stop request with an emergency-stop function.

Leaving reset and restart behavior undefined.

Ignoring communication loss and stale data.

Allowing SCADA or database failures to affect machine control unpredictably.

Defining only normal production behavior.

Using broad instructions such as “stop the machine” without specifying the intended result.

Treating Manual or Maintenance mode as an unrestricted bypass.

Failing to distinguish safety-system behavior from standard PLC behavior.

Allowing the HMI to conceal the reason a sequence is blocked.

Leaving the control philosophy inconsistent with the sequence description, interlock list, and software implementation.

Making major design decisions during commissioning without updating the approved documents.

## 22. Interview questions and answers

### Q1. What is a control philosophy?

It defines the overall operating and control principles governing a machine or process, including operating modes, command ownership, interlocks, fault handling, restart behavior, communications, and operator interaction.

### Q2. How is it different from a sequence description?

The control philosophy establishes system-wide rules. The sequence description defines the ordered steps and transitions for a specific operation.

### Q3. Why should the control philosophy be prepared before PLC programming?

It resolves architectural and behavioral decisions early, reduces ambiguity, supports consistent implementation, and provides a basis for software requirements and acceptance testing.

### Q4. What is the difference between Local/Remote and Auto/Manual?

Local/Remote generally defines where commands originate or which control source has authority. Auto/Manual defines how the equipment operates. They are separate concepts and may be combined.

### Q5. Why should reset and start be separate?

Reset clears eligible fault conditions. Start requests a new operation. Separating them prevents fault clearance from unintentionally initiating movement or production.

### Q6. What should happen after a communication failure?

The system should follow its defined response, which may include inhibiting new operations, stopping or holding affected equipment, reporting the fault, and requiring state verification before recovery.

### Q7. Why is control ownership important?

It prevents conflicting commands and clarifies which controller is responsible for each operation, feedback, and recovery action.

### Q8. Does Manual mode disable interlocks?

No. Manual mode changes the permitted operating method, but required interlocks and safety functions remain effective according to the approved design.

### Q9. How should a control philosophy address safety?

It should identify safety functions, interfaces, operating restrictions, reset and restart rules, and responsibilities. Detailed safety requirements, implementation, and validation remain governed by the applicable safety engineering documentation.

### Q10. How does the control philosophy support maintenance?

It gives maintenance engineers a reference for expected equipment behavior, fault responses, command authority, and recovery rules, making troubleshooting more systematic.

### Q11. What is the relationship between control philosophy and software specification?

The control philosophy establishes the overall rules. The software specification translates those rules into detailed software requirements, program structure, interfaces, diagnostics, and implementation constraints.

### Q12. What makes a control philosophy effective?

It is specific to the system, resolves important design decisions, assigns responsibility, defines abnormal behavior, and remains consistent with the detailed engineering documents and implemented control system.

## 23. Final control philosophy checklist

System boundaries and control architecture are defined.

Command ownership and Local/Remote behavior are explicit.

Auto, Manual, Setup, Maintenance, and Recovery modes are specified.

Start, stop, hold, abort, reset, and restart rules are documented.

Equipment control strategies are defined.

Interlocks, permissives, alarms, and fault responses are consistent.

Communication failure and reconnection behavior are addressed.

Startup, shutdown, power loss, and controller restart are covered.

Safety-system responsibilities and boundaries are clear.

HMI diagnostics and operator permissions are specified.

Production data ownership and supervisory-system dependencies are defined.

The philosophy agrees with the sequence description and related documents.

Review, approval, testing, and change-control requirements are established.

## NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\10_Detail_Eng\13_Software_Spec - Software specification