# CNC Programs

## 1. Definition and Purpose

A CNC program is a structured set of instructions that tells a Computer Numerical Control (CNC) machine how to perform a machining operation. It defines the tool movements, spindle operation, feed rates, coordinate systems, tool selections, machining cycles, and supporting machine functions required to manufacture a component.

CNC programs are commonly written using G-code and M-code conventions, although the exact syntax and available functions depend on the controller and machine builder.

A CNC program can control or request operations such as:

Axis movement along linear and circular paths.

Spindle start, stop, direction, and speed.

Tool selection and automatic tool changes.

Coolant operation.

Coordinate-system selection.

Tool-length and cutter-radius compensation.

Drilling, boring, tapping, milling, and turning cycles.

Program branching, subprogram calls, and supported macro operations.

Controlled stops and program completion.

CNC programs are essential to production quality, repeatability, cycle-time optimization, machine safety, and traceability.

Important distinction: A CNC program defines the machining operation. CNC parameters define controller and machine behavior. The PLC controls many machine sequences and interlocks. These layers interact, but they are not interchangeable.

## 2. CNC Program Structure

A typical CNC program contains a program identifier, initialization commands, tool selection, machining instructions, and a safe ending.

Example of a simplified milling program:

%

O1001 (SAMPLE MILLING PROGRAM)

G21 G17 G90 G40 G49 G80

G54

T1 M06

S1500 M03

G00 X0 Y0

G43 H01 Z50.0

M08

G00 X10.0 Y10.0

G01 Z-2.0 F100

G01 X50.0 F250

G00 Z50.0

M09

M05

G49

G53 G00 Z0

M30

%

This is an illustrative example, not a production-ready program. The tool, workholding, coordinates, spindle limits, clearance positions, tool-change position, machine configuration, and controller syntax must be verified before execution. In particular, the machine-coordinate movement and tool-change sequence may require different instructions on different machines.

Common program elements include:

| Element | Purpose |
| --- | --- |
| Program number or name | Identifies the program |
| Initialization block | Establishes required modal states |
| Work coordinate selection | Selects the workpiece coordinate system |
| Tool selection | Identifies the required tool |
| Tool change | Requests a tool change where supported |
| Spindle command | Sets spindle direction and speed |
| Positioning commands | Move axes to specified coordinates |
| Feed commands | Define the programmed feed rate |
| Machining cycle | Executes a defined machining operation |
| Coolant commands | Control applicable coolant functions |
| End-of-program command | Ends or resets program execution |

The exact program format depends on the CNC controller, its configuration, and the machine builder's implementation.

## 3. G-Code and M-Code

### 3.1 G-code

G-codes generally control motion, coordinate interpretation, compensation, machining modes, and canned cycles.

Common examples include:

| Code | Common meaning | Important consideration |
| --- | --- | --- |
| G00 | Rapid positioning | Rapid motion requires verified clearance |
| G01 | Linear interpolation | Requires an appropriate feed setting |
| G02 | Clockwise circular interpolation | Plane and arc format matter |
| G03 | Counterclockwise circular interpolation | Plane and arc format matter |
| G17 | XY plane selection | Common on milling controls |
| G18 | XZ plane selection | Common on milling and turning controls |
| G19 | YZ plane selection | Availability depends on controller |
| G20 | Inch programming mode | Units must match the program |
| G21 | Metric programming mode | Units must match the program |
| G28 | Reference return through a specified intermediate position on many controls | Exact behavior and safe usage are controller-dependent |
| G40 | Cancel cutter-radius compensation | Confirm the applicable compensation state |
| G41 | Cutter-radius compensation left | Requires correct tool data and lead-in geometry |
| G42 | Cutter-radius compensation right | Requires correct tool data and lead-in geometry |
| G43 | Positive tool-length compensation on many milling controls | Verify H-offset conventions |
| G49 | Cancel tool-length compensation on many milling controls | Confirm the resulting tool-position behavior |
| G54 | Select a work coordinate system | The stored offset must be correct |
| G80 | Cancel canned cycle on many controls | Confirm the active cycle is cancelled |
| G90 | Absolute programming on many controls | Interpret coordinates from the active coordinate system |
| G91 | Incremental programming on many controls | Movement depends on the current position |
| G94 | Feed per minute on many controls | Common milling feed mode |
| G95 | Feed per revolution on supported controls | Common in turning and synchronized operations |

These meanings are common conventions, not a guarantee of identical behavior on every CNC. Some codes have controller-specific extensions or different meanings.

### 3.2 M-code

M-codes generally request auxiliary machine functions.

| Code | Common meaning | Important consideration |
| --- | --- | --- |
| M00 | Program stop | Requires operator intervention to continue |
| M01 | Optional stop | Acts according to the optional-stop setting |
| M02 | Program end | Reset behavior varies by control |
| M03 | Spindle clockwise | Direction must match the tool and operation |
| M04 | Spindle counterclockwise | Confirm machine and tool requirements |
| M05 | Spindle stop | Does not necessarily mean every motion is stopped |
| M06 | Tool change on many milling controls | Often requires machine-specific sequencing |
| M08 | Coolant on, commonly flood coolant | Actual function depends on the machine |
| M09 | Coolant off | Confirm all relevant coolant functions |
| M30 | Program end and reset on many controls | Exact reset behavior is controller-dependent |

M-code assignments are especially dependent on the machine builder. A code that controls coolant on one machine may control a different auxiliary function on another. Never assume that an M-code is safe based only on its number.

## 4. Modal and Non-Modal Commands

Modal commands remain active until another command changes their state or the controller resets them. Non-modal commands generally apply only to the current block or operation.

Examples of commonly modal states include:

Motion mode, such as G00 or G01.

Coordinate interpretation, such as G90 or G91.

Plane selection, such as G17 or G18.

Units, such as G20 or G21.

Feed mode.

Work coordinate selection.

Cutter and tool-length compensation states.

A dangerous programming mistake occurs when a programmer assumes that a command has automatically returned to its default state.

For example, if incremental positioning remains active, a coordinate value may be interpreted as a movement from the current position rather than as an absolute destination.

A good program explicitly establishes the required initial states rather than relying on whatever settings happened to remain from the previous operation.

## 5. Coordinate Systems and Positioning

CNC programming involves several coordinate concepts that must be kept separate.

### 5.1 Machine coordinates

Machine coordinates reference the machine's established coordinate system. Their relationship to the physical machine depends on successful reference establishment and the controller's coordinate conventions.

### 5.2 Work coordinates

Work coordinate systems define the location of the workpiece relative to the machine coordinate system. Common milling systems include G54 through G59, with additional systems available on some controls.

An incorrect work offset can shift an otherwise correct program into a fixture, workpiece, or machine component.

### 5.3 Tool offsets

Tool-length and cutter-radius offsets account for tool geometry. Their meaning and activation depend on the CNC configuration.

A program can contain correct coordinates but still machine the wrong location if the wrong work offset or tool offset is active.

### 5.4 Absolute and incremental positioning

In absolute positioning, a coordinate represents a destination relative to the active coordinate system.

In incremental positioning, a coordinate represents a displacement from the current position.

For example, a 10 mm command in absolute mode may request a position of X10, whereas the same numeric value in incremental mode may request a 10 mm movement from the current location.

The active mode must be confirmed before running or editing a program.

## 6. Tool Selection and Tool Compensation

CNC programs frequently combine tool selection with offset activation.

Typical functions include:

Selecting a tool number.

Requesting a tool change.

Activating a tool-length offset.

Activating cutter-radius compensation.

Selecting tool geometry and wear data.

Returning to a safe position before or after a tool change.

The tool number and offset number are not necessarily identical. For example, T1 may select a tool while H01 selects a tool-length offset, depending on the controller and machine convention.

Before production, verify:

The correct physical tool is installed.

The tool identity matches the program.

Tool length and radius data are correct.

Tool wear values are reasonable.

The tool assembly fits the machine and workholding arrangement.

Clearance exists during rapid positioning and tool changes.

Compensation is activated and cancelled in the intended sequence.

An incorrect tool offset can cause dimensional errors or collisions even when the programmed tool path is correct.

## 7. Feed Rate, Spindle Speed, and Cutting Conditions

CNC programs define cutting conditions that must remain within the capability of the machine, tool, workpiece, and process.

### 7.1 Feed rate

Feed rate defines the commanded rate of tool movement under the active feed mode.

Common modes include feed per minute and feed per revolution. Their availability and behavior depend on the controller and operation.

### 7.2 Spindle speed

Spindle speed is commonly programmed in revolutions per minute. The permitted range depends on the spindle, drive, gear range, tool, workholding, and machine configuration.

### 7.3 Feed and speed relationship

For milling, cutting speed is related to spindle speed and cutter diameter:

Vc=πDN1000

Where:

Vc is cutting speed in metres per minute.

D is cutter diameter in millimetres.

N is spindle speed in revolutions per minute.

For milling, the theoretical feed rate can be calculated as:

F=fz×z×N

Where:

F is feed rate in millimetres per minute.

fz is feed per tooth in millimetres per tooth.

z is the number of cutting teeth.

N is spindle speed in revolutions per minute.

These equations support cutting-condition calculations, but the result must be checked against the tooling manufacturer's recommendations and the machine's capabilities.

## 8. Canned Cycles

Canned cycles simplify repetitive machining operations by replacing multiple low-level movement commands with a defined cycle.

Common operations include:

Drilling.

Peck drilling.

Boring.

Tapping.

Spot drilling.

Repeated hole machining.

Turning cycles on supported lathes.

Cycle codes and parameters differ across CNC families. The same cycle number must not be assumed to have identical behavior across different controllers.

Before using a canned cycle, confirm:

The cycle type and syntax.

The active plane and coordinate mode.

The hole position and depth.

The retract plane and return behavior.

The feed rate and spindle mode.

The required tool and work offsets.

Whether the cycle is modal.

The cancellation command and its behavior.

Incorrect retract settings or cycle depth values can cause a tool to collide with the workpiece or fixture.

## 9. Subprograms and Macros

### 9.1 Subprograms

A subprogram is a reusable sequence of instructions called from another program. It can reduce repetition and make a machining process easier to maintain.

Typical applications include repeated hole patterns, common machining sequences, and standardized operations.

Subprogram call syntax and return behavior depend on the CNC controller.

### 9.2 Macro programming

Macro programming introduces variables, calculations, conditions, and program flow into CNC programming.

Possible applications include:

Parametric machining.

Repeated patterns.

Calculated coordinates.

Conditional machining operations.

Part-family programs.

Measurement-based corrections where supported.

Production counters and controlled program branching.

Macro variables and syntax differ between controller families. Variable ranges, persistence, arithmetic behavior, and protected system variables must be checked in the applicable manual.

A poorly designed macro can repeatedly execute an incorrect movement or select the wrong machining conditions. Validate its logic and boundary conditions before production.

## 10. CNC Program Execution and Operating Modes

CNC programs are executed through a controlled sequence of operator and machine actions.

Common operating modes include:

EDIT: Create or modify a program.

MEMORY or AUTO: Execute a stored program.

MDI: Execute manually entered commands or short command sequences.

JOG: Move axes manually using the supported jog controls.

HANDLE or MPG: Move axes using a manual pulse generator.

SINGLE BLOCK: Execute one program block at a time.

DRY RUN: Use the controller's supported test behavior to check a program.

GRAPHICS: Simulate the tool path without normal physical cutting motion, where supported.

The exact modes and their behavior depend on the machine.

Single-block operation and graphics simulation are useful validation tools, but neither guarantees collision-free machining. Dry-run behavior may change feed rates or other functions and may not reproduce actual cutting conditions.

Before execution, verify the selected program, active tool, work offset, tool offsets, spindle commands, machine state, workholding, and travel envelope.

## 11. Program Validation Before Production

Program validation should be systematic rather than based on the optimistic belief that the previous shift probably checked everything.

### 11.1 Document and revision checks

Confirm the correct program number and file.

Verify the approved revision.

Check the part number and machining operation.

Confirm that the program matches the machine and controller.

Check any required postprocessor or machine-specific conventions.

Verify that the required tools and offsets are available.

### 11.2 Code and logic checks

Review initialization and modal states.

Confirm units and coordinate interpretation.

Check work coordinate selection.

Verify tool-change commands.

Confirm spindle direction, speed, and feed values.

Check movement order and clearance positions.

Review depth values, cycle parameters, and retract behavior.

Confirm compensation activation and cancellation.

Verify subprogram calls and macro logic.

Check end-of-program and reset behavior.

### 11.3 Physical and process checks

Verify workpiece clamping and fixture clearance.

Confirm the tool and holder assembly.

Check the machine's available travel.

Verify that spindle and feed commands are within approved limits.

Inspect the first-part measurement results.

Confirm the required surface finish and dimensional tolerances.

## 12. CNC Program Backup, Transfer, and Version Control

CNC programs may be stored in controller memory, an external computer, a DNC system, a network location, or a production-management system.

Program transfer methods may include USB storage, Ethernet, serial communication on legacy equipment, or approved networked manufacturing systems.

Important controls include:

Unique program identification.

Revision history.

Approved master copies.

Access control.

Transfer verification.

Machine-specific compatibility.

Traceability between the program and manufactured part.

Retention of validated previous revisions.

Protection against unauthorized edits.

The program displayed on the CNC should be verified against the approved version. A file with the correct name may still contain an incorrect revision.

Do not assume that copying a program file preserves tool offsets, work offsets, parameters, compensation tables, or machine-builder configuration. These are separate data sets unless the specific backup method explicitly includes them.

## 13. Common CNC Program Faults

| Symptom | Possible program-related cause | Additional checks |
| --- | --- | --- |
| Incorrect machining location | Wrong work coordinate system or coordinate values | Work offset, machine reference, setup |
| Unexpected axis movement | Incorrect modal state or coordinate interpretation | Active modes, previous blocks, offsets |
| Tool collision during rapid movement | Incorrect clearance or movement sequence | Fixture geometry, tool assembly, actual machine position |
| Incorrect component dimensions | Wrong tool offsets or programmed geometry | Tool measurement, work offset, tool wear |
| Spindle does not start | Missing or incorrect spindle command | Interlocks, spindle drive, PLC permissives |
| Tool change does not complete | Incorrect tool-change request or sequence | PLC status, tool changer sensors, mechanical condition |
| Canned cycle produces incorrect depth | Wrong cycle parameters or coordinate mode | Controller-specific cycle definition |
| Macro behaves unexpectedly | Incorrect variable, condition, or loop logic | Variable values, program flow, limits |
| Program stops with an alarm | Invalid code or unmet operating condition | Alarm history, controller manual, machine status |
| Program differs from approved file | Unauthorized edit or transfer mismatch | Revision control, file comparison, transfer logs |

A program alarm does not automatically mean that the program is defective. Machine interlocks, tool data, configuration, feedback, and mechanical conditions can also prevent execution.

## 14. Preventive Maintenance for CNC Programs

Program management is a maintenance and production-control responsibility, not merely a programming task.

A practical periodic review can include:

Confirming that approved programs are available.

Checking that master copies and revisions are traceable.

Verifying that required backups are current and readable.

Reviewing unauthorized or undocumented program changes.

Checking for obsolete programs and duplicate versions.

Confirming machine compatibility after CNC software or hardware changes.

Reviewing recurring program-related alarms and production defects.

Confirming that tool and work-offset procedures are documented.

Verifying recovery arrangements after controller replacement or memory loss.

### 14.1 Program change register

A program change register may contain:

| Field | Purpose |
| --- | --- |
| Machine ID | Identifies the CNC machine |
| Program number | Identifies the program |
| Part number | Links the program to the manufactured component |
| Revision | Identifies the approved version |
| Change description | Explains what was modified |
| Reason for change | Links the change to a process or quality requirement |
| Original file reference | Preserves the previous approved version |
| Approval | Records required authorization |
| Validation result | Documents testing and inspection |
| Release date | Records when the revision became approved |
| Responsible person | Establishes accountability |

In SAP PM or an associated document-management system, program-related work can be linked to the equipment, functional location, maintenance order, technical documentation, or change-control record, according to the site's workflow.

## 15. Practical CNC Program Troubleshooting Method

When a program fails, use a structured investigation.

1.	Record the exact alarm text or number, active program, block number, operating mode, and machine condition.

2.	Determine whether the fault is repeatable and whether it began after a program edit, tool change, offset change, parameter change, or maintenance activity.

3.	Confirm that the program revision is correct.

4.	Check the active coordinate system, units, absolute or incremental mode, compensation state, tool data, and spindle/feed commands.

5.	Review the relevant program blocks and preceding modal commands.

6.	Check the CNC diagnostics and PLC status for unmet machine conditions.

7.	Inspect relevant sensors, actuators, drives, feedback, and mechanical components where indicated.

8.	Correct the confirmed cause using the approved procedure.

9.	Validate the change in a controlled manner before returning the machine to production.

10.	Record the fault, cause, corrective action, and verified result.

Do not delete or bypass an alarm simply to resume production. The alarm may be the only indication that a required condition has not been met.

## 16. Key Engineering Principles

CNC programs must match the exact machine, controller, tooling, workholding, and approved process.

Program syntax and code meanings can vary between manufacturers and machine configurations.

Initialization should establish required modal states explicitly.

Coordinate systems, tool offsets, machine parameters, and compensation data must be treated as separate configuration elements.

Program simulation helps detect errors but does not prove that physical machining is safe.

Changes must be version-controlled, authorized, and validated.

A backup of the program alone does not constitute a complete CNC machine backup.

Repeated faults should be investigated for their underlying causes rather than repeatedly corrected at the program level.

Production release requires both program validation and confirmation of the actual machine setup.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\18_PM\05_CNC\08_Backup - Backup