# Industrial Robot Cables

## 1. Definition and Purpose

Industrial robot cables carry electrical power, feedback signals, communication data, and control signals between the robot controller, servo motors, encoders, brakes, sensors, end-of-arm tooling, and external automation equipment.

Robot cabling is especially important because cables installed along robot arms experience repeated bending, twisting, acceleration, vibration, and movement. A cable that works correctly in a stationary cabinet may fail prematurely when installed on a moving robot axis.

Cable faults can produce intermittent servo alarms, encoder errors, communication loss, unexpected I/O changes, brake-control faults, and production stoppages. Because some failures occur only at particular robot positions, diagnosis often requires correlating cable movement with alarm history.

## 2. Main Types of Industrial Robot Cables

| Cable type | Main purpose | Typical connections |
| --- | --- | --- |
| Motor power cable | Supplies electrical power to the servo motor | Servo drive to robot motor |
| Encoder or feedback cable | Carries position and speed feedback signals | Motor feedback device to servo drive |
| Brake cable or brake conductors | Supplies or controls the motor holding brake | Controller or drive interface to motor brake |
| Robot signal cable | Carries discrete control and sensor signals | Robot controller, sensors, grippers |
| Industrial Ethernet cable | Carries network data | Robot controller to PLC or network switch |
| Fieldbus cable | Carries industrial communication data | Robot controller to remote I/O or automation network |
| Safety circuit cable | Carries safety-related signals where applicable | Safety devices and safety-control interfaces |
| Tool or dress-pack cable | Connects end-of-arm tools and process equipment | Robot wrist to gripper, welding gun, or dispenser |
| External-axis cable | Connects equipment such as positioners and track axes | Controller or drive to external-axis motor and feedback |
| Teach-pendant cable | Connects the pendant to the controller where a wired design is used | Teach pendant to controller |

The cable construction, connector type, shielding, and electrical ratings must match the specific controller and robot model.

## 3. Robot Cable Architecture

A typical robot installation includes stationary cabinet wiring and moving cables routed along the robot.

Robot Controller Cabinet

↓

Servo Drive

↓

Robot Motor Power Cable

↓

Robot Joint Motor

Motor Feedback Device

↓

Feedback Cable

↓

Servo Drive / Controller

Controller I/O and Communication

↓

Robot Signal and Network Cables

↓

Sensors / PLC / End-of-Arm Tooling

The actual cable layout depends on the robot architecture. Some conductors may be combined within a manufacturer-supplied robot cable assembly, while other connections use separate cables.

## 4. Motor Power Cables

Motor power cables carry the controlled electrical power supplied by the servo drive to the motor.

They must withstand the voltage, current, switching characteristics, temperature, bending, and environmental conditions specified for the application.

### Common failure mechanisms

Conductor fatigue caused by repeated bending.

Insulation damage.

Connector loosening.

Cable crushing or abrasion.

Oil or chemical degradation.

Incorrect routing near moving joints.

Damage caused by collisions or maintenance activities.

Inadequate shielding or grounding where required.

### Typical symptoms

Servo drive overcurrent or short-circuit alarms.

Intermittent axis faults during movement.

Motor operation that changes with cable position.

Insulation faults.

Unexpected drive trips.

Localized heating or visible connector damage.

A motor power cable fault can resemble a motor or servo-drive failure. The diagnosis must distinguish the cable, motor, drive, and mechanical load.

## 5. Encoder and Feedback Cables

Feedback cables carry position-related signals from an encoder, resolver, or other feedback device to the relevant control electronics.

These signals are essential for accurate closed-loop motion control.

### Common failure mechanisms

Broken conductors.

Damaged insulation.

Loose connector contacts.

Contamination or moisture ingress.

Shielding or grounding defects.

Electrical interference.

Connector damage caused by repeated movement.

Incorrect replacement cable or pinout.

### Typical symptoms

Encoder or resolver alarms.

Loss of position feedback.

Servo-enable failure.

Intermittent position errors.

Unexpected axis stops.

Faults that occur only at certain joint positions.

Mastering or calibration-related warnings following a feedback-system repair.

An absolute encoder may retain position information, but cable replacement, feedback-device replacement, or interruption of its required retention system may still trigger manufacturer-specific recovery procedures. Do not assume that an absolute encoder eliminates all mastering or calibration requirements.

## 6. Cable Shielding and Electrical Noise

Robot cables may carry low-level feedback signals close to motor power conductors and other sources of electrical noise. Proper shielding, grounding, separation, and routing help preserve signal integrity.

### 6.1 Sources of electrical interference

Servo-drive switching.

Variable-frequency drives.

Welding equipment.

High-current conductors.

Contactors and solenoids.

Poor grounding or bonding.

Incorrect cable termination.

Damaged shields.

Long or poorly routed signal cables.

### 6.2 Shielding principles

Use the cable type specified by the manufacturer.

Maintain the specified shield termination and bonding arrangement.

Keep sensitive feedback cables separated from power cables where required.

Avoid unnecessary parallel routing with high-current conductors.

Maintain cabinet grounding and protective bonding.

Use appropriate connectors and shield clamps.

Do not improvise shield connections without understanding the design.

Shielding arrangements vary by cable and control system. A universal rule such as connecting every shield at only one end is not appropriate for every industrial servo or communication installation.

## 7. Cable Flexibility and Continuous Motion

Robot cables must tolerate repeated motion over their intended service life. Ordinary fixed-installation cable may not withstand the same movement.

### 7.1 Flex-rated cable

Flex-rated cable is designed for repeated bending within specified conditions.

### 7.2 Torsion-rated cable

Torsion-rated cable is designed to withstand repeated twisting around its longitudinal axis within specified limits.

### 7.3 Robot dress packs

A dress pack is the cable and hose arrangement routed along the robot arm to supply power, signals, air, coolant, or process media to the end effector.

A poorly arranged dress pack may snag, rub against the robot, exceed its twist limits, or become stretched during movement.

### Important cable-routing factors

Minimum bend radius.

Permitted torsion angle and direction.

Cable length and slack.

Clamp positions.

Strain relief.

Robot axis travel.

Collision exposure.

Abrasion protection.

Temperature and chemical exposure.

Separation from hot or sharp surfaces.

The cable manufacturer's bend-radius rating and the robot manufacturer's routing instructions must both be satisfied.

## 8. Minimum Bend Radius and Torsion

The minimum bend radius is the smallest permitted radius through which a cable may bend under specified conditions.

If a cable is bent too tightly, its conductors, insulation, shielding, or internal construction can be damaged.

For example, if a cable's specified minimum bend radius is 100 mm, routing it around a 50 mm radius is outside that stated limit.

The numerical value in this example is illustrative. Use the actual cable specification.

Torsion is twisting along the cable's length. It differs from ordinary bending and can be especially important at robot wrists, where several axes rotate in different directions.

A cable may have an adequate bend radius but still fail because the permitted torsion has been exceeded.

## 9. Connectors, Plugs, and Cable Interfaces

Connectors provide electrical and mechanical interfaces between cables and equipment.

Common connector issues include:

Loose locking mechanisms.

Bent or damaged pins.

Contamination.

Corrosion.

Damaged seals.

Broken strain relief.

Incorrect connector orientation.

Improper pinout.

Inadequate tightening.

Poor contact caused by mechanical wear.

Before disconnecting a connector, identify its function and follow the manufacturer's isolation procedure.

Never pull a connector by its cable. Use the intended connector body, locking mechanism, and specified removal method.

## 10. Cable Fault Symptoms and Diagnostic Direction

| Symptom | Possible cause | Initial diagnostic direction |
| --- | --- | --- |
| Intermittent servo fault | Damaged power or feedback cable, connector issue | Correlate fault timing with axis position and movement |
| Encoder alarm | Feedback conductor, connector, shield, or sensor fault | Check feedback diagnostics and inspect the approved cable path |
| Communication loss | Damaged network cable, connector, switch, or configuration | Check link status, device status, and network diagnostics |
| Motor trips during movement | Cable damage, motor fault, drive fault, mechanical resistance | Review the exact alarm and test the relevant components safely |
| Fault occurs near one robot position | Cable flexing, torsion, snagging, or joint-related issue | Inspect cable routing and movement limits |
| Gripper I/O changes intermittently | Signal cable damage, connector issue, sensor or supply fault | Monitor the I/O state and inspect the tool wiring |
| Repeated noise-related feedback errors | Shielding, grounding, routing, or interference problem | Review cable construction, shield termination, and routing |
| Visible cable cracking | Ageing, abrasion, chemical exposure, or excessive movement | Assess the full damaged section and replacement requirements |
| Cable becomes unusually hot | Electrical overload, poor connection, incorrect cable, or environmental heating | Isolate safely and inspect against rated specifications |
| Safety signal intermittently drops | Safety circuit wiring or device fault | Follow the validated safety-system diagnostic procedure |

Do not repeatedly reset faults without investigating the cause. Intermittent cable faults can worsen with continued motion.

## 11. Systematic Robot Cable Troubleshooting

A structured investigation reduces unnecessary replacement of motors, encoders, and servo drives.

1.	Record the fault. Capture the exact alarm, affected axis or device, robot position, operating mode, and timestamp.

2.	Check the event sequence. Determine whether the cable-related symptom preceded other faults.

3.	Identify the relevant circuit. Use the wiring diagram, cable schedule, connector drawing, and I/O list.

4.	Inspect the cable route. Look for abrasion, crushing, stretching, excessive twist, sharp bends, and snagging.

5.	Inspect connectors. Check locking, sealing, contamination, and visible contact damage.

6.	Correlate the fault with movement. Determine whether it occurs repeatedly at a particular joint angle or tool position.

7.	Perform approved electrical checks. Use the specified test equipment and isolation procedure.

8.	Verify cable specifications. Confirm part number, pinout, shielding, voltage rating, flex rating, and connector compatibility.

9.	Replace or repair only as permitted. Robot and safety-related cable assemblies may require approved replacement rather than field splicing.

10.	Validate the repair. Test the affected axis or function through the required operating range under controlled conditions.

11.	Record the result. Document the cause, cable identification, corrective action, and verification results.

### Important testing precautions

Isolate electrical energy before handling power or feedback connections.

Confirm that stored energy has been discharged according to the procedure.

Never disconnect motor or feedback connectors while energized unless the manufacturer explicitly authorizes that procedure.

Do not apply insulation-test voltage to connected encoders, servo drives, or electronic modules unless the test procedure explicitly permits it.

Do not use a continuity result alone to conclude that a moving cable is healthy. A conductor may pass a stationary test but fail when flexed.

Do not bend, twist, or manipulate energized cables to reproduce a fault unless a specifically approved diagnostic procedure permits it.

## 12. Continuity, Insulation, and Shield Tests

### 12.1 Continuity testing

Continuity testing checks whether an electrical path is complete.

It can help identify an open conductor, but it may not detect every intermittent fault, high-resistance connection, or insulation problem.

For moving robot cables, any flexing test must follow an approved method with the cable isolated and disconnected as required.

### 12.2 Insulation resistance testing

Insulation resistance testing evaluates insulation between specified conductors or between conductors and the appropriate reference.

The permitted test voltage and test configuration depend on the cable and equipment.

Never apply a megohmmeter directly to an encoder, servo drive, or electronic device unless the manufacturer explicitly specifies a safe test method. Disconnecting sensitive equipment may be necessary, and some cable assemblies have additional testing restrictions.

### 12.3 Shield and grounding checks

Verify the specified shield termination, continuity, bonding, and protective grounding arrangements.

A shield problem can cause intermittent signal errors even when the signal conductor itself has continuity.

Protective earth and signal shielding have related but distinct functions. Do not substitute one for the other without a validated design.

## 13. Cable Replacement and Installation

Cable replacement must preserve the electrical, mechanical, and environmental characteristics required by the robot.

### Replacement procedure

1.	Identify the cable part number, circuit, endpoints, connector types, and routing.

2.	Obtain the correct manufacturer-approved replacement.

3.	Save relevant fault details and configuration information.

4.	Place the robot and cell in the specified safe state.

5.	Isolate and verify the relevant energy sources.

6.	Support moving components and prevent hazardous movement.

7.	Label connectors and record routing where required.

8.	Remove the damaged cable without damaging adjacent wiring or seals.

9.	Install the replacement using the specified routing, clamps, and strain relief.

10.	Maintain minimum bend radius and permitted torsion.

11.	Verify connector engagement, locking, pinout, and shield termination.

12.	Restore connections and complete the required electrical checks.

13.	Perform controlled motion and functional testing.

14.	Verify affected communication, I/O, feedback, and safety functions.

15.	Update the maintenance record.

For robot-specific cable assemblies, a compatible-looking generic cable may still be unsuitable because of flex life, torsion rating, impedance, shielding, pinout, or connector requirements.

## 14. Cable Management and Dress-Pack Maintenance

Cable management prevents mechanical damage and reduces intermittent faults.

### Inspection checklist

Inspect moving cables for cracks, cuts, abrasion, and deformation.

Check cable clamps and strain relief.

Verify the dress pack does not snag or stretch during robot movement.

Check bend radius at moving joints.

Check torsion around wrist axes.

Inspect protective sleeves and conduits.

Check connector locking and sealing.

Verify cable separation where required for noise control.

Inspect for oil, coolant, welding spatter, heat, and chemical damage.

Review cable-related alarms and intermittent faults.

Replace damaged protective components as specified.

Record defects and corrective actions.

Cable inspections should consider the complete robot movement envelope, not just the cable condition when the robot is parked.

## 15. Robot Cable Failure Analysis

A failed cable should be investigated for the mechanism that caused the damage.

| Failure mechanism | Possible underlying cause | Preventive action |
| --- | --- | --- |
| Conductor fatigue | Repeated bending beyond design conditions | Use the correct flex-rated cable and routing |
| Torsional damage | Excessive twist at wrist or rotating joints | Correct the routing and torsion allowance |
| Abrasion | Contact with sharp edges or moving structures | Improve protection and clearance |
| Connector damage | Pulling cables, poor strain relief, repeated impact | Improve handling and connector support |
| Insulation breakdown | Heat, chemical exposure, ageing, or electrical stress | Use suitable cable construction and environmental protection |
| Electrical noise | Incorrect shielding, bonding, or cable separation | Correct installation according to the design |
| Water or oil ingress | Damaged seals or unsuitable environmental rating | Restore sealing and use suitable components |
| Cable crushing | Collision, pinching, or poor routing | Remove the pinch point and verify the movement envelope |
| Repeated breakage at one location | Localized bending, poor support, or excessive movement | Inspect the complete motion path and redesign the routing if approved |

Repeatedly replacing a cable without correcting the routing or mechanical cause is not a sustainable repair.

## 16. Cable Identification and Documentation

A structured cable register supports troubleshooting, replacement, and modification work.

| Field | Purpose |
| --- | --- |
| Equipment ID | Identifies the robot or cell |
| Cable tag | Provides a unique cable reference |
| Cable function | Identifies power, feedback, communication, signal, or safety use |
| Source and destination | Defines cable endpoints |
| Cable part number | Supports correct replacement |
| Connector details | Identifies interface types |
| Routing reference | Points to the approved cable route |
| Flex and torsion requirements | Captures moving-cable constraints |
| Shielding requirements | Records specified termination and bonding |
| Inspection date | Tracks maintenance history |
| Fault and repair details | Records observed failures and corrective actions |

Use the plant's approved drawings and equipment documentation to maintain traceability. In SAP PM, cable-related work may be associated with the relevant robot equipment, functional location, maintenance order, and technical documents.

## 17. Practical Industrial Example

Problem: A robot reports an intermittent encoder fault only when the wrist moves toward a particular orientation.

A suitable investigation is:

1.	Record the affected axis, alarm code, and wrist orientation.

2.	Review the fault history to confirm whether the fault correlates with a particular movement.

3.	Inspect the wrist cable routing, dress pack, clamps, and connectors with the system in the required safe state.

4.	Check for excessive twist, sharp bending, pinching, or damage near the moving joint.

5.	Use the manufacturer-approved procedure to inspect and test the feedback cable.

6.	Verify the connector, pinout, shield arrangement, and cable part number.

7.	Replace the damaged cable assembly if the fault is confirmed and replacement is required.

8.	Check the routing through the required movement envelope.

9.	Perform controlled functional testing and verify that the fault does not recur.

10.	Document the physical failure mechanism and corrective action.

If the fault disappears after cable replacement but returns after several weeks, investigate routing, torsion, strain relief, and cable compatibility rather than assuming the replacement cable was defective.

## 18. Key Engineering Principles

Robot cables must be selected for their electrical function and mechanical environment.

Power, feedback, signal, communication, and safety-related circuits have different requirements.

Flex rating and torsion rating are separate considerations.

Cable damage may produce intermittent faults that occur only at particular robot positions.

Correct shielding and grounding depend on the actual system design.

Continuity testing alone cannot establish the long-term health of a moving cable.

Insulation testing must protect connected electronic devices.

Cable replacement must preserve routing, connector compatibility, strain relief, and movement limits.

Repeated cable failures require root-cause analysis of the complete motion path.

Post-maintenance testing must verify the affected axis, communication, I/O, and safety functions as applicable.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\18_PM\06_Robots\05_Teach_Pendant - Teach pendant