# Drive Fault: Parameters, Settings Changed or Lost

Study path: 02_PLAN_WISE_2026\02_INDUSTRY3_0\19_Breakdown\03_Drive_Fault\06_Param

## 1. Definition and Purpose

Drive parameter faults occur when a VFD or servo drive's configuration is incorrect, unintentionally changed, lost, corrupted, or no longer matches the installed motor and machine application.

A drive can be electrically healthy and show no hardware fault while the machine operates incorrectly because one or more parameters have changed.

Examples include:

Motor rated voltage or current entered incorrectly.

Maximum frequency changed.

Acceleration or deceleration time altered.

Start/stop command source changed.

Speed reference source changed.

Motor-control mode changed.

Electronic overload settings altered.

Communication address or protocol settings changed.

Encoder or feedback configuration changed.

Factory defaults restored accidentally.

Parameters not retained after power loss.

Replacement drive configured with incorrect values.

Core principle: Before changing parameters during a breakdown, record the current values and compare them with the approved configuration, backup, or known-good machine settings. Random parameter changes can turn one fault into several.

## 2. What Is a Drive Parameter?

A parameter is a configurable value that determines how the drive operates, communicates, protects the motor, or interacts with the machine.

Parameters may be organized into groups such as motor data, command control, speed control, protection, communication, feedback, application functions, and monitoring.

| Parameter group | Typical function | Example of incorrect configuration |
| --- | --- | --- |
| Motor data | Defines motor electrical characteristics | Rated current entered incorrectly |
| Command control | Selects start/stop source | Drive configured for keypad instead of PLC |
| Speed reference | Selects and scales speed command | Analog input selected instead of fieldbus |
| Frequency limits | Restricts operating range | Maximum frequency set too low |
| Acceleration/deceleration | Controls speed ramping | Acceleration time too short |
| Motor control | Defines control method and tuning | Incorrect motor-control mode |
| Protection | Defines overload and fault response | Motor protection set incorrectly |
| Communication | Defines network operation | Wrong address or device configuration |
| Feedback | Configures encoder or other feedback | Incorrect encoder resolution |
| Application logic | Defines special machine behavior | Preset-speed selection configured incorrectly |
| Restart behavior | Defines recovery after power or fault events | Unexpected restart behavior enabled |
| Safety-related configuration | Supports approved safety functions where applicable | Safety configuration inconsistent with the machine design |

Parameter numbers and names differ between manufacturers and product families. Always use the manual for the exact drive model and firmware version.

## 3. Why Parameters Change or Get Lost

### 3.1 Accidental parameter modification

A technician may change a parameter while troubleshooting a different fault.

Examples:

Increasing current limits to suppress repeated trips.

Changing the acceleration ramp.

Selecting a different command source.

Changing motor data while comparing another machine's configuration.

The machine may run afterward, but its original operating behavior and protection settings may no longer be preserved.

### 3.2 Factory reset

A factory reset restores some or all parameters to the manufacturer's default configuration, depending on the reset function.

Possible consequences include:

PLC commands no longer controlling the drive.

Motor data no longer matching the installed motor.

Speed references being ignored.

Communication becoming unavailable.

Motor identification or tuning requiring repetition.

Application-specific protection and operating behavior changing.

A factory reset is not a routine first-line troubleshooting action. It should be performed only when authorized and supported by a suitable backup and recovery plan.

### 3.3 Parameter backup not restored

A replacement drive may be installed without transferring the approved configuration.

Possible consequences include:

Wrong acceleration or deceleration behavior.

Incorrect motor current settings.

Different input assignments.

Communication errors.

Wrong speed limits.

Unexpected start/stop behavior.

A drive of the same model may still have different firmware, hardware options, or parameter defaults.

### 3.4 Memory or retention problem

Some drives store parameters in nonvolatile memory. If the configuration does not persist through a power cycle, investigate the supported storage mechanism and the manufacturer's diagnostics.

Possible causes include:

Parameter save operation not completed.

Incorrect save procedure.

Memory-related fault.

Configuration transfer not finalized.

Unsupported or incompatible parameter data.

Firmware-specific behavior.

Do not assume that every drive saves every parameter immediately when it is edited. Some require a save, store, transfer, or confirmation operation.

### 3.5 Drive replacement or firmware change

A drive replacement, firmware upgrade, or parameter migration may change how settings are represented or interpreted.

Potential problems include:

Different parameter numbering.

Changed parameter ranges.

Unsupported functions.

Different default values.

Changed communication configuration.

Incompatible backup format.

Always verify compatibility before transferring parameters between devices.

## 4. Symptoms of Incorrect Parameters

| Symptom | Possible parameter-related cause |
| --- | --- |
| Motor does not start | Incorrect command source, enable configuration, or start logic |
| Motor runs at reduced speed | Low maximum frequency or incorrect reference scaling |
| Motor accelerates too slowly | Acceleration time or active ramp selection |
| Motor trips during acceleration | Incorrect ramp, motor data, current limits, or application settings |
| Motor overheats | Incorrect motor parameters, protection settings, or excessive load |
| PLC cannot communicate with drive | Incorrect address, protocol, baud rate, device name, or network configuration |
| Drive runs from keypad but not PLC | Wrong command source or local/remote configuration |
| Motor runs in the wrong direction | Direction assignment or application configuration |
| Motor restarts unexpectedly | Restart or maintained-command configuration |
| Feedback-related alarm appears | Incorrect feedback type, resolution, scaling, or configuration |
| Drive operates differently after maintenance | Parameters changed, defaults restored, or backup not restored |
| Settings revert after power loss | Save, retention, storage, or memory-related issue |

These are possible causes, not proof. Confirm the actual parameter values and diagnostic evidence before making changes.

## 5. Parameter Backup and Recovery

A parameter backup preserves the drive's configuration for maintenance, replacement, and fault recovery.

The backup mechanism may be a manufacturer's software tool, keypad transfer, removable memory device, engineering project, or supported network service.

### 5.1 What a useful backup should contain

Drive manufacturer and exact model.

Hardware variant and firmware version.

Motor nameplate data.

Command-source configuration.

Speed-reference source and scaling.

Minimum and maximum operating limits.

Acceleration and deceleration settings.

Protection settings.

Digital and analog input/output assignments.

Communication settings.

Feedback configuration, where applicable.

Application-specific settings.

Relevant safety configuration and validation records, where supported.

Date and reason for the last approved change.

Machine or asset identification.

A screenshot of a few parameters is not a complete backup. Important settings may be stored across multiple groups, and some device-specific configuration may require a separate project or safety tool.

### 5.2 Backup procedure

1.	Identify the exact drive model and firmware version.

2.	Confirm that the machine is in a safe state for the intended procedure.

3.	Connect using the approved manufacturer tool or supported method.

4.	Read the current parameters from the drive.

5.	Save the complete supported configuration.

6.	Record the asset identity and backup date.

7.	Store the file in the approved maintenance location.

8.	Verify that the saved file can be opened and identified.

9.	Preserve the previous approved version when creating a new backup.

10.	Record the reason for any approved parameter change.

### 5.3 Before restoring a backup

Verify:

The backup belongs to the correct machine.

The drive model is compatible.

Firmware compatibility has been checked.

The installed motor matches the stored motor data.

Communication settings match the current installation.

Application-specific assignments are correct.

Safety-related settings are handled using the approved safety process.

The machine is isolated or otherwise controlled according to the approved restoration procedure.

Do not blindly restore one machine's parameters to another machine. Similar-looking equipment can have different motors, gear ratios, control wiring, feedback devices, or safety requirements.

## 6. Parameter Comparison: Current Versus Approved

A parameter comparison identifies differences between the live drive configuration and the approved configuration.

A useful comparison table is:

| Parameter | Approved value | Current value | Difference significance |
| --- | --- | --- | --- |
| Motor rated current | 12.5 A | 8.0 A | May affect motor model or protection behavior |
| Maximum frequency | 50 Hz | 30 Hz | Can restrict maximum operating speed |
| Acceleration time | 5 s | 20 s | Can slow machine acceleration |
| Command source | Fieldbus | Keypad | PLC may no longer control the drive |
| Reference source | Fieldbus | Analog input | Network reference may be ignored |
| Motor rated voltage | Per nameplate | Different value | Can affect control and motor operation |

These values are illustrative, not recommendations for a real motor.

### 6.1 How to compare effectively

Compare the exact parameter identifiers, not just descriptions.

Verify units and scaling.

Check whether the parameter is read-only or editable.

Identify settings that depend on other parameters.

Distinguish intentional changes from unexplained differences.

Check whether parameters are stored in the drive, PLC project, engineering software, or another configuration source.

Record any approved change before applying it.

A difference does not automatically mean a fault. Some parameters may legitimately vary with operating mode or application state.

## 7. Motor Parameters and Their Effect

Motor-related parameters help the drive model or control the connected motor.

Common entries include:

Rated motor voltage.

Rated motor current.

Rated frequency.

Rated speed.

Rated power.

Motor power factor, where required.

Motor-control mode.

Motor identification or tuning settings.

Motor thermal model or overload settings.

Motor pole count or related motor data, where applicable.

The exact required fields depend on the drive and control method.

### 7.1 Consequences of incorrect motor data

Incorrect motor thermal estimation.

Poor torque response.

Unexpected current-related trips.

Reduced speed-control accuracy.

Unsuitable motor-control behavior.

Motor operating outside its intended conditions.

### 7.2 Diagnostic method

1.	Read the motor nameplate.

2.	Compare it with the drive's configured motor data.

3.	Confirm the motor connection and rated voltage.

4.	Check whether the application uses sensorless vector, scalar V/f, closed-loop vector, or another control mode.

5.	Verify whether motor identification or tuning is required after a change.

6.	Apply only the approved values.

7.	Test under controlled conditions and monitor current, speed, and temperature.

Never enter guessed motor data simply to make the drive start.

## 8. Command and Reference Parameters

A drive may have separate parameters for run-command source and speed-reference source.

For example:

Start/stop source: fieldbus.

Speed reference source: analog input.

Direction source: digital input.

Reset source: terminal input.

This arrangement can be valid if intentionally designed. It becomes a fault when the actual configuration differs from the machine's intended architecture.

### 8.1 Troubleshooting

Check the configured command source.

Check the configured reference source.

Check local/remote mode.

Check digital input assignments.

Check analog input type and scaling.

Check fieldbus data mapping.

Check preset-speed and jog functions.

Check for reference limits and overrides.

If the PLC is sending a valid run command but the drive is configured for keypad control, changing the motor parameters will not solve the command-source problem. Parameters have distinct jobs, despite humanity's ongoing habit of changing random things.

## 9. Communication Parameters

Communication settings allow the PLC, HMI, engineering workstation, and drive to exchange information.

Depending on the network, settings may include:

IP address and subnet.

Device name.

Baud rate.

Node address.

Protocol selection.

Communication profile.

Telegram or assembly configuration.

Process-data mapping.

Timeout or watchdog behavior.

Network-specific options.

### 9.1 Common symptoms

Drive disappears from the PLC network.

PLC reports a device fault.

Reference values stop updating.

Drive shows a communication timeout.

Data appears in the wrong PLC tags.

A replacement drive fails to communicate despite correct wiring.

### 9.2 Diagnostic approach

1.	Check physical network connections and device status.

2.	Confirm the drive's actual network identity.

3.	Compare its configuration with the approved network design.

4.	Verify the PLC hardware or device configuration.

5.	Check process-data mapping and communication profile.

6.	Check the timeout behavior and relevant diagnostics.

7.	Restore the approved configuration only after confirming device compatibility.

Do not change network addresses casually on a running plant network. Duplicate addresses or incorrect device names can disrupt other equipment.

## 10. Parameter Changes During Breakdown Troubleshooting

A disciplined troubleshooting sequence separates diagnosis from modification.

### 10.1 Recommended procedure

Record Fault and Operating Conditions

↓

Read Current Parameter Values

↓

Compare with Approved Configuration

↓

Identify Relevant Differences

↓

Determine Root Cause

↓

Authorize Corrective Change

↓

Apply and Record Change

↓

Verify Machine Function and Protection

↓

Save Approved Configuration

### 10.2 Before changing a parameter

Record the original value.

Understand what the parameter controls.

Check dependencies and related settings.

Confirm the required range and units.

Check machine and motor limitations.

Confirm whether a restart or tuning procedure is required.

Ensure the change is authorized.

Plan how the result will be verified.

Define a rollback method.

### 10.3 After changing a parameter

Confirm the new value was accepted.

Save it using the manufacturer's procedure if required.

Verify the drive remains in the intended control mode.

Test start, stop, and reference behavior.

Check relevant protection and alarms.

Verify operation under representative load.

Update the parameter record and approved backup.

## 11. Factory Reset: Risks and Recovery

A factory reset may remove application-specific settings and restore defaults. The scope depends on the drive model and reset option.

Potential consequences include:

Loss of PLC control.

Loss of network communication.

Incorrect motor data.

Changed digital input assignments.

Different operating limits.

Changed fault-response behavior.

Loss of application configuration.

Need for motor identification or tuning.

Changed restart behavior.

### 11.1 If a factory reset has already occurred

1.	Keep the machine in a safe state.

2.	Record the current drive identity and visible diagnostic information.

3.	Identify the exact model and firmware.

4.	Retrieve the correct approved backup.

5.	Confirm compatibility with the installed hardware and motor.

6.	Restore the configuration using the approved procedure.

7.	Verify communication and command-source settings.

8.	Check the motor data, limits, and protection settings.

9.	Verify feedback and application-specific configuration.

10.	Test the machine according to the approved commissioning procedure.

11.	Save and document the recovered configuration.

If no reliable backup exists, rebuild the configuration from verified drawings, motor nameplate data, approved project files, and manufacturer documentation. Do not treat factory defaults as a substitute for the machine's original engineering design.

## 12. Parameter Retention and Memory Faults

When settings disappear after a power cycle, first determine whether the drive actually lost its parameters or whether the wrong configuration is being loaded or selected.

Possible explanations include:

Changes were not saved.

A configuration download overwrote the settings.

A PLC or engineering tool reapplied another parameter set.

The drive's storage mechanism reported a fault.

A memory or hardware problem exists.

The wrong device was accessed during the original change.

### 12.1 Diagnostic method

Record several affected parameters before and after the power cycle.

Check the manufacturer's save procedure.

Review available memory or parameter-storage diagnostics.

Check whether the PLC or engineering software writes parameters at startup.

Verify that the same physical drive is being monitored.

Review the change history and maintenance records.

Follow manufacturer instructions if a memory fault is indicated.

Do not repeatedly power-cycle a drive in an attempt to recover settings without understanding the fault and machine consequences.

## 13. Drive Replacement and Configuration Migration

Replacing a failed drive requires more than matching its power rating.

Verify:

Voltage and current ratings.

Motor compatibility.

Control mode.

Command and reference sources.

Input/output assignments.

Communication configuration.

Feedback interfaces.

Braking requirements.

Protection settings.

Firmware and hardware compatibility.

Safety-related configuration.

Mechanical application limits.

A replacement drive may have different parameter numbers, supported features, or defaults even within a product family.

Use the manufacturer's migration or conversion tools where available, then validate every critical configuration area.

## 14. Practical Breakdown Example: Motor Runs at Half Speed After Maintenance

Machine condition: A conveyor runs at approximately half its normal speed after a maintenance intervention.

Initial observations:

The drive has no active fault.

The PLC run command works.

The speed reference changes.

Maximum speed is lower than expected.

Investigation:

1.	Record the current operating frequency and reference.

2.	Read the drive's maximum frequency parameter.

3.	Compare it with the approved configuration.

4.	Verify analog or network reference scaling.

5.	Check whether a preset-speed selection is active.

6.	Review the maintenance change history.

Possible root cause: The maximum frequency parameter was reduced during troubleshooting and not restored.

Corrective action: Restore the approved operating limit after verifying motor, gearbox, machine, and process constraints.

Verification: Test the reference range and machine speed under controlled conditions, then record the corrected configuration.

## 15. Practical Breakdown Example: Drive No Longer Communicates with PLC

Machine condition: A VFD operates locally, but the PLC reports a communication fault after the drive was replaced.

Investigation:

1.	Confirm physical network connections.

2.	Identify the replacement drive's actual device identity.

3.	Compare network settings with the approved configuration.

4.	Verify the PLC hardware or device configuration.

5.	Check the process-data mapping and communication profile.

6.	Compare the drive's current settings with the approved backup.

7.	Restore the compatible network configuration and test communication.

Possible root cause: The replacement drive has a different network identity or communication configuration.

Corrective action: Apply the approved device and network settings after confirming address uniqueness and hardware compatibility.

Verification: Confirm stable communication, correct status feedback, valid control-word behavior, and correct reference transfer.

## 16. Practical Breakdown Example: Settings Revert After Power Loss

Machine condition: A technician changes a parameter, but the original value returns after the drive is power-cycled.

Investigation:

1.	Record the parameter and its value before modification.

2.	Check whether the drive requires a save or store operation.

3.	Follow the manufacturer procedure for saving parameters.

4.	Check whether PLC logic or engineering software downloads settings during startup.

5.	Review available storage diagnostics.

6.	Repeat the test under an approved maintenance procedure.

7.	Escalate to manufacturer diagnostics if the setting still fails to persist.

Possible root cause: The parameter edit was not committed to nonvolatile storage, or an external configuration process overwrites it during startup.

Corrective action: Correct the save procedure or external configuration behavior and investigate hardware storage faults if evidence supports them.

Verification: Confirm the parameter persists across the approved restart test and remains consistent with the approved configuration.

## 17. Parameter Management and Preventive Maintenance

### 17.1 Recommended controls

Maintain an approved baseline parameter backup for each drive.

Record the drive model, firmware, motor, and machine identity.

Store backups in a controlled location.

Restrict parameter modification to authorized personnel.

Record original and revised values.

Document the reason for each change.

Compare configurations after major maintenance.

Verify parameter retention where required.

Maintain replacement-drive configuration procedures.

Keep communication and feedback configuration records.

Preserve safety-related configuration and validation evidence through the approved process.

Periodically confirm that backup files are readable and correspond to the installed equipment.

### 17.2 Suggested parameter change log

| Date | Parameter | Original value | New value | Reason and verification |
| --- | --- | --- | --- | --- |
| YYYY-MM-DD | Maximum frequency | Approved value | Revised value | Authorized change and test result |
| YYYY-MM-DD | Reference source | Approved source | Revised source | Corrected source selection |
| YYYY-MM-DD | Acceleration time | Approved value | Revised value | Application test completed |

Use actual values and identifiers from the installed equipment. The example rows above are templates, not instructions to change those parameters.

## 18. Maintenance Record Template

Machine / Station:

Drive Asset Number:

Manufacturer and Model:

Hardware Variant:

Firmware Version:

Motor Identification:

Drive Fault Code:

Fault Time and Operating Condition:

Current Parameter Backup Available:

Approved Backup Version:

Current Configuration Read:

Parameter Differences Identified:

Original Parameter Values:

Corrective Changes:

Reason for Each Change:

Motor Data Verified:

Command Source Verified:

Reference Source Verified:

Frequency Limits Verified:

Protection Settings Verified:

Communication Settings Verified:

Feedback Settings Verified:

Safety Configuration Status:

Parameter Save Operation:

Power-Cycle Retention Check:

Functional Test Results:

Load Test Results:

Final Backup Location:

Technician / Work Order:

## 19. Common Troubleshooting Mistakes

Changing parameters before recording the original values.

Restoring a backup from the wrong machine.

Assuming drives with the same power rating have identical configuration.

Performing a factory reset without a recovery plan.

Entering motor data without checking the nameplate.

Increasing current limits to suppress repeated trips.

Changing acceleration ramps without evaluating the mechanical load.

Ignoring the difference between command-source and reference-source parameters.

Assuming parameter edits are automatically saved.

Ignoring PLC or engineering software that may overwrite drive settings.

Transferring parameters between incompatible firmware versions.

Changing network addresses without checking for conflicts.

Failing to validate safety-related configuration through the approved process.

Returning the machine to production without testing the changed behavior.

## 20. Interview Questions and Answers

### Q1. What is a drive parameter?

A configurable value that determines how the drive controls the motor, accepts commands, processes references, communicates, and applies protection.

### Q2. Why can a drive operate incorrectly without showing a fault?

The hardware may be healthy while its configuration is incorrect, such as a wrong reference source, low maximum frequency, or unsuitable motor data.

### Q3. What should be done before changing a parameter?

Record the current value, compare it with the approved configuration, understand the parameter's effect, check related settings, and obtain the required authorization.

### Q4. Why is a parameter backup important?

It provides a recoverable record of the intended configuration for maintenance, fault recovery, and drive replacement.

### Q5. Why should a backup not be restored blindly?

The backup may belong to a different motor, drive variant, firmware version, machine, communication network, or safety configuration.

### Q6. What causes parameters to revert after power loss?

Possible causes include an incomplete save operation, external configuration overwrite, storage faults, or a misunderstanding of how the parameter is retained.

### Q7. Why might a replacement drive fail to communicate with the PLC?

Its device identity, communication settings, firmware, communication profile, or process-data mapping may differ from the original drive configuration.

### Q8. What is the difference between a factory reset and parameter restoration?

A factory reset returns selected settings to manufacturer defaults. Parameter restoration loads a saved configuration that should correspond to the intended machine setup.

### Q9. Why is changing a current limit dangerous as a troubleshooting shortcut?

It may weaken the intended protection behavior or conceal an underlying electrical or mechanical fault. Any adjustment must follow the manufacturer's requirements and approved application design.

### Q10. How can a technician verify that a parameter correction worked?

Confirm the value is accepted and saved, compare the final configuration with the approved baseline, test the affected function, check relevant protection and alarms, and document the results.

## 21. Completion Checklist

Explain what drive parameters control.

Identify common causes of parameter changes or loss.

Recognize symptoms of incorrect configuration.

Compare current parameters with an approved baseline.

Understand motor, command, reference, limit, and protection parameters.

Diagnose communication-related configuration problems.

Explain backup, restore, and factory-reset risks.

Verify drive and firmware compatibility before migration.

Investigate settings that revert after power loss.

Record original values and authorized changes.

Verify the parameter save operation.

Validate machine operation and protection after changes.

Maintain a reliable configuration backup and change history.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\19_Breakdown\03_Drive_Fault\07_Feedback - Feedback: Encoder or feedback signal