# Robot Fault: Mastering, Lost or Incorrect Reference

Study path: 02_PLAN_WISE_2026\02_INDUSTRY3_0\19_Breakdown\04_Robot_Fault\05_Mastering

## 1. Definition of Robot Mastering

Robot mastering is the process of establishing or restoring the correct relationship between a robot joint's physical mechanical position and the position value interpreted by its controller.

Industrial robots use position feedback devices, commonly encoders or resolvers, to determine joint position. The controller uses calibration and mastering data to interpret these measurements as robot joint coordinates.

If mastering data is lost, incorrect, or mismatched with the robot's mechanical configuration, the controller may report a position that does not correspond to the actual physical position. The robot can then move to incorrect locations even when the program and motion commands have not changed.

Critical distinction: Mastering is not the same as homing, program-position teaching, or simply resetting a servo alarm.

## 2. Important Terminology

| Term | Meaning |
| --- | --- |
| Mastering | Establishing the correct joint-position reference relationship |
| Calibration | Establishing or correcting the relationship between measured positions and the robot's coordinate system |
| Encoder | Device measuring motor or joint position |
| Absolute encoder | Encoder that reports position within its defined measurement range without necessarily requiring a complete reference search after every power cycle |
| Incremental encoder | Encoder that measures movement from a reference and may require homing or referencing |
| Mastering mark | Manufacturer-defined mechanical reference mark used during a specified mastering procedure |
| Zero position | Defined reference position for a robot joint; not necessarily the physical end of travel |
| Home position | A programmed or configured position used for reference, initialization, or process purposes |
| Joint coordinates | Position values for individual robot axes, usually expressed in degrees |
| Cartesian coordinates | Tool or reference-frame position expressed using coordinates such as X, Y, Z and orientation |
| Calibration data | Data describing the relationship between the robot's measured joint positions and its defined geometry |
| Mastering data | Reference data used to interpret joint feedback correctly |
| Encoder battery | A battery used by certain systems to preserve position-related information when power is removed |
| Position validity | The controller's determination that position feedback and reference data are trustworthy |

Terminology differs across manufacturers. Some systems distinguish mastering, calibration, and zeroing very strictly; others use overlapping terms in their interfaces.

## 3. Why Mastering Is Important

Robot motion depends on the controller interpreting feedback correctly.

Physical Joint Position

↓

Encoder / Position Feedback

↓

Mastering and Calibration Data

↓

Controller Joint Coordinates

↓

Robot Kinematics

↓

Calculated Tool Position

↓

Programmed Motion

If the reference data is incorrect, the controller may calculate the wrong joint coordinates and consequently the wrong tool position.

Potential consequences include:

Tool center point (TCP) deviation.

Incorrect approach or withdrawal paths.

Fixture or workpiece collision.

Incorrect welding, dispensing, handling, or assembly position.

Unexpected axis movement during recovery.

Inaccurate repeatability relative to the intended programmed path.

Interference with surrounding equipment.

Production quality defects.

A robot can appear to move smoothly and still be incorrectly mastered. Smooth motion alone does not prove coordinate accuracy.

## 4. Common Reasons Mastering Is Lost or Incorrect

| Cause | What may happen | Diagnostic direction |
| --- | --- | --- |
| Encoder battery failure | Position data may be lost on supported systems | Check battery and position-validity diagnostics |
| Battery disconnected during maintenance | Retained position information may be lost | Review maintenance history and controller alarms |
| Encoder replacement | New encoder reference may not match the existing calibration | Follow the manufacturer's replacement and mastering procedure |
| Motor replacement | Feedback reference or motor-to-axis relationship may change | Verify motor compatibility and mastering requirements |
| Servo motor removal | Mechanical coupling or reference relationship may be disturbed | Check installation and specified reference procedure |
| Gearbox or joint replacement | Mechanical relationship may change | Perform the specified calibration or mastering process |
| Robot collision | Mechanical damage or coupling movement may alter accuracy | Inspect for damage and compare reference measurements |
| Incorrect mastering procedure | Reference values may be set incorrectly | Review the exact procedure and recorded values |
| Wrong backup restored | Position data may not match the installed robot or hardware | Verify robot identity and backup compatibility |
| Controller or encoder fault | Position may become invalid or inconsistent | Check encoder diagnostics and controller alarms |
| Mechanical looseness | Actual motion may not correspond accurately to measured joint position | Inspect gearbox, coupling, and mechanical transmission |
| Incorrect configuration | Wrong encoder or joint configuration may be interpreted | Verify approved controller and hardware configuration |

Important: A collision does not automatically mean mastering is lost. It may cause mechanical damage, change a coupling relationship, or leave the mastering data intact. Diagnose before performing a mastering operation.

## 5. Symptoms of Incorrect Mastering

### 5.1 Position deviation

The robot reaches a programmed position, but the tool does not align with the expected physical location.

Possible observations:

TCP position differs from a verified reference.

Welding or dispensing points shift.

The robot approaches a fixture at the wrong angle.

The robot moves correctly in one region but shows unacceptable error in another.

### 5.2 Position mismatch after power-up

The controller reports invalid or unreferenced joint positions after power is restored.

Possible causes include:

Encoder position loss.

Battery-related issues on supported systems.

Encoder communication failure.

Missing or incompatible mastering data.

Controller configuration mismatch.

### 5.3 Position error after component replacement

A motor, encoder, gearbox, or joint component was replaced, after which the robot no longer reaches its previous physical positions.

Investigate whether the replacement procedure required:

Mechanical alignment.

Encoder reference transfer.

Mastering.

Calibration.

Configuration restoration.

Tool or frame verification.

### 5.4 Position error varies by robot pose

The robot reaches one reference point correctly but deviates at other locations.

This may indicate:

Incorrect mastering of one or more axes.

Mechanical backlash or looseness.

Joint or gearbox damage.

Incorrect kinematic or calibration data.

Tool or workpiece coordinate errors.

A single correct TCP position does not prove that every joint is correctly mastered.

## 6. Mastering Versus Homing Versus Teaching

These processes solve different problems.

| Operation | Purpose | What it does not necessarily do |
| --- | --- | --- |
| Mastering | Establishes the correct relationship between joint feedback and mechanical reference | Does not automatically correct a wrongly taught program point |
| Homing | Moves or references an axis using a defined home procedure | Does not necessarily restore the robot's factory mastering |
| Teaching | Records or modifies program positions | Does not repair incorrect encoder-to-joint reference data |
| TCP calibration | Determines the tool center point relative to the tool flange | Does not automatically correct joint mastering |
| User-frame calibration | Establishes a workpiece or user coordinate frame | Does not repair a mechanical or encoder reference fault |
| Servo reset | Clears a fault when the underlying condition permits | Does not restore lost mastering data |

Confusing these procedures can create additional errors. If a robot is incorrectly mastered, reteaching program points may conceal the symptom without correcting the underlying reference problem.

## 7. Encoder Batteries and Position Retention

Some industrial robot systems use battery-backed encoders or controller-supported retention mechanisms to preserve position information while the system is switched off.

Not every robot uses the same design. Some modern systems use absolute encoders or other retention arrangements and may not have the same battery requirements.

### 7.1 Battery-related failure mechanism

Robot Power Removed

↓

Position Retention Requirement

↓

Battery / Retention System Fails

↓

Position Data May Become Invalid

↓

Controller Reports Position or Encoder Alarm

↓

Manufacturer-Specified Recovery Required

### 7.2 Maintenance precautions

Follow the robot manufacturer's battery replacement procedure.

Confirm whether the battery must be replaced with controller power on or whether another approved retention method is specified.

Do not assume a battery can be replaced safely with the controller powered down.

Record the alarm before performing maintenance.

Check battery condition and service-life requirements.

Confirm position validity after replacement.

Do not perform mastering merely because a battery alarm occurred. First determine whether reference data was actually lost.

A low-battery warning and actual loss of mastering are not equivalent conditions.

## 8. Mastering Marks and Mechanical Reference

Many industrial robot manufacturers provide physical mastering marks, gauges, fixtures, or dedicated measurement procedures.

These references may be located on the robot axes, motor assemblies, or manufacturer-specified calibration features.

### 8.1 Why mechanical reference matters

The controller's encoder reading is meaningful only when interpreted using the correct reference relationship. A mastering procedure establishes that relationship according to the robot's design.

If the physical reference is incorrect, a controller may display plausible numerical values while the actual mechanical position is wrong.

### 8.2 Precautions

Identify the exact robot model and controller generation.

Use the specified mastering method for that robot.

Confirm that the correct axis and reference marks are being used.

Inspect for mechanical damage before relying on reference marks.

Use manufacturer-approved gauges or fixtures when required.

Follow the specified axis positioning and safety procedure.

Record the original mastering values and the reason for the work.

Do not assume that all robot axes use the same marks, reference positions, or mastering sequence.

## 9. Mastering Methods

The supported method depends on the robot manufacturer, encoder design, controller generation, and maintenance history.

| Method | General principle | Important limitation |
| --- | --- | --- |
| Witness-mark mastering | Aligns specified mechanical marks | Accuracy depends on the design and prescribed alignment method |
| Single-axis mastering | Establishes the reference for a particular axis | Requires correct identification and manufacturer procedure |
| Encoder-reference recovery | Restores a supported reference relationship using retained or recorded data | Requires valid, compatible reference data |
| Mastering fixture or gauge | Uses a dedicated mechanical or measurement fixture | Requires the correct tool and procedure |
| Absolute-reference method | Uses supported absolute-position information | Absolute feedback does not eliminate every calibration or validity requirement |
| Full calibration | Uses specified measurement methods to establish robot geometry or reference accuracy | May require specialist tools, procedures, and verification |

A mastering method should never be selected solely because it is the fastest available option. The correct procedure is determined by the robot's design and the confirmed failure mechanism.

## 10. Safe Diagnostic Procedure

### Step 1: Record the fault

Before changing parameters or resetting data, record:

Robot identification and controller model.

Affected axis or axes.

Exact alarm code and message.

Joint position values.

Position-validity status.

Recent battery, motor, encoder, gearbox, or controller work.

Recent collision or abnormal mechanical event.

Whether the fault appeared after power-up or during movement.

### Step 2: Make the equipment safe

Follow the site's robot safety and lockout/tagout procedure.

Prevent unexpected motion.

Consider gravity-loaded axes and stored mechanical energy.

Use approved mechanical supports where required.

Do not enter the robot's hazardous space based solely on a displayed servo-off state.

Follow the manufacturer's procedure for any inspection requiring power.

### Step 3: Check position validity

Use the controller's supported diagnostics to determine whether joint positions are valid.

Distinguish among:

Position data invalid.

Encoder communication fault.

Battery or retention warning.

Incorrect mastering data.

Mechanical accuracy error despite valid position feedback.

These conditions may require different corrective actions.

### Step 4: Review maintenance history

Check whether the problem started after:

Battery replacement.

Motor or encoder replacement.

Gearbox work.

Robot collision.

Controller replacement.

Backup restoration.

Parameter changes.

Cable or connector maintenance.

The timing of the fault can significantly narrow the investigation.

### Step 5: Inspect feedback and mechanics

Where indicated by diagnostics, inspect:

Encoder connectors and cables.

Encoder supply and communication status.

Motor mounting and mechanical coupling.

Gearbox and joint condition.

Signs of collision or deformation.

Relevant calibration or mastering records.

Do not assume a mastering problem when the actual cause is a defective encoder or damaged mechanical transmission.

### Step 6: Decide whether mastering is required

Mastering is appropriate only when the evidence and manufacturer procedure establish that the reference relationship is missing or incorrect.

If the controller has valid position data and the actual position is inaccurate, investigate mechanical damage, calibration, TCP, workpiece frame, and program-position errors before changing mastering data.

## 11. Mastering Recovery Procedure

Mastering recovery is robot-specific. The following is a general framework, not a substitute for the exact manufacturer procedure.

1.	Identify the robot model, controller, axis, and exact fault.

2.	Confirm the mechanical condition and investigate collision damage.

3.	Determine why mastering or position validity was lost.

4.	Retrieve approved records and compatible backup data.

5.	Confirm the correct mastering method for the installed hardware.

6.	Secure the equipment and follow the specified safety procedure.

7.	Perform the authorized reference or mastering procedure.

8.	Verify the controller's position-validity status.

9.	Confirm that the displayed joint positions correspond to the required reference.

10.	Perform the prescribed calibration and accuracy checks.

11.	Verify TCP and workpiece frames where applicable.

12.	Test the robot using the approved controlled-motion procedure.

13.	Record the final values, procedure, and verification results.

### 11.1 Why backup data must be checked

A backup may contain useful mastering information, but it must belong to the correct robot and match the relevant hardware and configuration.

Restoring unrelated or incompatible data may create additional position errors. Preserve the original data before making changes and use only an approved recovery path.

## 12. Distinguishing Mastering Faults from Other Position Errors

| Observation | Possible cause | What to investigate |
| --- | --- | --- |
| All program positions shift after encoder-reference loss | Mastering or position-validity problem | Controller position diagnostics and mastering records |
| Only one tool is consistently offset | TCP or tool configuration | Tool calibration and tool data |
| All positions are shifted relative to a fixture | User frame or workpiece setup | Frame calibration and fixture location |
| Error varies with robot pose | Mastering, calibration, or mechanical problem | Reference checks across multiple poses |
| Position changes unpredictably | Feedback, mechanical looseness, or encoder issue | Feedback diagnostics and joint transmission |
| Error began after gearbox replacement | Mechanical reference or calibration mismatch | Installation and mastering requirements |
| Error occurs only in one program point | Teaching or program-data issue | Program position and motion configuration |
| Position invalid alarm appears after power loss | Retention or encoder issue | Battery, encoder design, and position-validity status |
| Robot moves smoothly but TCP accuracy is poor | Mastering, calibration, TCP, or frame issue | Independent reference and calibration checks |

## 13. Mastering and Mechanical Accuracy

Correct mastering is necessary for accurate robot coordinates, but it does not guarantee complete mechanical accuracy.

Actual accuracy may also depend on:

Robot geometry and calibration.

Gearbox backlash.

Structural deflection.

Payload.

Tool mounting.

Temperature.

Mechanical wear.

TCP and user-frame accuracy.

Measurement method.

Repeatability and absolute accuracy are different. A robot may repeatedly return to the same position while consistently missing the intended physical target.

A mastering problem often affects the relationship between reported joint coordinates and physical joint position. A mechanical accuracy problem can persist even when the reference data is valid.

## 14. Practical Breakdown Example: Position Shift After Battery Failure

Reported condition: After a planned shutdown, the robot reports a position-related alarm. Once the controller is restarted, its position data is not valid.

Investigation:

1.	Record the exact encoder or position alarm.

2.	Review the battery and maintenance history.

3.	Determine whether the robot uses battery-backed position retention.

4.	Check controller diagnostics and position-validity status.

5.	Follow the approved retention and recovery procedure.

6.	Establish whether the mastering data remains valid or must be restored.

Possible root cause: Position retention was lost in a system that relies on battery-backed encoder data.

Corrective action: Repair the retention problem and perform the manufacturer-specified recovery procedure. If mastering has been lost, restore the reference using the approved method.

Verification:

Confirm position validity.

Verify the required reference positions.

Check TCP and workpiece coordinates where necessary.

Perform controlled-motion tests.

Record the final mastering status and maintenance action.

## 15. Practical Breakdown Example: Position Error After Gearbox Replacement

Reported condition: A robot joint reaches its programmed position, but the tool is consistently offset from the expected location after a gearbox replacement.

Investigation:

1.	Review the replacement work order.

2.	Confirm the gearbox part number and installation orientation.

3.	Check whether the manufacturer's procedure requires mastering or calibration after replacement.

4.	Inspect mechanical alignment and coupling integrity.

5.	Verify the joint reference and controller position data.

6.	Check TCP and workpiece frames if the mechanical reference is verified.

Possible root cause: The replacement changed the joint's mechanical reference relationship, and the required mastering or calibration was not completed.

Corrective action: Correct the installation or perform the specified mastering and calibration procedure.

Verification: Confirm the affected joint reference, test representative poses, and verify tool position using the approved measurement method.

## 16. Practical Breakdown Example: Incorrect TCP After Maintenance

Reported condition: The robot reaches the same joint coordinates as before maintenance, but a welding torch or gripper is incorrectly positioned relative to the workpiece.

Investigation:

1.	Confirm joint-position validity and mastering status.

2.	Check whether the tool was removed, replaced, or remounted.

3.	Verify the active TCP and tool-data selection.

4.	Check workpiece frame and fixture references.

5.	Inspect tool mounting and mechanical alignment.

6.	Recalibrate the tool if required.

Possible root cause: The tool mounting changed, invalidating the previous TCP data.

Corrective action: Restore the mechanical mounting and perform the required TCP calibration.

Verification: Confirm tool geometry and test representative program positions under the approved procedure.

This example demonstrates why not every robot position error is a mastering fault.

## 17. Preventive Maintenance

Monitor encoder battery and retention-system warnings.

Replace batteries according to the manufacturer's service procedure.

Maintain approved mastering and calibration records.

Back up compatible robot configuration data.

Record mastering values before relevant maintenance.

Follow prescribed procedures after motor, encoder, or gearbox replacement.

Inspect the robot after collisions.

Check mechanical looseness and abnormal joint behavior.

Maintain encoder cable and connector condition.

Verify position validity after applicable maintenance.

Perform TCP and frame checks when tooling or fixtures change.

Maintain records of accuracy verification.

Restrict mastering changes to trained, authorized personnel.

## 18. Mastering Maintenance Record Template

Robot ID:

Robot Model:

Controller Model:

Affected Axis:

Robot Serial Number:

Reported Alarm:

Exact Alarm Code:

Position Validity:

Reported Joint Position:

Expected Reference:

Observed Physical Position:

Recent Maintenance:

Battery / Retention Status:

Encoder Status:

Mechanical Condition:

Collision History:

Backup Identification:

Backup Compatibility Verified:

Mastering Method:

Manufacturer Procedure:

Mastering Data Before:

Mastering Data After:

Calibration Required:

TCP Verification:

User-Frame Verification:

Accuracy Test Result:

Controlled-Motion Test Result:

Root Cause:

Corrective Action:

Parts Replaced:

Final Position Validity:

Final Machine Status:

Technician:

Work Order:

Date:

## 19. Interview Questions and Answers

### Q1. What is robot mastering?

It establishes the correct relationship between the robot's physical joint reference and the joint-position values interpreted by the controller.

### Q2. What causes mastering loss?

Possible causes include position-retention failure, encoder replacement, mechanical work, incorrect recovery procedures, incompatible data restoration, or feedback-system faults.

### Q3. What is the difference between mastering and homing?

Mastering establishes the correct joint reference relationship. Homing moves or references an axis using a defined home procedure and does not necessarily restore mastering.

### Q4. Does an encoder battery alarm always mean mastering is lost?

No. The alarm may indicate a low battery before position information is lost. The actual condition must be confirmed using controller diagnostics and the manufacturer's procedure.

### Q5. Can a robot move normally with incorrect mastering?

Yes. The servo system may control motion smoothly while the relationship between physical joint position and interpreted coordinates is incorrect.

### Q6. What is the difference between mastering and TCP calibration?

Mastering concerns the relationship between joint feedback and mechanical reference. TCP calibration determines the tool center point relative to the robot flange.

### Q7. Why must mastering data backups be verified?

The data must correspond to the correct robot, controller configuration, and relevant hardware. Incompatible data can introduce additional position errors.

### Q8. Should all robot position errors be corrected by mastering?

No. The cause may be TCP data, workpiece frames, program positions, mechanical damage, feedback faults, or calibration errors.

### Q9. What must be verified after mastering?

Position validity, joint reference, required calibration, TCP and frame data where applicable, safety conditions, and controlled-motion accuracy.

### Q10. Why is mastering important after gearbox or motor replacement?

The mechanical and feedback reference relationship may have changed. The replacement procedure determines whether mastering, calibration, or another reference operation is required.

## 20. Completion Checklist

Understand robot mastering and joint-reference interpretation.

Distinguish mastering, homing, teaching, and calibration.

Identify encoder retention and battery-related issues.

Recognize symptoms of invalid or incorrect mastering.

Distinguish reference faults from mechanical and TCP errors.

Understand mastering requirements after motor or gearbox replacement.

Follow safe, manufacturer-approved recovery procedures.

Verify position validity and physical accuracy after recovery.

Maintain compatible backups and mastering records.

Document the confirmed root cause and corrective action.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\19_Breakdown\04_Robot_Fault\06_Program - Program: Position and logic errors