# Corrective and Preventive Action (CAPA)

## 1. Definition of CAPA

CAPA stands for Corrective and Preventive Action. It is a structured management process used to investigate problems, eliminate or control their causes, reduce the risk of recurrence, and prevent similar problems from occurring elsewhere.

In industrial maintenance, CAPA connects breakdown investigation, root cause analysis (RCA), corrective action reports, equipment reliability, quality management, safety, and continual improvement.

For example, if several machines experience repeated remote I/O communication failures, replacing individual network adapters may restore operation temporarily. A CAPA investigation examines whether the failures share a common cause, such as unstable device power, a faulty network design, or an inadequate installation standard, and implements controls to address the broader risk.

CAPA is not simply a list of repairs. It is a controlled process with defined responsibilities, evidence, implementation requirements, and effectiveness verification.

## 2. Objectives of CAPA

CAPA helps an organization to:

Correct detected failures and nonconformities.

Identify and address verified root causes.

Prevent recurrence of known problems.

Identify similar risks on other machines or production lines.

Reduce breakdown frequency and unplanned downtime.

Improve machine availability and reliability.

Prevent quality defects caused by equipment malfunction.

Improve safety and maintenance processes.

Track corrective actions through completion and effectiveness review.

Standardize successful improvements across the plant.

Maintain auditable records of decisions, changes, and results.

Identify ineffective actions and reopen investigations when necessary.

## 3. Correction, Corrective Action, Preventive Action, and CAPA

These terms describe different activities.

| Term | Meaning | Industrial example |
| --- | --- | --- |
| Correction | Fixes the detected problem | Replace a failed proximity sensor |
| Containment | Limits immediate impact while investigation continues | Inspect similar machines for the same defect |
| Corrective action | Addresses the cause of an existing problem to prevent recurrence | Correct the cable-routing design that damaged the sensor wiring |
| Preventive action | Addresses a potential problem before it occurs, using risk assessment and available evidence | Assess other machines using the same cable-routing design and improve them before failure |
| CAPA | Manages the investigation, actions, verification, and effectiveness review through a controlled process | Record the failure, investigate the cause, assign actions, verify implementation, and monitor recurrence |

Terminology varies by management system. Some frameworks treat preventive action as part of risk-based planning and continual improvement rather than as a separate formal workflow.

## 4. Why CAPA Is Important in Industrial Maintenance

Industrial equipment failures often have causes that extend beyond the component that initially failed.

A failed sensor may result from poor mounting. Repeated drive trips may result from excessive mechanical load. Repeated communication faults may result from unstable power or a common network weakness.

Without CAPA, maintenance may repeatedly replace components, reset controllers, and restore production while the underlying problem remains.

A functioning CAPA process changes the objective from restoring the machine repeatedly to reducing the likelihood of the failure recurring.

CAPA is especially useful for:

Repeated breakdowns.

High-impact failures.

Safety-related findings.

Quality defects.

Repeated component failures.

Failed previous corrective actions.

Common problems affecting multiple machines.

Maintenance procedure weaknesses.

Obsolete equipment and recurring supportability problems.

Audit findings and documented nonconformities.

## 5. CAPA Workflow

A typical CAPA workflow follows this sequence.

Problem, failure, or risk identified

↓

Assess impact and determine CAPA requirement

↓

Contain the problem and protect people, equipment, and product

↓

Define the problem and collect evidence

↓

Perform RCA or an appropriate risk assessment

↓

Identify corrective and preventive measures

↓

Assign owners, due dates, and acceptance criteria

↓

Implement actions under controlled procedures

↓

Verify implementation and functional performance

↓

Evaluate effectiveness over suitable operating exposure

↓

Close, or reopen and revise ineffective actions

↓

Update standards and share lessons learned

The organization should define when a formal CAPA is mandatory and who has the authority to approve, extend, reopen, or close it.

## 6. When Should CAPA Be Initiated?

CAPA should be initiated when the problem's severity, recurrence, risk, or systemic significance justifies a structured response.

Typical triggers include:

A recurring breakdown despite previous repairs.

A major production interruption.

A safety incident or significant hazard finding.

Repeated product-quality defects.

Multiple machines showing the same failure mode.

An audit nonconformity.

A failed corrective action.

A component defect affecting a fleet of machines.

A recurring weakness in PM, inspection, or repair procedures.

A design defect affecting equipment reliability.

An obsolete component creating significant operational risk.

A supplier or vendor repair problem recurring across equipment.

Evidence that current controls cannot adequately prevent or detect a failure.

Not every routine breakdown needs a formal CAPA. Use approved escalation criteria based on risk, impact, recurrence, and the possibility of a broader problem.

## 7. Reactive CAPA and Proactive Prevention

CAPA can address both existing failures and risks identified before a failure occurs.

| Aspect | Reactive CAPA | Proactive prevention |
| --- | --- | --- |
| Starting point | A failure, defect, or nonconformity has occurred | A potential failure or weakness is identified |
| Main objective | Eliminate or control the cause of the existing problem | Reduce the likelihood or consequence of a potential problem |
| Evidence | Breakdown records, measurements, alarms, inspection results | Risk assessments, trends, design reviews, similar-equipment findings |
| Example | Correct the confirmed cause of repeated sensor failure | Inspect similar installations before their sensor cables fail |
| Verification | Check whether the original failure recurs | Check whether the identified risk has been reduced and controls are effective |

A proactive improvement may be managed through CAPA or another risk-management process, depending on the organization's procedures.

## 8. CAPA Identification and Classification

Each CAPA should have a unique identifier and an approved classification.

### Suggested CAPA categories

Electrical.

Automation and control systems.

Mechanical.

CNC and robotics.

Industrial networks.

Quality.

Safety.

Maintenance process.

Supplier or component quality.

Obsolescence and lifecycle risk.

Documentation and training.

Environmental or energy performance.

### Suggested priority levels

| Priority | General interpretation | Example |
| --- | --- | --- |
| Critical | Immediate control and escalation required | A serious safety hazard or a critical protective function failure |
| High | Major operational, quality, or reliability risk | Repeated failure of a critical production machine |
| Medium | Significant recurring issue requiring planned action | Repeated noncritical equipment fault causing measurable downtime |
| Low | Limited impact with an improvement opportunity | A documentation gap with low immediate operational risk |

These are illustrative categories. Use the site's approved risk matrix and escalation rules.

Priority should reflect consequence and risk, not just how easy the issue is to repair.

## 9. CAPA Problem Statement

A CAPA problem statement must define the issue clearly enough to support investigation and verification.

Include:

Equipment or process affected.

Observed failure or nonconformity.

Date, time, and operating conditions where relevant.

Frequency or recurrence.

Operational, quality, safety, or financial impact.

Applicable requirement that was not met.

Evidence supporting the observation.

### Weak statement

"Drive problems occur repeatedly."

### Better statement

"Conveyor CV-08 experienced five VFD overcurrent trips during startup over three production shifts, causing a combined 52 minutes of downtime. The trips occurred during conveyor acceleration and were recorded in the drive fault history."

The second statement defines the observed problem and measurable impact without assuming the cause.

## 10. Containment and Immediate Risk Control

Containment protects the operation while investigation and permanent actions are pending.

Examples include:

Safely stopping affected equipment.

Isolating a hazardous subsystem.

Preventing suspect product from progressing.

Inspecting equivalent machines for the same failure mode.

Applying an approved temporary operating restriction.

Replacing a confirmed defective component to restore function.

Preserving diagnostic buffers and fault histories.

Informing production, quality, safety, and engineering teams.

Establishing additional inspections until permanent action is completed.

Temporary measures must be documented, assigned an owner, and given a review or expiry date where appropriate.

A temporary measure must not bypass required safety functions or become a permanent workaround merely because production can continue.

## 11. Root Cause Analysis Within CAPA

RCA provides the evidence-based explanation needed to choose suitable actions.

CAPA should not prescribe a permanent solution before the cause has been sufficiently investigated.

Consider a recurring robot servo fault.

Possible causes include:

Motor or drive defect.

Encoder feedback interruption.

Damaged cable.

Mechanical binding.

Power-supply instability.

Incorrect configuration.

A common installation weakness.

The investigation should use robot diagnostics, servo history, electrical measurements, mechanical inspection, and event timing to distinguish these possibilities.

The confirmed cause determines the appropriate action. Replacing a servo drive without evidence may waste resources and leave the actual problem unresolved.

## 12. Corrective Action Planning

Every CAPA action should be specific, assigned, and verifiable.

A useful action record includes:

Action ID.

Description of the required change.

Cause or risk addressed.

Responsible owner.

Due date.

Required resources.

Required approvals.

Safety and operational controls.

Acceptance criteria.

Implementation evidence.

Verification owner.

Effectiveness review method.

### Example action plan

| Identified cause or risk | Required action | Acceptance criteria | Evidence |
| --- | --- | --- | --- |
| Sensor cable damaged by repeated motion | Correct routing and mechanical protection | Full motion range completed without interference; bend radius meets specification | Inspection and functional test |
| Similar machines use the same routing design | Assess equivalent installations | Every identified at-risk installation assessed and required actions completed or risk-controlled | Inspection register |
| PM does not address the failure mechanism | Update the relevant PM inspection | Approved task released with defined inspection criteria | Revised PM and release record |
| Replacement cable specification is unclear | Standardize the approved cable specification and part identification | Engineering approval and correct material identification verified | Approved drawing and material master |

The acceptance criteria should describe the required result rather than merely state that the task is finished.

## 13. Engineering Controls vs Administrative Controls

Corrective actions differ in how strongly they control a failure mechanism.

### Engineering controls

Examples include:

Redesigning cable routing.

Adding suitable mechanical protection.

Improving enclosure sealing.

Correcting a deficient electrical supply.

Replacing an unreliable design with a verified compatible solution.

Improving diagnostic coverage.

Removing a mechanical interference point.

### Administrative controls

Examples include:

Revising PM instructions.

Improving SOPs.

Adding inspection checklists.

Training technicians.

Revising approval workflows.

Improving spare-parts identification.

Adding periodic reviews.

Administrative controls can be necessary, but they may depend on people consistently following procedures. When a feasible engineering measure directly addresses the cause, it is often more robust than relying only on reminders or additional checks.

The action selection must be justified by evidence, risk, feasibility, and the site's engineering standards.

## 14. Preventive Action Across Similar Equipment

One of the most important CAPA activities is checking whether the same weakness exists elsewhere.

Suppose a cable-routing defect is confirmed on one robot. Several other robots may use the same installation design.

A preventive review should identify the affected population, inspect relevant installations, assess the risk, and define the required changes.

Confirmed failure or design weakness

↓

Identify equipment sharing the same design or condition

↓

Assess similarity and exposure

↓

Inspect or evaluate affected equipment

↓

Classify findings by risk

↓

Implement required actions

↓

Verify the affected population has been addressed

Do not assume that all similar machines have the same defect. Verify the relevant design, installation, operating conditions, and inspection results.

Record both the number of machines assessed and the disposition of each machine.

## 15. CAPA Implementation Control

Some actions affect critical production equipment, PLC programs, CNC configurations, robot motion, safety functions, or network architecture.

Such changes require controlled implementation.

Relevant controls may include:

Engineering approval.

Change request or management-of-change process.

Backup of existing software and configurations.

Revision control.

Compatibility assessment.

Risk assessment.

Test plan and acceptance criteria.

Approved maintenance window.

Rollback or recovery plan.

Post-change verification.

Updated drawings and technical documentation.

Operator and technician communication.

For example, changing a PLC timer may alter the response of a machine sequence. The change must be justified by the verified failure mechanism and tested against the required process and safety behavior.

CAPA must not become an informal route for modifying production systems without the required technical controls.

## 16. Implementation Verification and Effectiveness Verification

These are two different stages.

Implementation verification confirms that the action was completed correctly.

Effectiveness verification confirms that the action achieved its intended result.

| Verification stage | Example |
| --- | --- |
| Action completion | New cable carrier installed |
| Technical verification | Cable clearance and bend radius checked throughout the full motion range |
| Functional verification | Robot completes the specified motion test without the original fault |
| Effectiveness review | Alarm history and maintenance records reviewed after the agreed operating exposure |
| Standardization check | Revised drawings and PM tasks released and available |

A successful installation test is necessary but may not be sufficient to prove long-term effectiveness.

## 17. CAPA Effectiveness Criteria

Effectiveness criteria should be defined before the review wherever practical.

Examples include:

No recurrence of the same verified failure mode over a defined operating period.

Completion of a specified number of production cycles.

Equipment parameters remain within approved limits.

No recurrence of the relevant fault code under the original failure conditions.

Successful functional testing.

Completion and verification of actions on similar equipment.

A measurable reduction in failure rate over a defined exposure.

Improved compliance with an approved inspection or maintenance standard.

### Example

A corrective action addresses a cable failure that previously occurred approximately every few weeks.

A brief startup test is not sufficient to establish effectiveness. The verification plan should combine the relevant motion test, physical inspection, operating exposure, and review of recurrence evidence.

If the observation period is too short to evaluate the failure mode reliably, the result should remain provisional or pending according to the site's procedure.

## 18. CAPA Closure and Reopening

A CAPA should be closed only when its required closure conditions are satisfied.

Typical closure requirements include:

Problem and impact documented.

Immediate risk controlled.

Cause established to the required level.

Required actions implemented.

Technical verification passed.

Implementation evidence recorded.

Effectiveness criteria evaluated.

Similar-equipment risks addressed where applicable.

Required documentation updated.

Approval obtained from the authorized reviewer.

Some organizations allow implementation completion to be recorded before the final effectiveness review. In that case, the record must clearly show that effectiveness monitoring remains open.

### When should CAPA be reopened?

Reopen or initiate a linked investigation when:

The same verified failure recurs.

Effectiveness criteria are not met.

New evidence disproves the original cause.

A corrective action was not implemented as intended.

A similar failure reveals that the risk was broader than expected.

The implemented action creates an unacceptable secondary problem.

Do not treat the absence of a new complaint as proof of success. Check actual operating exposure and relevant diagnostic records.

## 19. CAPA Example: Repeated VFD Overcurrent

### Problem

A conveyor experiences repeated VFD overcurrent trips during startup.

### Containment

Stop and isolate the equipment as required.

Preserve the drive fault history.

Inspect the conveyor and motor circuit.

Restore operation only after the relevant checks and approvals.

### RCA finding

The investigation establishes excessive mechanical resistance caused by a binding conveyor roller. The drive trips because the startup load exceeds the expected operating condition.

This is an illustrative case. In actual maintenance work, the cause must be demonstrated using suitable inspection and diagnostic evidence.

### Corrective action

Repair or replace the defective roller.

Verify mechanical movement and load.

Test conveyor startup under approved conditions.

### Preventive action

Inspect equivalent rollers if they share the same design or operating exposure.

Review whether the PM process adequately identifies binding or abnormal resistance.

Update the PM task if a genuine inspection gap is confirmed.

### Effectiveness verification

Required startup and operating tests pass.

Drive current and diagnostics remain within applicable limits.

The same failure does not recur during the defined operating exposure.

Required PM changes and similar-equipment actions are verified.

## 20. CAPA Example: Repeated PLC Remote I/O Failures

### Problem

Several machines experience intermittent remote I/O communication faults.

### Evidence

PLC diagnostic buffers.

Remote I/O event history.

Network switch diagnostics.

Device power-supply measurements.

Cable and connector inspection.

Fault timestamps and equipment topology.

### RCA finding

The investigation confirms that several affected devices share an unstable power supply connection. The finding is supported by measured supply behavior and event correlation.

### Corrective actions

Repair the confirmed supply connection.

Inspect the associated terminals and conductors.

Verify supply stability under the relevant load.

Inspect other equipment using the same connection method.

Update installation or inspection standards if the investigation identifies a systemic weakness.

### Effectiveness verification

Supply voltage remains within specification under the required conditions.

Communication remains stable.

The relevant machine sequences complete successfully.

Diagnostic history is reviewed over the defined operating period.

## 21. CAPA Example: Repeated CNC Axis Feedback Fault

### Problem

A CNC machine experiences recurring axis feedback alarms.

### RCA finding

Investigation confirms intermittent encoder feedback caused by a damaged cable in the moving cable carrier.

### Corrective actions

Replace the damaged cable with an approved compatible component.

Correct the cable routing and protection.

Inspect the cable carrier and adjacent cables.

Verify axis feedback and motion using approved tests.

Assess equivalent CNC machines using the same routing arrangement.

### Preventive actions

Update the relevant inspection criteria if a gap is identified.

Standardize cable-routing requirements where necessary.

Update controlled drawings and maintenance documentation.

Review spare availability for the approved replacement cable.

### Effectiveness verification

Confirm the required axis tests pass, the installation meets the applicable routing requirements, and the original failure does not recur over the agreed operating exposure.

## 22. CAPA Register and Database Design

A structured CAPA register helps maintenance and quality teams manage actions consistently.

### Recommended fields

| Field | Purpose |
| --- | --- |
| CAPA_ID | Unique identifier |
| Open_Date | Date CAPA was initiated |
| Source | Breakdown, audit, quality finding, risk review, or other source |
| Equipment_ID | Affected asset or functional location |
| Problem_Statement | Factual description of the problem |
| Risk_Priority | Approved priority or risk classification |
| Containment_Status | Status of immediate risk controls |
| RCA_ID | Linked root cause investigation |
| Confirmed_Cause | Evidence-supported cause |
| Action_Description | Required corrective or preventive measure |
| Action_Owner | Accountable owner |
| Due_Date | Planned completion date |
| Implementation_Status | Controlled action status |
| Verification_Status | Result of implementation checks |
| Effectiveness_Due_Date | Planned review date |
| Effectiveness_Result | Pending, effective, or ineffective |
| Closure_Date | Authorized closure date |
| Related_Record_ID | Linked CAR, work order, audit, or other record |
| Evidence_Link | Reference to controlled evidence |

Use unique IDs and controlled status values. Keep action completion, implementation verification, and effectiveness review as separate states.

## 23. CAPA in SAP and Maintenance Workflows

CAPA often spans several organizational processes.

A conceptual relationship is:

Equipment Master

↓

Breakdown / SAP Notification

↓

Failure and Downtime Records

↓

Fault and Alarm Evidence

↓

RCA Investigation

↓

CAR / CAPA Action Register

↓

SAP Work Order or Approved Change

↓

Implementation and Test Evidence

↓

Effectiveness Review

↓

Closure and Standardization

The exact SAP object relationships depend on the organization's configuration. A CAPA may be managed in a quality module, an EHS process, a separate action register, or an integrated workflow.

Useful linked information includes:

Equipment ID and functional location.

Notification and work-order references.

Failure and cause codes.

RCA and CAR identifiers.

Action owner and due date.

Material and vendor references.

Test records and document revisions.

Effectiveness-review date and result.

The goal is traceability. A reviewer should be able to follow the path from the original failure to the investigation, actions, evidence, and final decision.

## 24. CAPA Dashboard in Power BI

A CAPA dashboard should help prioritize risk and identify ineffective processes, not merely count closed reports.

Useful views include:

Open CAPAs by priority.

Overdue actions.

Actions due in the next defined period.

CAPAs awaiting effectiveness review.

CAPAs closed as ineffective or subsequently reopened.

Recurring failures associated with previous CAPAs.

CAPAs by equipment family or failure category.

Average implementation and closure duration.

Effectiveness rate by action type or department.

Similar-equipment assessments completed versus planned.

### Important interpretation

A department with many open CAPAs may be investigating more seriously than a department that closes everything quickly. Closure volume alone is not a reliable performance measure.

Use risk, action quality, recurrence, and effectiveness to interpret results.

## 25. CAPA Performance Metrics

### 25.1 On-time action completion rate

On-time completion rate=Actions completed by their due datesActions due in the period×100

Define how formally approved extensions and cancelled actions are treated.

### 25.2 Effectiveness rate

Effectiveness rate=Actions verified effectiveActions evaluated×100

Pending actions should be handled according to a consistent reporting definition.

### 25.3 Recurrence rate

Track the number of qualifying recurrences of the same verified failure mode over a defined time period or operating exposure after action implementation.

### 25.4 Overdue action rate

Overdue action rate=Open actions past due dateOpen actions requiring completion×100

### 25.5 Average completion time

Measure elapsed time from action assignment to implementation completion, and separately from CAPA opening to final closure when required.

Always document the metric definition. Otherwise, two departments may report different numbers while claiming to measure the same thing.

## 26. Common CAPA Mistakes

Raising actions without clearly defining the problem.

Confusing correction with corrective action.

Selecting an action before verifying the cause.

Treating a temporary workaround as a permanent solution.

Failing to inspect similar equipment.

Assigning actions without owners or due dates.

Using training as the default response to every problem.

Changing PLC logic or drive settings without controlled engineering approval.

Closing CAPA when the work order is complete.

Defining no measurable effectiveness criteria.

Monitoring for too short a period.

Failing to document evidence.

Allowing temporary controls to remain indefinitely.

Ignoring repeated failure after closure.

Measuring success by closure rate rather than risk reduction and effectiveness.

## 27. Practical CAPA Checklist

Problem is documented clearly and factually.

Risk and impact are assessed.

Immediate containment is in place where required.

Relevant evidence is preserved.

RCA or risk assessment is completed to the required level.

Corrective actions address verified causes.

Preventive measures assess relevant similar equipment and shared risks.

Owners, due dates, and acceptance criteria are defined.

Required approvals and change controls are completed.

Implementation evidence is recorded.

Functional and technical verification passes.

Effectiveness criteria and monitoring exposure are defined.

Relevant drawings, PM tasks, SOPs, and training are updated.

Related SAP and maintenance records are linked.

Ineffective actions are reopened or revised.

Final closure is approved according to procedure.

## 28. Interview Questions and Answers

Q1. What is CAPA?

CAPA is a structured process for investigating problems, implementing corrective measures, preventing similar problems where applicable, and verifying that the actions are effective.

Q2. What is the difference between RCA and CAPA?

RCA investigates and establishes the causes of a problem. CAPA manages the resulting actions, including implementation, verification, and effectiveness review.

Q3. What is the difference between correction and corrective action?

Correction fixes the immediate problem. Corrective action addresses its cause to prevent recurrence.

Q4. What is containment?

Containment limits the immediate impact or risk while the investigation and permanent solution are pending.

Q5. Why should similar machines be assessed during CAPA?

They may share the same design weakness, component defect, installation practice, or operating exposure. A broader assessment helps prevent the same failure elsewhere.

Q6. How do you verify CAPA effectiveness?

Define measurable criteria, collect evidence over suitable operating exposure, and confirm that the intended result was achieved without unacceptable recurrence.

Q7. What should happen if the failure returns after CAPA closure?

Preserve the new evidence, reassess the original cause and actions, reopen or initiate a linked investigation according to procedure, and implement a stronger evidence-based solution.

Q8. Is a high CAPA closure rate proof of good maintenance?

No. Effective risk reduction, quality of investigation, recurrence prevention, and timely completion of appropriate actions are more meaningful than closure volume alone.

Q9. Why should CAPA actions be linked to SAP work orders?

The linkage provides traceability between approved actions, maintenance execution, material usage, technical evidence, and the original failure.

Q10. What is the most important indicator of successful CAPA?

The actions address the verified causes, meet the defined acceptance criteria, and demonstrate effectiveness through suitable evidence and operating exposure.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\22_Maint_Logs\08_MTBF - MTBF