# Min/Max Stock in Industrial Maintenance

## 1. Definition

Min/Max stock is an inventory control method that defines a minimum stock level and a maximum stock level for a material. When inventory reaches or falls below the minimum threshold, replenishment is initiated to restore stock toward the maximum level.

The method helps ensure that maintenance materials, spare parts, and consumables are available without holding excessive inventory.

In industrial maintenance, Min/Max stock is particularly useful for items such as proximity sensors, bearings, pneumatic fittings, fuses, contactors, filters, PLC modules, and other regularly consumed materials.

The central objective is to balance two competing risks:

Stockout risk: Required material is unavailable when maintenance needs it.

Overstock risk: Excess material consumes capital, storage space, and may become obsolete or deteriorate.

Min/Max stock is an inventory policy, not simply two arbitrary numbers entered into SAP or Excel. The values should reflect demand, replenishment lead time, criticality, supplier reliability, and operational risk.

## 2. Meaning of Minimum and Maximum Stock

Minimum stock is the defined lower inventory threshold used to trigger replenishment or escalation.

Maximum stock is the desired upper inventory level after replenishment, subject to the organization's inventory policy and actual requirements.

Example:

Minimum stock: 5 units.

Maximum stock: 15 units.

Current stock: 4 units.

The stock has fallen below the minimum threshold. If the inventory position is 4 units and there are no open orders or other adjustments, the replenishment quantity required to reach the maximum is:

Qorder=Smax−IP

Qorder=15−4=11 units

Where:

Qorder = proposed order quantity.

Smax = maximum stock level.

IP = inventory position.

The order quantity may need to be adjusted for pack size, minimum order quantity, supplier constraints, outstanding reservations, and approved purchasing rules.

## 3. How Min/Max Stock Works

Inventory Above Maximum

↓

No Routine Replenishment Required

↓

Stock Decreases Through Consumption

↓

Stock Reaches or Falls Below Minimum

↓

Check Inventory Position and Open Orders

↓

Calculate Replenishment Quantity

↓

Create or Release Replenishment Request

↓

Receive and Inspect Material

↓

Inventory Returns Toward Maximum

This is a simplified process. Actual replenishment may be triggered by projected inventory position rather than physical stock alone.

For example, a warehouse may hold 6 units physically, but if 4 units are reserved for confirmed maintenance work, the quantity available for other demand is only 2 units.

## 4. Important Inventory Terms

| Term | Meaning |
| --- | --- |
| Minimum stock | Lower threshold used to trigger replenishment |
| Maximum stock | Target upper inventory level |
| On-hand stock | Quantity physically recorded in inventory |
| Available stock | Stock available for new requirements after applicable restrictions |
| Reserved stock | Stock allocated to specific requirements |
| Inventory position | Stock on hand plus valid incoming supply minus applicable committed demand |
| Reorder point | Inventory position at which replenishment should be triggered |
| Safety stock | Buffer intended to protect against demand or supply uncertainty |
| Lead time | Time between defined replenishment events |
| Order quantity | Quantity proposed for replenishment |
| Minimum order quantity | Smallest quantity accepted by the supplier |
| Economic order quantity | Calculated order quantity intended to balance ordering and holding costs under specified assumptions |

Definitions of inventory position and available stock can vary by ERP configuration. Use consistent rules when implementing the calculation.

## 5. Min/Max Stock Formulas

### 5.1 Replenishment trigger

A basic policy triggers replenishment when:

IP≤Smin

Where IP is inventory position and Smin is the minimum threshold.

Some systems use a strict less-than condition, while others trigger at less than or equal to the minimum. The policy must be defined consistently.

### 5.2 Replenishment quantity

A basic replenishment calculation is:

Qorder=max(0,Smax−IP)

This calculates the quantity needed to restore inventory position to the maximum.

It is a starting point, not a complete purchasing decision. Minimum order quantities, pack sizes, open orders, reservations, and demand forecasts can change the final quantity.

### 5.3 Minimum stock based on demand and lead time

A simple planning model is:

Smin=dL+SS

Where:

d = average demand per time unit.

L = replenishment lead time in the same time unit.

SS = safety stock.

This resembles a reorder-point calculation. It is appropriate only when the minimum threshold is intended to cover expected demand during lead time plus a safety buffer.

### 5.4 Maximum stock

A basic model is:

Smax=d(L+R)+SS

Where:

R = review period between replenishment opportunities.

L = replenishment lead time.

d = average demand rate.

SS = safety stock.

This model assumes a periodic-review policy in which inventory must cover demand during the review period and the replenishment lead time. Other Min/Max systems use different maximum-stock rules.

## 6. Practical Numerical Example

A maintenance store manages a proximity sensor used regularly on an automated production line.

Given:

Average demand: 3 sensors per month.

Replenishment lead time: 2 months.

Safety stock: 2 sensors.

Review period: 1 month.

Minimum stock:

Smin=(3×2)+2=8

Maximum stock:

Smax=(3×(2+1))+2=11

Therefore:

Minimum stock = 8 sensors.

Maximum stock = 11 sensors.

Suppose the inventory position falls to 7 sensors. Replenishment is triggered.

Qorder=11−7=4 sensors

The proposed order is 4 sensors, assuming no additional constraints.

The maximum exceeds the minimum because it also provides coverage for demand during the review period. These figures are illustrative; actual values should be calculated from reliable consumption and lead-time data.

## 7. Difference Between Min/Max and Other Inventory Methods

| Method | Main principle | Typical application |
| --- | --- | --- |
| Min/Max | Replenish when inventory reaches the lower threshold, aiming toward the upper threshold | Regularly consumed maintenance materials |
| Reorder point | Trigger an order when inventory position reaches a specified point | Items with defined demand and replenishment lead time |
| Fixed order quantity | Order a predetermined quantity when a trigger occurs | Standard materials with stable ordering quantities |
| Periodic review | Review inventory at defined intervals and replenish according to the policy | Stores with scheduled purchasing cycles |
| Safety stock | Maintain a buffer against uncertainty | Critical or variable-demand items |
| Just-in-time | Coordinate supply closely with actual requirements | Suitable items with reliable supply and demand coordination |
| Two-bin system | Consume one bin while the other acts as a replenishment signal or reserve | Low-cost, frequently used consumables |

Min/Max can incorporate reorder-point and safety-stock concepts. These are not necessarily competing methods.

## 8. Selecting Min/Max Levels

The values should be selected according to the material's demand pattern, supply characteristics, and consequences of shortage.

### 8.1 Consumption rate

Calculate historical usage over a meaningful period.

d=Total consumptionObservation period

For example, if 36 units were consumed over 12 months:

d=3612=3 units/month

Historical consumption should be checked for abnormal events, one-time projects, planned shutdowns, and stockouts that suppressed recorded issues.

### 8.2 Replenishment lead time

Use actual historical replenishment duration where reliable data is available.

Consider:

Supplier manufacturing time.

Internal approval delays.

Transportation.

Goods receipt.

Inspection and release.

Variability between orders.

A nominal supplier quotation may underestimate the time required for usable stock to become available.

### 8.3 Safety stock

Safety stock should reflect the uncertainty and consequences associated with a stockout.

Factors include:

Demand variability.

Lead-time variability.

Equipment criticality.

Supplier reliability.

Repair alternatives.

Availability of substitutes.

Consequences of production interruption.

A high-value critical PLC module with rare demand may require a different stocking strategy from a low-cost standard fuse consumed daily.

### 8.4 Order constraints

Check:

Minimum order quantity.

Supplier pack size.

Shelf life.

Storage capacity.

Purchase-order restrictions.

Budget constraints.

Approved supplier conditions.

A calculated order of 3 units may need to become 5 if the supplier sells only packs of 5. The resulting excess stock should be evaluated rather than silently ignored.

## 9. Min/Max Stock for Different Material Categories

| Material category | Stocking approach | Important consideration |
| --- | --- | --- |
| General consumables | Consumption-based Min/Max | Usage rate and storage conditions |
| Wear parts | Demand and service-interval-based planning | Wear rate and replacement schedule |
| Critical spares | Risk-based stocking with suitable buffers | Failure consequence and replenishment delay |
| Insurance spares | Individual risk and lifecycle assessment | Rare demand and severe consequences |
| Repairable components | Pool, exchange, and repair-cycle planning | Repair turnaround and return rate |
| Obsolete components | Controlled stock and migration planning | No future replenishment may be possible |
| Safety components | Approved risk-based stock policy | Compatibility and required certification |
| Long-lead-time components | Lead-time-aware inventory policy | Extended replenishment exposure |

Not every material should be assigned Min/Max levels. Some items are better managed as project-specific purchases, repairable pools, strategic insurance spares, or controlled end-of-life stock.

## 10. Criticality-Based Min/Max Planning

Classify materials according to the consequences of shortage.

A practical framework is:

High criticality: Failure can stop production, affect safety, or create a severe operational consequence.

Medium criticality: Failure affects a subsystem but may allow limited operation or temporary mitigation.

Low criticality: Failure has limited consequences and recovery is straightforward.

Criticality should influence the buffer, sourcing strategy, review frequency, and escalation process. It should not automatically dictate a large quantity for every critical item.

For example, a spare safety controller may have high criticality but very low demand. The correct policy could be one validated spare with a controlled storage and replacement plan rather than a large quantity.

## 11. ABC and XYZ Analysis

Min/Max stock can be improved by classifying items according to value and demand behavior.

### ABC analysis

ABC classification groups materials according to their share of annual consumption value.

Vannual=Dannual×Cunit

Where:

Dannual = annual consumption quantity.

Cunit = unit cost.

Typical interpretation:

A items: High share of annual consumption value; close control.

B items: Intermediate value; moderate control.

C items: Lower share of annual consumption value; simpler control may be appropriate.

The exact thresholds should be established from the site's consumption-value distribution.

### XYZ analysis

XYZ classification groups items according to demand predictability.

X items: Relatively stable and predictable demand.

Y items: Variable or seasonal demand.

Z items: Irregular, intermittent, or difficult-to-predict demand.

ABC describes value, while XYZ describes demand behavior. Neither directly measures equipment criticality.

A high-value, irregular-demand spare may require a risk-based policy rather than ordinary consumption-driven Min/Max settings.

## 12. Min/Max Stock for PLC, CNC, Robot, and Automation Spares

Industrial automation components often have low consumption frequency but high consequences when unavailable.

### PLC and remote I/O

Track:

Exact catalog number.

Hardware revision and compatibility.

Firmware and engineering-software requirements.

Communication adapter compatibility.

Installed quantity across equipment.

Obsolescence and successor models.

Supplier lead time.

Do not treat similar-looking modules as interchangeable without technical verification.

### CNC systems

Consider:

CNC CPU and operator panel.

Servo and spindle drives.

Encoder and feedback components.

Power supplies.

Communication modules.

Machine-builder-specific parts.

A spare drive may require matching ratings, compatible feedback interfaces, parameter backups, and commissioning capability.

### Industrial robots

Consider:

Controller power supplies.

Servo amplifiers.

Motors and feedback devices.

Teach pendants.

Communication modules.

Robot-specific cables and accessories.

A stocked component is not a complete recovery solution if its configuration, compatibility, or safety requirements have not been addressed.

### Sensors and field devices

For proximity sensors, photoelectric sensors, pressure switches, and analog transmitters, specify output type, voltage, range, connector, mounting, and environmental ratings.

For a replacement sensor, verify whether the PLC expects PNP or NPN, discrete or analog signals, and the appropriate scaling or switching behavior.

## 13. Stockout and Overstock Management

### Stockout indicators

Inventory position falls below minimum.

Open maintenance reservations cannot be fulfilled.

Replenishment orders are overdue.

Critical items have no validated alternative.

Production equipment is waiting for a material.

Repeated emergency purchasing is required.

### Overstock indicators

Stock exceeds maximum for an extended period.

Consumption is declining or has stopped.

The installed equipment population has changed.

A part has become obsolete.

Materials are nearing expiry.

Multiple interchangeable items are being stocked unnecessarily.

Excess material exists at one location while another location experiences shortages.

A stock level above maximum is not automatically waste. It may result from a large supplier pack, a planned shutdown, a bulk purchase, or a temporary risk response. The reason should be documented and reviewed.

## 14. Inventory Position and Open Orders

A common mistake is to calculate replenishment only from the physical shelf quantity.

A simplified inventory-position formula is:

IP=OH+OO−AD

Where:

OH = on-hand quantity.

OO = valid open-order quantity expected to arrive.

AD = applicable committed demand not already deducted from on-hand stock.

The exact calculation depends on how reservations and allocated stock are represented. Avoid subtracting the same demand twice.

Example:

On-hand stock: 4 units.

Valid open purchase order: 6 units.

Additional committed demand: 2 units.

IP=4+6−2=8 units

If the minimum is 5 and the maximum is 12, a basic policy would not trigger replenishment because the inventory position is 8.

However, if the open order is overdue, cancelled, or not expected before the material is needed, it may need to be excluded or treated separately.

## 15. Excel Implementation

A basic inventory table can contain:

| Field | Purpose |
| --- | --- |
| Material Number | Unique material identifier |
| Description | Material identification |
| Equipment Group | Associated equipment or production area |
| Criticality | Operational consequence classification |
| On-Hand Quantity | Recorded physical stock |
| Open-Order Quantity | Valid incoming supply |
| Committed Demand | Outstanding requirements |
| Inventory Position | Calculated supply position |
| Minimum Stock | Replenishment threshold |
| Maximum Stock | Target upper level |
| Reorder Status | Indicates replenishment requirement |
| Suggested Quantity | Calculated replenishment amount |
| Lead Time | Expected replenishment duration |
| Supplier | Approved supply source |

Example Excel formulas, assuming the table uses the same column names:

Inventory position:

=[@[On-Hand Quantity]]+[@[Open-Order Quantity]]-[@[Committed Demand]]

Reorder status:

=IF([@[Inventory Position]]<=[@[Minimum Stock]],"REORDER","OK")

Suggested replenishment quantity:

=IF([@[Inventory Position]]<=[@[Minimum Stock]],MAX(0,[@[Maximum Stock]]-[@[Inventory Position]]),0)

These formulas assume the inventory-position fields are already calculated correctly and the stock policy triggers at or below minimum.

For production use, also account for order multiples, minimum order quantity, expiry, reservations, and invalid or overdue supply.

## 16. SAP Application

Depending on the SAP configuration, Min/Max-related planning may be implemented using material master planning parameters and replenishment processes.

Relevant concepts include:

Material master and material type.

Plant and storage-location stock.

Reorder-point planning.

Maximum stock level.

Safety stock.

Planned delivery time.

Lot-sizing procedure.

Minimum lot size and rounding values.

Purchase requisitions and purchase orders.

Reservations and goods movements.

Important: SAP field names and behavior depend on the system version, planning method, and configuration. A Min/Max policy should be translated into the correct planning parameters rather than assuming every system uses the same field or automatic replenishment logic.

Validate the proposed settings against actual material movements and procurement history before applying them broadly.

## 17. Power BI Monitoring

A Min/Max dashboard should help maintenance and stores teams identify inventory exceptions.

Useful indicators include:

Number of materials below minimum.

Number of materials above maximum.

Critical spares with zero available stock.

Materials with overdue open orders.

Stock value by material category.

Inventory without consumption over a defined period.

Emergency purchases.

Stockout-related downtime.

Supplier lead-time performance.

Materials approaching expiry.

Obsolete items still held in stock.

Useful filters include plant, storage location, equipment group, material category, criticality, supplier, and stock status.

The dashboard should distinguish physical stock, available stock, and inventory position. Combining them into one ambiguous "stock" field is a reliable way to manufacture confusion.

## 18. Review and Optimization Frequency

Min/Max levels should be reviewed when conditions change, not merely because a calendar reminder has appeared.

Review triggers include:

A change in consumption rate.

A change in supplier lead time.

Repeated stockouts.

Persistent excess stock.

Equipment additions or removals.

New production volumes.

Planned shutdowns.

Component obsolescence.

Supplier discontinuation.

A change in repair or substitution options.

Review frequency can vary by material category. Frequently consumed materials may need routine periodic review, while critical automation spares should also be reviewed when equipment or lifecycle conditions change.

## 19. Common Mistakes

Setting minimum and maximum quantities without demand data.

Using historical issues without checking unusual events.

Ignoring replenishment lead time.

Treating all critical spares as high-consumption items.

Ignoring supplier pack sizes and minimum order quantities.

Counting overdue open orders as reliable incoming stock.

Confusing physical stock with available inventory.

Ignoring reserved material.

Applying identical Min/Max settings to every material category.

Failing to review obsolete or superseded part numbers.

Increasing stock without investigating repeated demand spikes.

Reducing inventory without assessing production and safety risk.

Allowing planning parameters to remain unchanged after equipment modifications.

## 20. Practical Implementation Checklist

Identify materials suitable for Min/Max planning.

Validate material numbers and descriptions.

Determine actual consumption and demand variability.

Establish realistic replenishment lead times.

Classify criticality and stockout consequences.

Calculate minimum stock using an appropriate policy.

Define the maximum stock and replenishment quantity.

Account for open orders, reservations, and committed demand.

Verify supplier minimum quantities and pack sizes.

Check storage conditions, expiry, and obsolescence.

Configure and test the policy in SAP or the approved inventory system.

Monitor stockouts, overdue orders, and excess inventory.

Review levels after major operational or supply changes.

Document policy ownership and approval.

## 21. Interview Questions and Answers

Q1. What is Min/Max stock?

It is an inventory control method that triggers replenishment when inventory reaches a lower threshold and aims to restore inventory toward an upper target.

Q2. How do you calculate the replenishment quantity?

A basic calculation is maximum stock minus inventory position, provided the minimum threshold has been reached and no additional ordering constraints apply.

Q3. How does lead time affect minimum stock?

Longer lead time increases the expected demand that must be covered before replenishment arrives. A suitable safety buffer may also be needed for demand or supply uncertainty.

Q4. Is Min/Max suitable for every spare part?

No. It is especially useful for suitable regularly consumed materials. Rare, high-value, repairable, obsolete, or insurance spares may require individual risk-based policies.

Q5. What is the difference between minimum stock and safety stock?

Minimum stock is the operational threshold defined by the inventory policy. Safety stock is a buffer against uncertainty. A minimum threshold may include safety stock, but the terms are not interchangeable.

Q6. Why should open purchase orders be considered?

Valid incoming supply affects the expected inventory position. However, overdue or unreliable orders must be assessed rather than blindly counted.

Q7. How would you reduce excess stock without increasing breakdown risk?

Review consumption, installed equipment, lead times, criticality, supplier constraints, and obsolete materials. Adjust planning parameters using evidence and monitor stockout consequences after changes.

## 22. Expected Outcome

A well-designed Min/Max system makes replenishment more predictable, reduces avoidable stockouts, and limits unnecessary inventory.

Its effectiveness depends on accurate material data, realistic lead times, a suitable replenishment policy, and regular review. For industrial automation, those controls must also account for equipment compatibility, lifecycle status, and the operational consequences of a missing component.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\23_Spares\01_Strategy\09_Vendor_Availability - Vendor availability