# CNC Backup and Disaster Recovery

## 1. Purpose and scope

A Computer Numerical Control (CNC) system controls machine-tool operations such as axis movement, spindle speed, tool changes, machining cycles, and coordinate positioning. Depending on the machine, it may include a CNC controller, PLC, servo drives, spindle drive, operator panel, machine-builder software, tool data, and external communication systems.

CNC backup and disaster recovery preserve the data and configuration needed to restore machine operation after controller failure, storage-device failure, parameter corruption, accidental program deletion, component replacement, or obsolescence migration.

CNC recovery requires special care because a machine may power up successfully while still having incorrect axis references, tool offsets, compensation data, PLC sequences, or machine-builder settings. Such errors can cause scrap, collisions, tool damage, or injury.

The objectives are to:

Preserve CNC programs, machine parameters, and controller configuration.

Restore machine-specific PLC logic and builder settings.

Protect tool offsets, work offsets, compensation data, and required production information.

Reduce recovery time after controller or storage failure.

Maintain traceability of program and configuration revisions.

Support CNC replacement, firmware migration, and machine modernization.

Verify machine behavior before releasing it for production.

Core principle: A CNC backup must preserve the machine's approved configuration and the data required to reproduce its intended behavior, not merely the visible part programs.

## 2. CNC system architecture

Operator Panel / HMI

↓

CNC Controller

├── Part Programs and Machining Cycles

├── Machine Parameters and Compensation Data

├── Coordinate Systems and Tool Data

└── Machine-Builder PLC / Sequence Logic

↓

Axis and Spindle Control

↓

Servo Drives / Spindle Drive

↓

Motors, Encoders, and Feedback

↓

Machine Mechanics and Tooling

Supporting systems:

Network configuration, engineering software, manuals,

backup storage, tool-management data, and recovery records

The exact architecture depends on the CNC manufacturer and machine builder. Some machines store data across multiple devices, while others use centralized controller storage or separate PLC and CNC processors.

## 3. Essential terminology

| Term | Meaning | Industrial example |
| --- | --- | --- |
| CNC | Computer Numerical Control system that executes programmed machine operations | CNC machining center |
| Part program | Program defining machining operations and movements | G-code program for drilling and milling |
| Machine parameters | Settings defining machine behavior and controller configuration | Axis travel limits and reference parameters |
| PLC program | Logic controlling machine sequences, interlocks, and auxiliary functions | Tool changer sequence |
| Machine-builder data | OEM-specific configuration, cycles, settings, and interface information | Spindle or magazine management configuration |
| Work offset | Coordinate transformation locating a workpiece relative to the machine coordinate system | G54 work coordinate system |
| Tool offset | Data describing tool geometry or compensation | Tool length and radius compensation |
| Backlash compensation | Correction for mechanical lost motion where supported | Compensation for screw or transmission backlash |
| Pitch-error compensation | Correction for measured axis positioning errors | Linear-axis compensation table |
| Reference return | Procedure establishing the machine axis reference | Returning an axis to its reference position |
| Absolute encoder data | Position information retained by a supported absolute-feedback system | Absolute position after power loss |
| Ladder program | Graphical or textual PLC logic used for machine control | Machine-builder interlock logic |
| PMC | Programmable Machine Controller, a term used in some CNC families for machine-sequence control | FANUC PMC logic |
| SRAM backup | Backup of supported data held in static random-access memory | CNC parameter or compensation backup on certain systems |
| System image | A captured image of supported storage or system partitions | Controller storage recovery image |
| Recovery media | Manufacturer-supported media used to restore software or system data | CNC recovery USB or service media |
| Golden backup | Approved reference backup representing a known-good machine configuration | Released baseline after commissioning |

## 4. What must be backed up?

CNC data is often divided into several categories. The exact categories and storage locations vary by controller, machine builder, and software version.

| Backup item | Typical contents | Why it matters |
| --- | --- | --- |
| CNC part programs | Approved machining programs and subprograms | Preserves production capability |
| CNC parameters | Controller-specific machine configuration | Restores controller behavior |
| PLC or PMC program | Machine sequencing, interlocks, and auxiliary logic | Restores machine functions |
| Machine-builder data | OEM-specific cycles, screens, macros, and configuration | Preserves machine-specific operation |
| Tool offsets | Tool length, radius, wear, and related data | Maintains machining accuracy |
| Work offsets | Workpiece coordinate systems and fixture references | Maintains part location and machining coordinates |
| Compensation data | Supported pitch-error, backlash, and other compensation tables | Preserves axis accuracy |
| Servo parameters | Drive settings, motor configuration, and supported tuning data | Restores axis behavior |
| Spindle configuration | Spindle parameters, orientation settings, and drive interface data | Restores spindle operation |
| Tool-management data | Tool inventory, pocket mapping, and tool-life records where supported | Supports correct tool handling |
| Custom macros | User-defined cycles and macro programs | Preserves specialized machining functions |
| System configuration | Controller options, installed software, and supported system settings | Supports compatible restoration |
| Network configuration | IP addresses, device names, and communication settings | Restores network integration |
| Alarm history | Relevant alarms and diagnostic records | Supports fault investigation |
| Documentation | Electrical diagrams, manuals, parameter lists, and recovery procedures | Supports maintenance work |
| Firmware and software records | CNC, PLC, drive, and engineering-software versions | Helps verify compatibility |
| Validation records | Machine geometry, reference checks, and acceptance results | Establishes the approved machine condition |

Not every item is stored in the CNC controller. Some data may reside in the PLC, servo drives, tool-management system, network server, external PC, or machine-builder-specific storage.

## 5. CNC backup classification

### 5.1 Part-program backup

Includes machining programs, subprograms, custom macros, and related files.

Important controls:

Preserve approved production revisions.

Record program numbers and file names.

Keep revision history and change authorization.

Protect critical programs against accidental deletion.

Verify that required subprograms and macro dependencies are included.

Preserve units, tool-call assumptions, and machine-specific requirements.

Confirm that the saved program matches the approved production version.

A program file by itself does not preserve the machine's tool offsets, work offsets, parameters, or PLC sequence logic.

### 5.2 Machine-parameter backup

Includes supported controller parameters and configuration values that define axis, spindle, communication, reference, and machine behavior.

Parameter backup must be specific to the CNC model and machine configuration. Parameters from a different machine may look similar but have different values because of different motors, encoders, gearboxes, travels, spindle arrangements, or machine-builder options.

### 5.3 PLC or PMC backup

Includes the logic responsible for auxiliary machine functions, such as:

Tool changer sequences.

Coolant and lubrication control.

Hydraulic and pneumatic sequences.

Door and chuck interlocks.

Pallet and fixture handling.

Spindle orientation and readiness signals.

Machine alarms and status signals.

Communication between the CNC and machine I/O.

A CNC can display normally while its PLC or PMC program is missing or incompatible. The resulting machine may be unable to perform normal sequences.

### 5.4 System and storage backup

Some CNC controllers support full or partial system images, recovery media, or manufacturer-specific system restoration procedures.

These backups may include operating-system files, application software, configuration, or selected data partitions. They may also be tied to particular controller hardware, storage layouts, firmware versions, or license arrangements.

Do not assume that copying the controller's storage device creates a valid recovery image. Use manufacturer-supported methods and retain any required recovery media or service documentation.

## 6. CNC backup methods

### 6.1 Manufacturer-supported backup utility

Use the controller manufacturer's supported utility to save the CNC parameters and other supported configuration data.

Typical workflow:

1.	Identify the CNC controller model, machine model, software version, and machine-builder configuration.

2.	Confirm the approved maintenance state and required access level.

3.	Identify which data is stored in the CNC, PLC/PMC, drives, and external systems.

4.	Use the supported utility to save parameters and configuration files.

5.	Back up the PLC/PMC program separately if required.

6.	Export part programs, macros, offsets, and compensation data where supported.

7.	Save a readable parameter listing for comparison.

8.	Verify the backup files and record their revisions.

9.	Store the files in the controlled repository.

10.	Confirm that the documented restoration method is available.

Examples of manufacturer environments include FANUC CNC data backup and restoration functions, Siemens SINUMERIK Operate and supported service tools, and Mitsubishi CNC maintenance and data-transfer utilities. The exact utility and supported data depend on the controller generation and machine builder.

### 6.2 External storage or network backup

Depending on the CNC, programs and selected data may be transferred using supported USB storage, network shares, Ethernet transfer tools, or an engineering workstation.

Before using external storage:

Verify the supported file system and media type.

Confirm the permitted transfer method for the controller.

Check file names, program numbers, extensions, and directory structure.

Preserve the original files before conversion or editing.

Confirm that network paths and permissions are correct.

Verify that transferred files are readable on the target system.

Do not connect unapproved storage devices to production controllers. External media and engineering PCs can introduce malware or unauthorized configuration changes.

### 6.3 Parameter listing or printed report

A parameter listing can provide a readable record of the controller configuration.

It is useful for:

Comparing values before and after a repair.

Reviewing machine-specific settings.

Supporting troubleshooting when electronic files are unavailable.

Recording important values during migration.

Providing an audit trail.

A printed or exported list may not include all binary data, system files, compensation tables, options, or proprietary configuration. Treat it as supporting documentation rather than a complete system backup.

### 6.4 Storage-device image

A storage image may be appropriate where the manufacturer supports recovery of the controller's storage device.

Before creating or restoring an image:

Identify the exact controller hardware and storage arrangement.

Follow the manufacturer's approved imaging procedure.

Confirm image completeness and integrity.

Verify any hardware-bound licensing or controller identity requirements.

Preserve the original storage device where practical.

Confirm the recovery method and required boot or service media.

A raw storage image is not automatically portable between different CNC controllers. Differences in hardware, storage geometry, firmware, or software licensing may prevent successful restoration.

## 7. CNC-specific data that requires special protection

### 7.1 Tool offsets

Tool data may include tool length, radius, wear corrections, tool orientation, and tool-management information.

Incorrect offsets can cause dimensional errors, collisions, or tool damage.

During recovery:

1.	Retrieve the approved offset data.

2.	Confirm that tool numbers and tool identities match the actual tools.

3.	Verify the units and offset conventions used by the controller.

4.	Confirm that tool-length and radius compensation are assigned correctly.

5.	Check whether offsets are stored in the CNC, tool-management system, or another device.

6.	Validate critical tool data using the approved setup and inspection process.

Do not restore an old offset file without confirming whether tools, holders, fixtures, or machine setup have changed.

### 7.2 Work offsets and coordinate systems

Work offsets define the relationship between the machine coordinate system and the workpiece or fixture coordinate system.

Examples include G54 through G59 on controllers that support these standard work-coordinate selections, plus additional coordinate systems where available.

Recovery checks should include:

Work-offset values and units.

Fixture and workpiece identification.

Machine coordinate reference.

Coordinate rotation or transformation settings where used.

Tool-center-point or kinematic transformation settings where applicable.

Any separate probing or measurement offsets.

A correct part program combined with an incorrect work offset can move the tool to the wrong physical location.

### 7.3 Compensation data

Compensation data can be critical to dimensional accuracy.

| Compensation type | Purpose | Recovery consideration |
| --- | --- | --- |
| Backlash compensation | Corrects supported effects of mechanical lost motion | Verify the machine's current mechanical condition |
| Pitch-error compensation | Corrects measured positioning error along an axis | Confirm the correct axis and machine-specific table |
| Thermal compensation | Corrects supported thermal effects | Verify sensors, model settings, and calibration |
| Tool-length compensation | Adjusts for tool length | Match data to the actual tool and holder |
| Tool-radius compensation | Adjusts the programmed tool path for tool radius | Verify tool geometry and controller conventions |
| Rotary-axis compensation | Corrects supported rotary-axis errors | Verify axis configuration and calibration data |
| Kinematic compensation | Corrects machine-specific geometric relationships where supported | Preserve machine-specific calibration and validation records |

Do not replace calibrated compensation tables with generic values. If compensation data is missing or questionable, follow the machine builder's metrology and calibration procedure.

### 7.4 Machine coordinates and reference positions

CNC machines may rely on reference-return procedures, absolute encoders, machine-coordinate parameters, or combinations of these methods.

Before recovery:

Verify whether reference position data was preserved.

Check encoder and battery status where relevant.

Confirm the actual physical position of the axes.

Verify the correct reference-return sequence.

Review machine travel limits and reference switches.

Check whether mechanical movement occurred during power loss or maintenance.

Never assume that the displayed axis position is physically correct merely because the controller starts without an alarm.

## 8. CNC backup frequency and triggers

| Trigger | Required action |
| --- | --- |
| Initial machine commissioning | Create the approved baseline backup |
| Part-program release | Archive the released program and its dependencies |
| Parameter modification | Save the approved parameter revision after verification |
| PLC/PMC change | Preserve the released machine-control logic |
| Tool or work-offset change | Back up data where required by the recovery policy |
| Compensation adjustment | Preserve the updated compensation data and calibration record |
| CNC controller replacement | Restore compatible data and validate machine behavior |
| Storage-device replacement | Preserve the supported system image and recovery files |
| Firmware or software update | Save the pre-update configuration and verify the post-update state |
| Machine geometry work | Preserve relevant calibration and compensation records |
| Network or system integration change | Back up updated communication and interface settings |
| Obsolescence migration | Preserve source configuration and separately release target configuration |
| Periodic audit | Verify backup currency, integrity, and restoration readiness |

For critical CNC equipment, use event-driven backups after approved changes and define periodic verification intervals according to equipment criticality and plant policy.

## 9. Backup integrity, storage, and version control

### 9.1 Backup validation

| Validation level | What it confirms | What remains unverified |
| --- | --- | --- |
| File existence | Backup files are present | Completeness and correctness |
| File integrity | Files match their recorded checksums | Suitability for the target controller |
| File readability | Supported software can read the saved files | Successful restoration |
| Parameter comparison | Values match the approved baseline | Correct physical machine behavior |
| Compatibility check | Controller and software requirements are satisfied | Full recovery success |
| Restore test | Restoration succeeds in the tested environment | Untested production conditions |
| Functional verification | Required machine functions operate correctly | Any functions not included in the test |

A SHA-256 checksum can detect unexpected file changes when compared with a trusted recorded checksum. It does not prove that the configuration is correct or safe.

### 9.2 Version naming

Use a controlled naming convention based on the plant's approved asset identifiers.

MACHINE_ID_CNC_DATA_TYPE_REVISION_DATE

MACHINE1059_CNC_PARAMS_R005_20261017

MACHINE1059_PLC_PROGRAM_R012_20261017

MACHINE1059_PART_PROGRAM_O1234_R003_20261017

These are illustrative examples. Follow the controller's supported naming rules, especially where program numbers or file-name formats are restricted.

Record:

Machine ID and CNC controller identification.

Controller model and software version.

Machine-builder and PLC/PMC program revisions.

Backup type and included data.

Backup timestamp and responsible person.

Approved revision and change reference.

Checksum where required.

Storage location and independent recovery-copy status.

Validation or restoration-test status.

### 9.3 Storage architecture

Primary controlled engineering repository

↓

Independent backup repository

↓

Offline or isolated recovery copy

↓

Verified recovery media and procedure

Apply the 3-2-1 principle where practical: maintain at least three copies, use two storage types or platforms, and keep one copy off-site or isolated.

Protect CNC programs and configuration files from unauthorized changes. Use controlled access, retention policies, and appropriate cybersecurity controls.

## 10. CNC restoration procedure

### 10.1 Pre-restoration checklist

Identify the machine, CNC model, controller version, and machine builder.

Determine which component failed and which data was lost.

Retrieve the latest approved backup package.

Verify file integrity and configuration revision.

Confirm controller, firmware, hardware, and software compatibility.

Identify required options, licenses, and recovery media.

Check whether CNC parameters, PLC/PMC logic, and system files require separate restoration.

Preserve the current state before making further changes where practical.

Confirm tool offsets, work offsets, compensation data, and machine-reference information.

Review electrical isolation, machine safeguarding, and commissioning requirements.

Establish an approved recovery and acceptance-test plan.

### 10.2 Restoration sequence

1.	Diagnose the failure and determine the required recovery scope.

2.	Place the machine in the approved safe state and secure any hazardous mechanical movement.

3.	Confirm that the backup matches the intended machine and controller generation.

4.	Verify the required software, hardware, options, and service access.

5.	Restore the controller system using the manufacturer's supported procedure where required.

6.	Restore CNC parameters and machine-specific configuration.

7.	Restore PLC/PMC programs and required machine-builder data.

8.	Restore part programs, macros, tool data, work offsets, and compensation data as applicable.

9.	Verify axis and spindle drive configuration.

10.	Check network communication and controller-to-drive interfaces.

11.	Confirm the machine-reference method and physical axis state.

12.	Perform required reference return and controlled movement checks.

13.	Verify tool-change sequences, spindle functions, coolant, lubrication, and other relevant auxiliaries.

14.	Complete machine-specific functional and safety verification.

15.	Save the verified final configuration and update the recovery register.

The sequence depends on the controller and the machine builder. Some systems require a particular order of restoration or commissioning. Follow the applicable manufacturer's procedure rather than treating all CNC systems as interchangeable.

### 10.3 Safety precautions

Follow the plant's lockout/tagout and hazardous-energy isolation procedures.

Secure gravity-loaded or mechanically unstable components.

Verify the machine's physical state before commanding axis movement.

Do not bypass guards, interlocks, or safety functions to complete a recovery test.

Check that the spindle, tool changer, pallet system, and auxiliary equipment are in safe conditions.

Use reduced-speed or other approved commissioning modes where applicable.

Ensure that the correct tool, fixture, workpiece, and coordinate system are selected before machining tests.

Verify required safety functions according to the approved safety-validation process.

A CNC controller that boots without errors is not proof that the machine is safe or ready to machine a part.

## 11. Post-restoration verification

### 11.1 Functional test checklist

CNC controller starts correctly and reports the expected software version.

CNC parameters match the approved configuration.

PLC/PMC logic and machine-builder functions are correct.

Servo and spindle drives are recognized and report healthy status.

Axis reference and displayed positions are consistent with the machine state.

Axis directions, travel limits, and reference-return functions are verified.

Tool and work offsets match the approved setup.

Compensation tables and required calibration data are present.

Spindle speed, orientation, and relevant feedback are verified.

Tool changer and other auxiliary sequences operate correctly.

Coolant, lubrication, hydraulic, and pneumatic functions are verified as applicable.

Network communication and required interfaces are working.

Alarms and fault-recovery behavior are checked.

Safety functions and interlocks are verified.

A controlled test program or approved proving procedure is completed.

Dimensional results meet the specified acceptance criteria.

The final verified backup is saved and documented.

### 11.2 Proving a CNC after recovery

A suitable proving process may include:

1.	Confirming machine configuration and reference status.

2.	Verifying motion under approved conditions without cutting where practical.

3.	Checking spindle and auxiliary functions.

4.	Reviewing the test program, tool list, work offset, and intended machine path.

5.	Running the approved dry-run, simulation, or reduced-risk proving method where supported.

6.	Performing the required test cut or calibration procedure.

7.	Measuring the result against the drawing and approved tolerance.

8.	Recording the acceptance results and releasing the machine through the plant's process.

The test method must suit the machine, tooling, workpiece, and risk assessment. Simulation does not replace physical verification of the restored machine configuration.

## 12. Common CNC disaster-recovery scenarios

### 12.1 CNC controller failure

Situation: The controller fails to start or must be replaced.

Identify the failed controller and supported replacement.

Verify the system software, hardware options, licenses, and storage requirements.

Retrieve the approved CNC, PLC/PMC, and machine-builder backups.

Restore data using the supported procedure.

Verify machine parameters, axis references, offsets, and compensation.

Complete the required machine-specific commissioning and acceptance tests.

A controller replacement may require machine-builder service data or software options that are not included in a routine parameter backup.

### 12.2 CNC storage-device failure

Situation: The controller's storage device fails or its contents become corrupted.

Preserve the failed device for authorized investigation where practical.

Confirm the required recovery media and controller compatibility.

Restore the supported system image or software environment.

Restore machine-specific parameters and programs.

Verify PLC/PMC logic and other separately stored data.

Confirm licensing, controller identity, and software options.

Perform the full required functional verification.

A replacement storage device may need a specific partition layout, boot configuration, or manufacturer-supported recovery image.

### 12.3 Incorrect machine parameter modification

Situation: An axis behaves incorrectly after a parameter change.

1.	Stop the machine using the approved procedure.

2.	Record the current alarm and configuration state.

3.	Identify the modified parameters and change history.

4.	Compare current values against the approved baseline.

5.	Restore the appropriate approved values under authorization.

6.	Verify axis reference, limits, scaling, and compensation.

7.	Complete the required controlled movement and acceptance tests.

8.	Record the corrective action and final configuration revision.

Do not restore every parameter indiscriminately when only one configuration group changed. Some parameters may have been legitimately updated since the last full backup.

### 12.4 Part program or macro loss

Situation: A production program or custom macro is missing.

Retrieve the approved program revision from the controlled repository.

Verify program number, machine compatibility, and dependencies.

Confirm the tool list, work offset, and machining assumptions.

Check whether the program calls subprograms or custom macros.

Verify the program against the approved production revision.

Prove the program using the approved procedure before machining production parts.

A syntactically valid program can still be unsuitable for the installed tooling, fixture, or machine configuration.

### 12.5 Tool offset or work offset corruption

Situation: A machine reports unexpected dimensions or incorrect tool positions.

Stop production and protect the workpiece and tooling.

Confirm the actual tool and fixture configuration.

Compare current offsets with the approved records.

Verify the machine coordinate reference.

Check for changes to tool geometry, holders, or workpiece setup.

Restore or re-establish the correct values.

Verify dimensional accuracy before releasing production.

Do not assume that the previous offset values remain valid after a tool, fixture, or mechanical change.

### 12.6 PLC/PMC recovery failure

Situation: The CNC powers up but machine sequences do not work.

Potential causes include:

Incorrect PLC/PMC program revision.

Missing machine-builder data.

Incompatible controller or PLC firmware.

Incorrect I/O configuration.

Changed drive or network interface.

Missing machine options or dependencies.

Incorrect machine-builder parameter configuration.

Verify the approved program, I/O mapping, interface signals, and machine-builder requirements before attempting to modify logic.

### 12.7 CNC obsolescence migration

Situation: An obsolete controller must be replaced by a newer CNC generation.

1.	Preserve the original CNC and PLC/PMC data.

2.	Record controller, firmware, options, drives, motors, encoders, and network details.

3.	Obtain the machine builder's migration requirements.

4.	Check part-program and macro compatibility.

5.	Map supported parameters and identify changed or removed functions.

6.	Review axis scaling, reference positions, compensation, and tool data.

7.	Validate drive and communication compatibility.

8.	Test machine sequences, interlocks, and safety functions.

9.	Prove machining performance against approved acceptance criteria.

10.	Retain the original backup and released target configuration separately.

A successful software conversion does not prove that the replacement machine configuration is mechanically accurate or safe.

## 13. CNC backup troubleshooting

| Symptom | Possible cause | Corrective action |
| --- | --- | --- |
| Backup transfer fails | Unsupported media, communication issue, or access restriction | Verify supported transfer methods and permissions |
| Parameter file is rejected | Wrong controller generation or incompatible data format | Confirm controller model and supported restoration method |
| CNC starts but machine functions are missing | PLC/PMC or machine-builder data is absent | Restore and verify the required machine-control data |
| Axis reference is incorrect | Reference configuration, encoder state, or physical position differs | Verify the actual machine state and reference procedure |
| Machine dimensions are incorrect | Tool offsets, work offsets, compensation, or scaling is wrong | Compare configuration and verify using approved measurement procedures |
| Part program fails | Missing macro, subprogram, option, or unsupported command | Check program dependencies and controller compatibility |
| Spindle does not operate | Spindle configuration, drive interface, or interlock issue | Check the approved configuration and machine sequence |
| Tool changer fails | PLC/PMC sequence, I/O, or machine-builder data issue | Diagnose sequence conditions and interface signals |
| Recovery image will not boot | Image incompatibility, storage layout, or missing recovery components | Follow the manufacturer-supported recovery procedure |
| Network communication fails | Incorrect address, device configuration, or interface settings | Verify network configuration and controller mappings |
| Alarms occur after restoration | Incomplete configuration, hardware mismatch, or unresolved machine fault | Diagnose the actual cause before resetting alarms |
| Backup appears valid but restoration fails | Missing dependencies or unsupported file format | Review backup scope and perform a controlled restore test |

## 14. Practical maintenance implementation

### 14.1 CNC backup register

| Field | Required information |
| --- | --- |
| Machine ID | Plant-approved machine identifier |
| CNC model | Controller manufacturer and exact series |
| Controller software | Installed software and firmware revision |
| Machine builder | OEM and machine model |
| PLC/PMC revision | Approved machine-control program revision |
| Drive configuration | Relevant axis and spindle-drive identification |
| Backup scope | Parameters, programs, offsets, PLC, system image, or other data |
| Approved revision | Released configuration revision |
| Backup date | Actual completed date |
| Program revision | Approved production-program revision where applicable |
| Compensation status | Availability and revision of required compensation data |
| Reference information | Required homing and machine-reference records |
| Storage location | Controlled repository path |
| Independent copy | Secondary or isolated recovery-copy status |
| Restore-test date | Actual completed test date |
| RTO/RPO | Approved recovery targets |
| Status | Current, overdue, failed, or under review |
| Responsible owner | Assigned maintenance or engineering role |

### 14.2 Suggested folder structure

CNC_BACKUP_SYSTEM/

01_ASSET_REGISTER/

02_APPROVED_BACKUPS/

MACHINE_1059/

CNC_CONTROLLER/

CNC_PARAMETERS/

SYSTEM_CONFIGURATION/

PART_PROGRAMS/

CUSTOM_MACROS/

TOOL_OFFSETS/

WORK_OFFSETS/

COMPENSATION_DATA/

PLC_PMC/

PLC_PROGRAM/

MACHINE_BUILDER_DATA/

I_O_CONFIGURATION/

AXIS_AND_SPINDLE/

DRIVE_PARAMETERS/

MOTOR_AND_ENCODER/

SYSTEM_RECOVERY/

RECOVERY_MEDIA/

SOFTWARE_VERSIONS/

LICENSE_RECORDS/

DOCUMENTATION/

ELECTRICAL_DRAWINGS/

MACHINE_MANUALS/

COMMISSIONING_RECORDS/

CHANGE_HISTORY/

RECOVERY_PROCEDURE/

03_INDEPENDENT_RECOVERY_COPIES/

04_RESTORE_TEST_RECORDS/

05_MIGRATION_RECORDS/

Use the actual machine and controller identifiers in the repository. Restrict access to production programs, machine configuration, and safety-related files according to plant policy.

### 14.3 Work-order integration

For a CNC breakdown, configuration change, or controller replacement:

1.	Identify the affected controller and machine subsystems.

2.	Record the alarm history and failure symptoms.

3.	Preserve the current configuration where practical.

4.	Retrieve the approved recovery package.

5.	Complete the authorized repair or restoration.

6.	Verify machine references, offsets, compensation, and program revisions.

7.	Perform the required functional and acceptance tests.

8.	Save the verified final configuration.

9.	Update the asset register and work-order history.

10.	Record unresolved obsolescence or recovery risks.

## 15. Recovery drills and audit

### 15.1 Recovery drill

A recovery drill verifies that the saved CNC files and supporting procedures can restore the required machine configuration.

1.	Select an approved test controller, nonproduction environment, or controlled recovery exercise.

2.	Retrieve the released backup package.

3.	Verify the controller identity, file integrity, and backup revision.

4.	Confirm the required software, hardware, recovery media, and licensing.

5.	Restore the selected data using the documented procedure.

6.	Verify parameters, programs, PLC/PMC data, and required dependencies.

7.	Review reference, offset, and compensation information.

8.	Perform appropriate functional checks in the approved environment.

9.	Record recovery time and any missing files or dependencies.

10.	Correct the recovery package and repeat the test if required.

Do not test a full system restore on a live production machine without formal authorization and an approved recovery plan.

### 15.2 Audit checklist

Critical CNC machines are recorded in the installed-base register.

Required CNC parameter backups exist.

PLC/PMC and machine-builder data are preserved.

Approved part programs and custom macros are archived.

Tool offsets and work offsets are protected according to policy.

Required compensation and calibration data are available.

Firmware, software, and option dependencies are documented.

Recovery media and supported restoration procedures are available.

Independent recovery copies are maintained.

Backup integrity and revision currency are verified.

Restore tests are performed at the defined interval.

Machine acceptance checks are documented after recovery.

Obsolete controllers have documented migration plans.

Configuration changes trigger the required backup updates.

Recovery times are compared against approved targets.

## 16. CNC backup management KPIs

| KPI | Calculation | Purpose |
| --- | --- | --- |
| Backup coverage | Machines with required backups divided by machines requiring backups, multiplied by 100% | Measures backup availability |
| Backup currency | Machines with current approved backups divided by machines requiring backups, multiplied by 100% | Measures revision accuracy |
| Restore-test success | Successful restore tests divided by completed restore tests, multiplied by 100% | Measures demonstrated recoverability |
| Recovery time | Total measured recovery time divided by recovery events | Measures recovery performance |
| Program revision compliance | Audited programs matching approved revisions divided by programs audited, multiplied by 100% | Measures program control |
| Missing-dependency rate | Recovery packages missing required dependencies divided by packages audited, multiplied by 100% | Identifies incomplete backups |
| Post-recovery acceptance rate | Restored machines passing required acceptance checks divided by restored machines tested, multiplied by 100% | Measures recovery quality |

For example, if 24 of 25 critical CNC machines have current approved backups:

Backup coverage=2425×100=96

High coverage alone does not prove successful recovery. The backup must also be complete, compatible, restorable, and verified against the machine's acceptance requirements.

## 17. Interview questions and technical answers

1\. Why is a CNC backup more than a part-program backup?

A CNC machine also depends on controller parameters, PLC/PMC logic, machine-builder data, offsets, compensation tables, drive settings, and system configuration.

2\. Why must CNC parameters be machine-specific?

Parameters depend on the controller, motors, encoders, axis travel, mechanical arrangement, spindle, and machine-builder configuration.

3\. What is the difference between a CNC parameter backup and a system image?

A parameter backup preserves supported configuration values. A system image may preserve supported software and storage contents. Their scope and restoration requirements differ by controller.

4\. Why are tool and work offsets critical during recovery?

They establish the relationship between the programmed tool path, the tool geometry, the workpiece, and the machine coordinate system. Incorrect values can cause collisions or dimensional errors.

5\. Why must PLC/PMC logic be backed up separately?

The machine sequence logic may be stored separately from CNC parameters and may control tool changes, interlocks, coolant, lubrication, and other auxiliary functions.

6\. Why is compensation data important?

Compensation tables can correct machine-specific positioning or geometric errors. Missing or incorrect data may reduce accuracy even if the controller and programs work normally.

7\. Can a backup from one CNC machine be restored directly to another identical model?

Not automatically. Differences in hardware, machine geometry, options, motors, encoders, firmware, and machine-builder configuration may make the backup incompatible.

8\. What must be verified after CNC controller replacement?

Verify controller configuration, PLC/PMC logic, axis references, tool and work offsets, compensation, spindle behavior, communication, interlocks, safety functions, and machining acceptance results.

9\. Why should recovery procedures be tested?

A stored file may be incomplete, incompatible, corrupted, or missing required dependencies. A controlled restore test helps demonstrate that the recovery process works.

10\. Does a CNC starting without alarms prove that it is ready for production?

No. Machine references, offsets, compensation, axis movement, spindle behavior, auxiliary sequences, safety functions, and required machining accuracy must be verified.

## 18. Final engineering principles

Back up the complete CNC system, not just its part programs.

Preserve controller parameters, PLC/PMC logic, machine-builder data, and required offsets.

Keep machine-specific compensation and calibration records.

Record controller, firmware, software, and hardware dependencies.

Maintain approved program revisions and controlled change history.

Verify compatibility before restoring data to replacement equipment.

Use manufacturer-supported restoration methods.

Protect machine references and coordinate data from uncontrolled changes.

Perform required functional and machining acceptance tests before production release.

Maintain independent recovery copies and periodically test restoration.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\25_Backup_DR\01_Systems_Backup\06_Robot - Robot