# Paint Shop Safety

Study Path: 02_PLAN_WISE_2026\02_INDUSTRY3_0\29_Line_Systems\02_Paint_Shop\08_Safety - Safety

## 1. Purpose and Scope of Paint-Shop Safety

Paint-shop safety covers the people, equipment, materials, control systems, and operating procedures involved in automotive surface preparation, painting, drying, curing, and material handling.

Paint shops combine several hazards that can interact: flammable materials, combustible vapors, electrostatic application equipment, moving robots, conveyors, high temperatures, pressurized fluids, rotating machinery, and electrical systems. A fault in one system can create a hazard in another. Safety therefore depends on coordinated engineering controls, validated interlocks, maintenance discipline, and operating procedures.

The objectives are to:

Prevent fire, explosion, electric shock, crushing, entanglement, burns, and harmful exposure.

Maintain safe ventilation and control hazardous atmospheres.

Prevent unintended machine movement or release of stored energy during maintenance.

Ensure robot cells, conveyors, ovens, and application systems operate only under permitted conditions.

Protect workers from paint chemicals, cleaning agents, fumes, noise, and thermal hazards.

Preserve safety-system integrity during troubleshooting, modification, and equipment replacement.

Maintain inspection records, risk assessments, training, and incident-learning records.

Core principle: production availability never justifies defeating a safety function or entering a hazardous area without the required authorization and safeguards.

## 2. Paint-Shop Hazard Categories

| Hazard category | Typical examples | Potential consequences |
| --- | --- | --- |
| Fire and explosion | Flammable paint solvents, vapor accumulation, ignition sources | Fire, flash fire, explosion |
| Chemical exposure | Paint, solvents, pretreatment chemicals, cleaning agents | Irritation, burns, poisoning, sensitization |
| Mechanical | Robots, conveyors, pumps, fans, rotating shafts | Crushing, impact, entanglement |
| Electrical | Damaged cables, exposed conductors, failed isolation | Electric shock, burns, arc flash |
| Thermal | Curing ovens, heated pipes, hot surfaces | Burns, heat stress, fire |
| Pressure and fluid | Pressurized paint lines, compressed air, hydraulic systems | Injection injuries, hose whip, uncontrolled discharge |
| Atmosphere and ventilation | Poor extraction, abnormal airflow, vapor accumulation | Exposure, flammable atmosphere, oxygen-related hazards where applicable |
| Electrostatic | High-voltage electrostatic applicators and charging equipment | Electric shock, ignition under unsuitable conditions |
| Ergonomic | Manual handling, awkward access, repetitive tasks | Strains and musculoskeletal injuries |
| Slip and trip | Paint spills, wet floors, hoses, cables | Falls and secondary injuries |
| Noise | Fans, pumps, compressed air, machinery | Hearing damage |
| Confined spaces | Tanks, pits, ducts, enclosed process areas | Exposure, engulfment, oxygen deficiency, difficult rescue |

Risk depends on the chemical, concentration, temperature, equipment design, location, and operating state. A hazard assessment must use the actual plant conditions rather than generic assumptions.

## 3. Safety Architecture

A paint shop typically uses multiple layers of protection. The exact arrangement depends on the plant's risk assessment, applicable standards, and equipment design.

Plant Safety Policy and Risk Assessment

↓

Elimination / Substitution / Engineering Controls

↓

Process Protection and Equipment Interlocks

↓

Safety-Related Control Systems

↓

Guarding / Detection / Ventilation / Fire Protection

↓

Operating Procedures / Training / Authorization

↓

Inspection / Testing / Audits / Incident Learning

The layers are complementary. Procedures and PPE cannot reliably compensate for a missing guard, an inadequate ventilation system, or an improperly designed safety interlock.

### 3.1 Hierarchy of controls

The preferred order for reducing risk is generally:

1.	Elimination: remove the hazardous task or exposure.

2.	Substitution: use a less hazardous material or process where feasible.

3.	Engineering controls: isolate people from hazards using guards, ventilation, interlocks, and suitable equipment.

4.	Administrative controls: procedures, permits, training, warning signs, and restricted access.

5.	Personal protective equipment (PPE): protect against residual risks.

PPE is important, but it is the last line of defense rather than a replacement for effective engineering controls.

## 4. Fire and Explosion Prevention

### 4.1 Why paint shops can be hazardous

Some paints, solvents, cleaning agents, and coatings release flammable vapors. If a flammable atmosphere forms and an ignition source is present, a fire or explosion may occur.

The risk depends on factors such as:

Material flash point and flammability characteristics.

Vapor concentration and dispersion.

Ventilation and extraction performance.

Temperature and pressure.

Equipment condition and electrical classification.

Electrostatic charge and discharge.

Hot work and other potential ignition sources.

A material's flash point is not the same as its autoignition temperature or its lower and upper flammable limits. These properties describe different aspects of ignition and combustion.

### 4.2 Common ignition sources

Welding, grinding, and cutting.

Non-approved electrical equipment.

Static discharge.

Hot surfaces and overheated bearings.

Electrical faults and damaged cables.

Improperly selected motors, lights, switches, or instruments.

Smoking or other unauthorized ignition sources.

Sparks generated during mechanical failure.

Equipment suitable for an ordinary industrial area is not automatically suitable for a classified hazardous area.

### 4.3 Preventive controls

Use approved materials and containers.

Maintain the required ventilation and extraction.

Prevent leaks and promptly control spills using approved procedures.

Bond and ground equipment where required by the design.

Use electrical and mechanical equipment appropriate to the hazardous-area classification.

Control hot work through the approved permit system.

Maintain fire detection, suppression, and emergency shutdown systems.

Keep escape routes and emergency equipment accessible.

Store and dispose of flammable materials according to approved requirements.

Investigate recurring leaks, abnormal temperatures, and ventilation alarms.

Do not assume that a fan running indication proves adequate airflow or that an ordinary gas sensor can detect every relevant solvent vapor. The detection technology must be suitable for the hazard.

## 5. Hazardous-Area Classification and Equipment Selection

Hazardous-area classification identifies locations where flammable gas, vapor, or combustible dust may be present under specified conditions.

Depending on the applicable classification system, gas or vapor zones commonly include:

Zone 0: an explosive gas atmosphere is present continuously, for long periods, or frequently.

Zone 1: an explosive gas atmosphere is likely to occur occasionally during normal operation.

Zone 2: an explosive gas atmosphere is not likely during normal operation and, if it occurs, persists only briefly.

Dust-hazard classifications use separate criteria and terminology. Gas-zone definitions should not be applied indiscriminately to dust hazards.

### 5.1 Equipment selection requirements

Equipment selection must consider:

Approved hazardous-area classification.

Equipment protection concept and certification.

Gas or vapor group where applicable.

Temperature class and maximum surface temperature.

Ambient temperature range.

Enclosure protection and environmental exposure.

Installation, wiring, earthing, and maintenance requirements.

The appropriate classification and protection method must be confirmed from the plant's approved hazardous-area documentation. A technician must not infer a zone simply because a location is called a paint booth.

## 6. Ventilation and Airflow Safety

Ventilation supports vapor control, contaminant removal, and the intended process environment. It can also be part of a safety-critical protection strategy.

Typical components include:

Supply and exhaust fans.

Air ducts and dampers.

Filter banks.

Airflow or differential-pressure instruments.

Fan and damper control systems.

Monitoring and interlock logic.

Alarm and shutdown functions.

### 6.1 What maintenance should verify

Fan rotation and operating condition.

Required airflow or pressure differential, measured using the approved method.

Filter loading and condition.

Damper position and mechanical movement.

Sensor health and calibration.

Correct response to airflow failure.

Correct PLC input mapping and alarm indication.

Required response of application equipment when ventilation is inadequate.

Approved restart conditions after a ventilation fault.

The permitted response to ventilation failure must come from the approved safety and process design. Do not create or modify shutdown thresholds by guesswork.

## 7. Electrostatic Application Safety

Electrostatic spray equipment can use high voltage to charge coating particles and improve transfer efficiency. It introduces additional electrical and ignition-control requirements.

Potential hazards include:

High-voltage electrical exposure.

Electrostatic discharge.

Ignition in an unsuitable atmosphere.

Incorrect grounding or bonding.

Damaged applicator cables.

Contaminated or damaged insulation.

Inadequate ventilation or incorrect process permissives.

### 7.1 Essential controls

Follow the equipment manufacturer's approved isolation and discharge procedure.

Confirm the required grounding and bonding arrangements.

Inspect cables, connectors, applicators, and relevant interlocks.

Verify that required ventilation and other permissives are satisfied.

Use only approved cleaning materials and methods.

Prevent unauthorized operation during maintenance.

Do not touch or service high-voltage components until the specified safe condition has been established and verified.

A displayed OFF command does not prove that stored electrical energy has been discharged. Use the specified procedure and appropriate test equipment.

## 8. Robot Cell Safety

Paint-shop robots can move quickly and have a large working envelope. Hazards include crushing, trapping, impact, unexpected motion, and hazards associated with the applicator or coating material.

### 8.1 Main protective measures

Fixed guards and interlocked access doors.

Safety-rated protective devices where required.

Defined operating modes and access permissions.

Emergency-stop devices.

Approved robot safety functions and configuration.

Safe isolation for maintenance.

Controlled recovery from faults.

Verification of safety functions after relevant maintenance or modification.

### 8.2 Robot operating modes

Common modes include automatic operation, manual or teach operation, and maintenance or recovery modes. Names and permissions vary by robot manufacturer and installation.

Each mode must have defined access rules and protective measures. Teach mode is not automatically safe simply because the robot moves slowly.

### 8.3 Robot maintenance precautions

Before entering or working inside a robot cell:

1.	Follow the site's risk assessment, permit requirements, and robot-specific procedure.

2.	Stop the process using the approved sequence.

3.	Isolate all relevant energy sources when required.

4.	Apply lockout/tagout and verify the safe state.

5.	Account for pneumatic, hydraulic, gravitational, and stored electrical energy.

6.	Use the specified method to prevent movement of suspended or gravity-loaded axes.

7.	Verify the required safety conditions before beginning work.

8.	Restore guards and test affected safety functions before releasing the cell.

Never rely on a robot's displayed program state, a PLC run bit, or a standard HMI stop command as proof of safe isolation.

## 9. Conveyor and Material-Handling Safety

Conveyors may present nip points, pinch points, trapping hazards, suspended loads, and unexpected movement.

Important protective measures include:

Fixed or interlocked guards at hazardous locations.

Accessible emergency-stop devices or pull cords where required by the design.

Clearly defined safe access routes.

Prevention of unexpected restart.

Safe procedures for jam clearing and body recovery.

Mechanical support for loads that could fall or move.

Inspection of chains, sprockets, rollers, couplings, and drive systems.

Isolation of electrical, pneumatic, hydraulic, and stored mechanical energy.

An emergency-stop pull cord is a protective measure, not a substitute for guarding or energy isolation during maintenance.

## 10. Oven and Thermal Safety

Paint ovens may contain hot air, heated structures, burners or electric heaters, exhaust systems, and process materials that release vapors.

Hazards can include burns, fire, overheating, unsafe atmosphere, fan failure, and uncontrolled heating.

### 10.1 Important safeguards

Temperature monitoring and approved high-limit protection.

Airflow and exhaust monitoring where required.

Burner management or heater protection systems appropriate to the equipment.

Correct purge and ignition sequences for fuel-fired systems.

Approved responses to flame failure or loss of required airflow.

Controlled entry and access procedures.

Suitable guarding and warning signs for hot surfaces.

Safe isolation before maintenance.

Verification of restart conditions after a trip.

Burner-management and combustion-safety functions must be handled according to the approved design and manufacturer procedures. PLC sequence logic must not be casually substituted for required dedicated protection.

## 11. Chemical Handling and Exposure Control

Paint shops may use coatings, thinners, cleaning agents, pretreatment chemicals, sealants, and other process materials. Their hazards differ considerably.

### 11.1 Required information

Use the current safety data sheet (SDS) and approved site documentation to determine:

Chemical hazards and exposure routes.

Required ventilation and handling precautions.

Incompatible materials.

Spill-response methods.

Firefighting restrictions.

First-aid measures.

Storage and disposal requirements.

Suitable gloves, eye protection, clothing, and respiratory protection.

### 11.2 Practical controls

Keep containers identified and properly closed.

Use approved transfer equipment.

Prevent uncontrolled mixing of incompatible chemicals.

Maintain suitable local exhaust and general ventilation.

Use spill kits appropriate to the material.

Prevent spills from entering drains unless the approved response specifically permits it.

Follow exposure-control and hygiene requirements.

Store chemicals according to compatibility and fire-safety rules.

Report leaks, symptoms of exposure, and damaged containers promptly.

Respiratory protection must be selected for the identified hazard and exposure conditions. A generic dust mask is not a universal protection against solvent vapors.

## 12. Electrical Safety and Control Panels

Paint-shop electrical systems may include PLCs, remote I/O, VFDs, servo drives, robot controllers, heaters, pumps, sensors, and high-voltage application equipment.

### 12.1 Typical hazards

Electric shock and arc flash.

Incorrect isolation.

Stored energy in DC-link capacitors.

Unexpected motor or actuator restart.

Incorrect replacement of protective devices.

Damaged insulation and contamination.

Incorrect earthing or bonding.

Unapproved changes to safety-related circuits.

### 12.2 Safe maintenance principles

Only authorized and competent personnel should perform the relevant electrical work.

Identify all energy sources and possible backfeeds.

Isolate and secure the equipment using the approved procedure.

Verify absence of voltage using suitable equipment and the required proving method.

Observe manufacturer-specified discharge waiting times.

Follow arc-flash and electrical PPE requirements where applicable.

Keep enclosure protection and cable-entry sealing intact.

Restore covers, barriers, and protective conductors before energization.

Test the equipment under the approved commissioning and release procedure.

A VFD may have a de-energized input while dangerous voltage remains inside. Its DC bus must be treated according to the manufacturer's discharge and verification instructions.

## 13. Lockout/Tagout and Hazardous-Energy Control

Lockout/tagout (LOTO) is a formal method of preventing unexpected energization, startup, or release of hazardous energy during servicing.

Potential energy sources include:

Electrical power.

Pneumatic pressure.

Hydraulic pressure.

Gravity and suspended loads.

Springs and mechanical tension.

Thermal energy.

Chemical pressure or hazardous material.

Stored electrical energy in capacitors.

Residual movement and process pressure.

### 13.1 General LOTO sequence

The site's approved procedure takes precedence. A typical sequence is:

1.	Identify the equipment and all energy sources.

2.	Notify affected personnel.

3.	Stop the equipment using the approved shutdown procedure.

4.	Isolate each relevant energy source.

5.	Apply personal locks and tags according to site rules.

6.	Release, restrain, or otherwise control stored energy.

7.	Verify isolation and the safe state using the approved method.

8.	Perform the authorized work.

9.	Inspect the work area and reinstall required guards.

10.	Remove locks according to the established ownership and removal procedure.

11.	Notify affected personnel and restore energy under controlled conditions.

12.	Verify safe operation before releasing the equipment.

### 13.2 Try-start and zero-energy verification

A try-start may be part of the approved verification procedure, but it is not sufficient for every type of energy or equipment.

Verification may require electrical testing, pressure gauges, mechanical restraints, discharge verification, or other methods specified by the risk assessment and procedure.

A zero-energy condition must be established and verified for the hazards relevant to the task. Never assume that turning a selector to OFF or pressing an emergency stop has isolated the equipment.

## 14. Safety PLCs, Interlocks, and Emergency Stops

A safety-related control system performs functions intended to reduce risk, such as preventing hazardous motion when access is open.

Depending on the risk assessment and design, the system may use safety relays, safety PLCs, safety-rated drives, redundant channels, monitored contactors, or other protective components.

### 14.1 Standard PLC versus safety controller

| Feature | Standard control system | Safety-related control system |
| --- | --- | --- |
| Primary purpose | Process sequencing and equipment control | Risk-reduction safety functions |
| Typical signals | Run commands, process values, normal status | Guard switches, safety sensors, emergency-stop channels |
| Design basis | Functional and process requirements | Defined safety requirements and risk assessment |
| Fault response | Depends on application logic | Designed and validated to meet specified safety requirements |
| Modification | Approved control change process | Safety change process, validation, and documentation as required |

A standard PLC can monitor a safety-system status, but that does not make its ordinary logic a safety function.

### 14.2 Emergency stop

An emergency stop is intended to reduce an emergency risk. It is not a replacement for guarding, isolation, or safe work procedures.

Important principles include:

The required stop behavior must be determined by the machine's risk assessment.

Resetting an emergency-stop device must not automatically initiate hazardous motion unless the approved design explicitly permits the resulting sequence and all other requirements are met.

Resetting is distinct from restarting.

Emergency-stop functions must be tested using the approved schedule and procedure.

### 14.3 Safe Torque Off (STO)

STO is a safety-related drive function that prevents the drive from generating torque, when implemented and used within its specified conditions.

STO does not necessarily provide electrical isolation, prevent gravity-driven motion, stop a load within a required distance, or discharge stored energy. Mechanical brakes and other protective measures may be required.

## 15. Safety Sensors and Protective Devices

| Device | Typical function | Important maintenance checks |
| --- | --- | --- |
| Guard interlock | Detects access-door state | Alignment, actuation, wiring, approved functional test |
| Safety light curtain | Detects interruption of a protected field | Alignment, damage, muting configuration, response test |
| Safety laser scanner | Monitors configured protective fields | Field configuration, mounting, contamination, test results |
| Emergency-stop device | Initiates an emergency-stop function | Accessibility, mechanical condition, approved functional test |
| Pressure switch | Monitors a defined pressure condition | Range, calibration, switching behavior |
| Temperature limit device | Detects an abnormal temperature condition | Sensor condition, trip setting, independent function where required |
| Flame detector | Detects flame in a combustion system | Correct model, condition, diagnostics, approved functional testing |
| Gas or vapor detector | Detects specified substances or atmospheric conditions | Correct target gas, calibration, response test, placement |
| Safety relay or PLC | Processes safety-related inputs and outputs | Diagnostics, approved validation, configuration control |
| Safety-rated drive | Implements specified drive safety functions | Correct configuration, diagnostics, functional validation |

A device's presence does not prove that the entire safety function works. The sensor, logic, output element, wiring, configuration, and final machine response must be considered as a complete function.

## 16. Pneumatic, Hydraulic, and Stored-Pressure Safety

Paint-shop machinery may use compressed air, hydraulic actuators, pressure vessels, and pressurized fluid-delivery systems.

Potential hazards include hose rupture, injection injuries, unexpected cylinder movement, and stored pressure remaining after shutdown.

Maintenance requirements include:

Identify all pressure sources and accumulators.

Isolate the supply using approved devices.

Depressurize the relevant circuit through the designated method.

Verify pressure has been reduced to the required safe state.

Restrain moving components that may fall or shift.

Inspect hoses, fittings, regulators, and pressure-relief devices.

Use components with appropriate pressure ratings.

Never search for a high-pressure leak using bare hands.

Never loosen a fitting merely to determine whether pressure remains.

High-pressure fluid injection injuries can be severe even when the external wound appears small. Such an incident requires immediate medical assessment.

## 17. Safety Documentation and Machine Knowledge Base

Safety documentation helps technicians understand hazards, approved controls, and the evidence required to release equipment after work.

A machine knowledge base should include:

Equipment identification and location.

Risk assessment and applicable operating restrictions.

Hazardous-area classification and approved equipment list.

LOTO procedures and energy-isolation points.

Safety circuit drawings and I/O mapping.

Safety PLC or relay configuration and revision, where applicable.

Guard, scanner, light-curtain, and emergency-stop locations.

Robot-cell entry and recovery procedures.

Oven and burner safety documentation.

Chemical SDS access and spill-response procedures.

Inspection and proof-test schedules.

Test records and outstanding safety defects.

Approved backups and change history.

Authorized personnel and escalation contacts.

Never store safety configuration only in an individual technician's laptop or depend on undocumented knowledge held by one person.

## 18. Safety Inspection and Preventive Maintenance

A safety maintenance program should be based on risk, manufacturer requirements, applicable standards, and the plant's approved inspection schedule.

| Inspection item | Typical checks | Record required |
| --- | --- | --- |
| Guards and doors | Damage, gaps, interlock condition | Inspection result and defect |
| Emergency stops | Accessibility and approved function test | Test date and result |
| Safety sensors | Alignment, contamination, mounting, diagnostics | Test record |
| Safety PLC or relay | Diagnostics, configuration status, fault history | Configuration and validation record |
| Ventilation | Fan condition, measured performance, alarms | Measurement and corrective action |
| Fire protection | Inspection status and identified defects | Inspection record |
| Gas or vapor detection | Calibration and response testing | Calibration and test record |
| LOTO points | Identification, accessibility, isolation verification | Procedure review record |
| Pressure equipment | Hoses, fittings, relief devices, leakage | Inspection and repair history |
| Oven protection | Temperature limits and approved combustion safeguards | Test record |
| Robot cell | Guarding, access controls, approved safety functions | Validation and maintenance record |
| Safety documentation | Current drawings, procedures, and revisions | Document review history |

Routine visual inspection and formal safety-function validation are different activities. A visual check alone cannot establish the required performance of a safety function.

## 19. Troubleshooting Safety Faults

Safety-related faults must be diagnosed without defeating the function intended to protect personnel.

### 19.1 Guard door will not permit restart

Possible causes include:

Door not fully closed.

Misaligned or damaged interlock.

Failed sensor or wiring.

Safety relay or safety PLC fault.

Incorrect reset sequence.

Another safety input remains active.

Recommended approach:

1.	Keep personnel outside the hazardous area.

2.	Read the fault indication and identify the affected safety function.

3.	Inspect the door and actuator for visible damage without exposing personnel to hazards.

4.	Check diagnostics and approved drawings.

5.	Verify the input and output states using authorized methods.

6.	Repair the confirmed fault.

7.	Complete the required safety-function test before returning the cell to production.

Do not bridge the interlock to prove that it is the cause.

### 19.2 Ventilation fault inhibits paint application

Possible causes include:

Fan trip or drive fault.

Filter blockage.

Damper malfunction.

Failed airflow or pressure instrument.

Wiring or communications fault.

Actual ventilation performance below the required limit.

The investigation must determine whether ventilation is genuinely inadequate or the measurement/control system has failed. Application must remain inhibited whenever the approved safety design requires it.

### 19.3 Oven trips on a protective condition

Possible causes include loss of required airflow, temperature-limit activation, flame failure, or a control-system fault.

Follow the approved shutdown and fault-recovery procedure. Do not repeatedly reset a protective trip without identifying the cause, and do not alter the trip threshold to avoid production interruption.

## 20. Incident Response and Near-Miss Learning

A near miss is an event that could have caused injury, illness, environmental damage, or equipment loss but did not produce that outcome.

Examples include:

A robot moving unexpectedly during recovery.

A solvent leak detected before ignition.

A conveyor restarting while personnel were clearing a jam.

A guard interlock failing to prevent access.

An oven ventilation alarm repeatedly occurring without investigation.

A technician finding a damaged electrical enclosure or defeated protective device.

A useful incident investigation records:

What happened and where.

Equipment state and operating mode.

Energy sources and exposure conditions.

Relevant alarms, logs, and physical evidence.

Immediate containment.

Direct and contributing causes.

Corrective and preventive actions.

Responsible owner and due date.

Verification of effectiveness.

The objective is to eliminate or reduce the hazard and prevent recurrence, not merely to assign blame.

## 21. Safety Audits and Legal Compliance

Paint-shop safety must be managed according to applicable legislation, plant requirements, equipment standards, and documented risk assessments.

Depending on the facility and process, relevant requirements may include:

Occupational health and safety requirements.

Factory and machinery safety provisions.

Fire prevention and emergency preparedness.

Hazardous chemical handling and storage.

Electrical installation and maintenance requirements.

Hazardous-area equipment selection and inspection.

Pressure equipment requirements.

Environmental controls for emissions, waste, and spills.

Machine safety and functional-safety standards.

Standards such as ISO 12100 for machinery risk assessment, ISO 13849-1 for safety-related control systems, IEC 62061 for machinery functional safety, and IEC 60204-1 for electrical equipment of machinery may be relevant to specific equipment and projects. Applicability and current editions must be confirmed by the responsible engineering and compliance teams.

### 21.1 Practical safety audit checklist

Risk assessments reflect the installed equipment and current operating conditions.

Required guards and interlocks are present and functional.

Safety-related modifications have been reviewed and validated.

Hazardous-energy isolation points are identified and documented.

Electrical panels and hazardous-area equipment are suitable for their locations.

Ventilation and fire-protection systems meet their defined requirements.

Chemical containers, storage locations, and SDS documentation are compliant.

Safety inspections and required functional tests are current.

Open safety defects have assigned owners and controlled risk-reduction measures.

Personnel have the required training and authorization.

Emergency routes and equipment remain accessible.

Corrective actions are closed with evidence of effectiveness.

A completed checklist is evidence of an inspection, not proof that every hazard has been eliminated. Findings must be evaluated against the actual risk.

## 22. Practical Exercises

Identify the major fire, chemical, mechanical, electrical, and thermal hazards in a paint-shop area.

Locate the approved LOTO procedure for a conveyor drive and identify every relevant energy source.

Trace one robot-cell guard interlock from its field device to the safety controller and final protective response using approved documentation.

Review the ventilation-failure response for a paint application area.

Identify which equipment in a designated area requires hazardous-area approval.

Inspect a machine's safety documentation and identify missing or outdated records.

Review a safety-related fault and distinguish the initiating fault from secondary alarms.

Create an RCA/CAPA record for a recurring safety defect.

Identify the differences between an emergency stop, a normal stop, STO, and LOTO.

Review the required inspection and validation records for a safety-related device.

Develop a safety maintenance checklist for one robot cell, conveyor, or oven.

Explain how a safety-related configuration change must be approved, tested, recorded, and released.

## 23. Competency Checklist

A competent industrial maintenance engineer should be able to:

Explain the major hazards associated with paint-shop equipment and materials.

Apply the hierarchy of controls when evaluating risks.

Understand fire prevention, ventilation, and hazardous-area requirements.

Recognize chemical exposure and electrostatic application hazards.

Follow approved LOTO procedures and verify hazardous-energy isolation.

Distinguish standard control logic from safety-related control functions.

Explain the limitations of emergency stops and STO.

Diagnose safety faults without bypassing protective functions.

Understand the safety requirements for robots, conveyors, and ovens.

Maintain safety documentation, configuration backups, and inspection records.

Participate in risk assessments, safety audits, RCA, and CAPA.

Verify that required safety tests are complete before equipment is returned to service.

# NEXT TOPIC

02_PLAN_WISE_2026\02_INDUSTRY3_0\29_Line_Systems\02_Paint_Shop\09_Oven_Control - Oven control